3GPP TSG-RAN WG2 Meeting #120	R2-2211654

Toulouse, France, 14th – 18th November, 2022

Source: 	vivo

Title:  	Discussion on per-FR gap capability

Agenda Item:	6.24.1.1

Document for:	Discussion and Decision

Introduction

Regarding per-FR gap capability, the following agreements are achieved during RAN2#119bis-e [1]:

Exclude Alt 1.1 for now. On the table: Alt 1.3, Alt 1.3 per BC, Alt 2 (add info, based on current config as today, FFS excl/incl DC)

In this contribution, in order to determine the final form of per-FR gap capability reporting, our analyses and proposals on this issue will be provided.

Discussion

Introduction

According to the existing TS 38.306 [2], independentGapConfig is defined to indicate whether the UE supports two independent measurement gap configurations for FR1 and FR2, this field also indicates whether the UE supports the FR2 inter-RAT measurement without gaps when (NG)EN-DC is not configured. This per-FR gap feature is defined per UE, which means that if a UE supports two independent measurement gap configurations for FR1 and FR2, the UE support this feature on all CC combinations.

Whether UE support such per-FR gap capability depends on whether RF links of FR1 and FR2 are independent. That is if UE support such per-FR gap capability, the dedicated resource is used for FR1 and FR2 measurement respectively. But increasingly higher order of CC combinations is supported by UE. In the case of higher-order CC combinations, the UE may not have enough baseband resources to support two independent measurement gaps for FR1 and FR2 on all CC combinations. In other words, the UE may support per-FR gap capability on some CC combinations, while it may not support such capability on other CC combinations. So the existing per-UE capability indication is not appropriate and a new solution is needed to indicate whether the UE supports two independent measurement gap configurations for FR1 and FR2.

After RAN2#119bis-e, two candidate solutions remain:

- Alt 1.3 (more fine grained capability for Per-FR-Gaps - limited by number of carriers), and

- Alt 2 (Use similar framework/procedure as for “NeedForGap”).

For Alt 1.3, the number of carriers does not fully reflect the occupancy of baseband resources, which also be related to the bandwidth of the carrier. So Alt 1.3 may not reflect the per-FR gap capability correctly. For Alt 2, the UE can indicate whether it supports per-FR gap capability on its working/configured band. This approach is flexible and has low signaling overhead, thus we propose that RAN2 consider Alt 2 to indicate UE’s per-FR gap capability.

Proposal 1: Use similar framework/procedure as for “NeedForGap” to indicate per-FR gap capability.

“NeedForGap”-like procedure for per-FR gap capability report

- 2.2.1 Procedure for per-FR capability report

Similar to the mechanism of “NeedForGap”, we think that the request-response mechanism can also be used for per-FR gap capability reporting, i.e., the network configures UE to report the per-FR gap capability on its working/configured CC combination, this configuration can be included in the RRCReconfiguration message and RRCResume message, and then UE provides its per-FR gap capability in the RRCReconfigurationComplete message and RRCResumeComplete message. Furthermore, to further save signaling overhead, if the per-FR gap capability indication is not changed compared to last time the UE reported this information, this indication may not be reported, the network assumes that UE’s per-FR gap capability is same as the last report information by UE.

Proposal 2: The network configures UE to report the per-FR gap capability in the RRCReconfiguration message and RRCResume message.

Proposal 3: If the network configures and if the per-FR gap capability indication is changed compared to last time the UE reported this information, UE reports per-FR gap capability indication.

According to TS 38.306 [2], independentGapConfigPRS is defined to indicate whether the UE supports two independent measurement gap configurations for FR1 and FR2 for PRS measurement. In our understanding, in order to report per-FR gap capability for PRS in the case of higher-order CC combinations, similar mechanism of “NeedForGap” can also be adopted.

Proposal 4: The network configures UE to report the per-FR gap capability for PRS in the RRCReconfiguration message and RRCResume message.

Proposal 5: if the network configures, if the per-FR gap capability indication for PRS is changed compared to last time the UE reported this information, UE reports per-FR gap capability indication for PRS.

In our understanding, this new per-FR gap capability report feature should be supported from release 17.

Proposal 6: The new feature for per-FR gap capability report should be supported from Rel-17.

Furthermore, to support the new feature of per-FR gap capability report, the corresponding capability should be defined, i.e. UE should indicate whether UE supports reporting the per-FR gap capability in the UE response to a network configuration RRC message. Moreover, if UE supports independentGapConfig, UE doesn’t need to report this capability. Similarly, the corresponding capability for new feature of per-FR gap capability report for PRS should also be introduced.

Proposal 7: A new capability should be introduced to indicate whether UE supports reporting the per-FR gap capability response to a network configuration RRC message. if UE supports independentGapConfig, UE doesn’t need to report this capability.

Proposal 8: A new capability should be introduced to indicate whether UE supports reporting the per-FR gap capability for PRS response to a network configuration RRC message. if UE supports independentGapConfigPRS-r17, UE doesn’t need to report this capability.

- 2.2.2 Discussion on MR-DC support

It should be noted that the current NeedForGaps-framework doesn’t currently support MR-DC. However, for per-FR gap capability report, we think it makes sense to support the above mechanism in the MR-DC scenario. Otherwise, the gNB may always configure per-UE gap for the UE, which may affect the experience of the UE.

Proposal 9: “NeedForGap”-like procedure for per-FR gap capability report should be supported in MR-DC.

In order to support “NeedForGap”-like procedure for per-FR gap capability report, the following issues should be discussed further:

- Issue 1: which node configures to request per-FR gap capability from UE;

- Issue 2: what information needs to be interacted between MN and SN.

Next, we will analyze these issues one by one.

Issue 1: which node configures to request per-FR gap capability from UE

For MR-DC cases, the following final decision node for gap configuration principles are defined in R15 as shown in below Table 1:

Table 1  Node to decide measurement gap configuration

| MR-DC | Per-UE gap | Per-FR1 gap | Per-FR2 gap |
| --- | --- | --- | --- |
| (NG)EN-DC | MN | MN | SN |
| NE-DC | MN | MN | MN |
| NR-DC | MN | MN | MN |

As we can see from the table above, regardless of the MR-DC scenario, the MN needs to know the per-FR gap capability for UE since MN anyway needs to be configured per-UE gap or per-FR1 gap, so naturally MN can configure to request per-FR gap capability from UE and transmit configuration to UE. Besides, for (NG)EN-DC scenario, if UE supports independent gap between FR1 and FR2, it is SN's responsibility to configure per-FR2 gap for UE, SN needs to obtain UE’s per-FR gap capability. To make the interaction between the UE side and the network side simpler, we think it is reasonable that MN sends the configuration in the (NG)EN-DC scenario, in addition, SN can request the MN to send the configuration to the UE.

Proposal 10: MN sends the configuration to UE to request per-FR gap capability report in“NeedForGap”-like procedure.

Issue 2: what information needs to be interacted between MN and SN

Based on the analyses in issue 1, since SN can request the MN to send the configuration to the UE to obtain whether the UE support in (NG)EN-DC scenario, the corresponding indication should be introduced. Furthermore, upon UE reporting the per-FR capability, MN should forward this information to SN in  (NG)EN-DC scenario while this information forwarding is not needed in NE-DC and NR-DC scenarios.

Proposal 11: For (NG)EN-DC, SN can send an indication to request MN configures UE to report per-FR gap capability in“NeedForGap”-like procedure, MN should forward per-FR gap capability to SN.

To reflect the impact on TS 38.331 and TS 38.306, CRs are presented in R2-2211655 [3] and R2-2211656 [4].

Proposal 12: If P1-P11 are agreed, RAN2 to adopt the corresponding CR in R2-2211655 and R2-2211656.

Conclusions

Based on the analyses given above, we have the following proposals:

Proposal 1: Use similar framework/procedure as for “NeedForGap” to indicate per-FR gap capability.

Proposal 2: The network configures UE to report the per-FR gap capability in the RRCReconfiguration message and RRCResume message.

Proposal 3: If the network configures and if the per-FR gap capability indication is changed compared to last time the UE reported this information, UE reports per-FR gap capability indication.

Proposal 4: The network configures UE to report the per-FR gap capability for PRS in the RRCReconfiguration message and RRCResume message.

Proposal 5: if the network configures, if the per-FR gap capability indication for PRS is changed compared to last time the UE reported this information, UE reports per-FR gap capability indication for PRS.

Proposal 6: The new feature for per-FR gap capability report should be supported from Rel-17.

Proposal 7: A new capability should be introduced to indicate whether UE supports reporting the per-FR gap capability response to a network configuration RRC message. if UE supports independentGapConfig, UE doesn’t need to report this capability.

Proposal 8: A new capability should be introduced to indicate whether UE supports reporting the per-FR gap capability for PRS response to a network configuration RRC message. if UE supports independentGapConfigPRS-r17, UE doesn’t need to report this capability.

Proposal 9: “NeedForGap”-like procedure for per-FR gap capability report should be supported in MR-DC.

Proposal 10: MN sends the configuration to UE to request per-FR gap capability report in“NeedForGap”-like procedure.

Proposal 11: For (NG)EN-DC, SN can send an indication to request MN configures UE to report per-FR gap capability in“NeedForGap”-like procedure, MN should forward per-FR gap capability to SN.

Proposal 12: If P1-P11 are agreed, RAN2 to adopt the corresponding CR in R2-2211655 and R2-2211656.

Reference

- Chairman_nots of 3GPP TSG RAN WG2 meeting #119bis-e, Online

- TS 38.306, User Equipment (UE) radio access capabilities

- R2-2211655, 38.331 CR on per-FR gap capability

- R2-2211656, 38.306 CR on per-FR gap capability