3GPP TSG-RAN2 Meeting #120	R2-2211764

# Toulouse, France, 14th - 18th November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3640 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.331 CR on the periodicity of the MIB |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |  |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |
| Reason for change: |  | According to the current specification, the MIB is transmitted on a regular basis with the periodicity of 80ms; besides, the period of SSB holds up till 160ms, in which PBCH is located to carry MIB. However, when pushing to a limit that SSB is transmitted in a period of 160ms, it is irrational that the MIB is transmitted with the periodicity of 80ms. ssb-periodicityServingCell ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 } OPTIONAL, -- Need S In this context, it is necessary to prolong the periodicity of the MIB to be equal to that of SSB for some circumstances, leading that no repetition for MIB in such situations. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Section 5.2.1, modify the description of periodicity of the MIB with conditional repetitions. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: MIB transmission Inter-operability analysis: 1. If the network is implemented according to the CR and the UE is not, no inter-operability issues are expected. 2. If the UE is implemented according to the CR and the network is not, no inter-operability issues are expected. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | misunderstanding on the periodicity of the MIB |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1 |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

Start of change

## 5.2	System information

### 5.2.1	Introduction

System Information (SI) is divided into the MIB and a number of SIBs and posSIBs where:

-	the MIB is always transmitted on the BCH and includes parameters that are needed to acquire SIB1 from the cell. If the period of SSB is larger than or equal to 80 ms, the periodicity of the MIB equals to the period of SSB, otherwise the MIB is transmitted with a periodicity of 80 ms and repetitions are made within 80 ms (TS 38.212 [17], clause 7.1). The first transmission of the MIB is scheduled in subframes as defined in TS 38.213 [13], clause 4.1 and repetitions are scheduled according to the period of SSB;

-	the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

-	SIBs other than SIB1 and posSIBs are carried in SystemInformation (SI) messages, which are transmitted on the DL-SCH. Only SIBs or posSIBs having the same periodicity can be mapped to the same SI message. SIBs and posSIBs are mapped to the different SI messages. Each SI message is transmitted within periodically occurring time domain windows (referred to as SI-windows with same length for all SI messages). Each SI message is associated with an SI-window and the SI-windows of different SI messages do not overlap. That is, within one SI-window only the corresponding SI message is transmitted. An SI message may be transmitted a number of times within the SI-window. Any SIB or posSIB except SIB1 can be configured to be cell specific or area specific, using an indication in SIB1. The cell specific SIB is applicable only within a cell that provides the SIB while the area specific SIB is applicable within an area referred to as SI area, which consists of one or several cells and is identified by systemInformationAreaID;

-	The mapping of SIBs to SI messages is configured in schedulingInfoList and schedulingInfoList2, while the mapping of posSIBs to SI messages is configured in posSchedulingInfoList and schedulingInfoList2. SIBs and posSIBs are mapped to separate SI messages even when configured using a common schedulingInfoList2. Each SIB is contained only in a single SI message. In the case of posSIB, a posSIB carrying GNSS Generic Assistance Data for different GNSS/SBAS [49] is contained in different SI messages. Each SIB and posSIB, including a posSIB carrying GNSS Generic Assistance Data for one GNSS/SBAS, is contained at most once in that SI message;

-	For a UE in RRC_CONNECTED, the network can provide system information through dedicated signalling using the RRCReconfiguration message, e.g. if the UE has an active BWP with no common search space configured to monitor system information, paging, or upon request from the UE.

-	For PSCell and SCells, the network provides the required SI by dedicated signalling, i.e. within an RRCReconfiguration message. Nevertheless, the UE shall acquire MIB of the PSCell to get SFN timing of the SCG (which may be different from MCG). Upon change of relevant SI for SCell, the network releases and adds the concerned SCell. For PSCell, the required SI can only be changed with Reconfiguration with Sync.

End of change