3GPP TSG-RAN WG2 Meeting #120	R2-2211772

Toulouse, France, 14 – 18 November 2022

Agenda item:	6.9.1

Source:	Nokia, Nokia Shanghai Bell

Title:	PEI monitoring with UE specific DRX cycle

WID/SID:	NR_UE_pow_sav_enh-Core

Document for:	Discussion and Decision

# 1	Introduction

As part of the release 17 UE power saving work item the RAN1 and RAN2 have specified the Paging Early Indication (PEI) for RRC Idle and RRC Inactive UEs. The PDCCH-based PEI is transmitted by the gNB to indicate to a UE whether to monitor for the paging DCI in the Paging Occasion (PO).

The location of the PEI is defined with reference to the Paging Frame (PF), which further defines the location of the PO. The PF depends on the DRX cycle, which can be either cell specific or UE specific. In this paper we discuss PEI monitoring behaviour, when the UE is using a UE specific DRX cycle.

# 2	PEI monitoring

Currently multiple PEI occasions can be configured per DRX cycle with association of PEI-O with the PF(s)/PO(s), and the UE monitors one PEI occasion per DRX cycle.

However, the PF depends on the DRX cycle T, which is the shortest of the UE specific DRX cycle and the cell specific DRX cycle. So the UE specific PF could be different from the cell specific PFs in case the UE have a UE specific DRX cycle shorter than the cell specific cycle.

Observation 1: UE’s PF is defined based on the shortest of the UE specific DRX cycle and the cell specific DRX cycle thus could be different from cell specific ones.

The issue is that the PEI monitoring behaviour is not clear in case there is no PEI occasions configured for those UE specific PF, when the UE is configured with a UE specific DRX cycle, which is shorter than the cell specific DRX cycle. The specification text in section 7.2.1 only defines “The UE monitors one PEI occasion per DRX cycle”.

Observation 2: The PEI monitoring behaviour is not clear when the UE is configured with a UE specific DRX cycle shorter than the cell specific DRX cycle.

As noted above the PEI-O depends on the PF. If the UE specific DRX cycle and the cell specific DRX cycle do not have common PFs it may result in PEI not being configured/transmitted for the UE specific DRX cycle because the network would need to configure a search space, which has PDCCH monitoring occasions (MO) associated with the UE specific PEI-O and the network has no idea UEs with which UE specific DRX cycle has camped on the cell and would be paged in the cell thus only possible to assume worst case. It would consume additional resources, because the network would have to configure the UE specific PEI-O without knowing if there actually is a UE with the corresponding UE specific DRX cycle in the cell which does not make much sense.

Observation 3: There may not be a search space for the PEI-O associated with the UE specific DRX cycle if the PFs of the UE specific and cell specific DRX cycles are not common.

The issue needs to be resolved by RAN2 such that it is clear to the UE whether it can expect PEI transmission when the UE is configured with a DRX cycle, which is shorter than the cell specific DRX cycle.

Note that the issue only exists when the PF of the UE specific and the cell specific DRX cycle are not common. When there are multiple PFs per DRX cycle, the UE specific PF might still be possible to align with cell specific ones (could be for this UE or other UEs though) even though the UE’s DRX cycle is shorter than the cell specific one. Therefore, one solution could be to define that the UE shall monitor for PEI in the PEI-O, which is associated with a common PF, while the UE shall not monitor for PEI when the corresponding PF is only associated with the UE specific DRX cycle. In other words, if there is a PEI-O associated with the UE’s PO the UE shall monitor for PEI. The calculation of the association between PEI-O and UE’s PO is straight forward based on the existing definitions of PEI-O and PO.

A simplification could be to define the UE does not monitor for PEI, when the UE is configured with a UE specific DRX cycle which is shorter than the cell specific DRX cycle. This is a simple approach, but it artificially restricts the UE from reading PEI when it is available and will not harvest the benefit of the release 17 PEI.

Proposal: RAN2 to specify if UE with UE specific DRX cycle shorter than the cell specific cycle, it monitors for PEI if there is a PEI-associated with the UE’s PF/PO, i.e. for a common PO for the UE specific and cell specific DRX cycles, otherwise the UE monitors PO directly.

In the appendix we have provided a text proposal for both options of the UE monitoring the PEI or not depending on whether UE’s PO overlaps with the cell specific one or simply skip PEI when the UE specific DRX cycle is shorter than the cell specific one.

# 3	Conclusion

PEI monitoring with UE specific DRX cycle is discussed in this contribution with the following observations and proposal made:

Observation 1: UE’s PF is defined based on the shortest of the UE specific DRX cycle and the cell specific DRX cycle thus could be different from cell specific ones.

Observation 2: The PEI monitoring behaviour is not clear when the UE is configured with a UE specific DRX cycle shorter than the cell specific DRX cycle.

Observation 3: There may not be a search space for the PEI-O associated with the UE specific DRX cycle if the PFs of the UE specific and cell specific DRX cycles are not common.

Proposal: RAN2 to specify if UE with UE specific DRX cycle shorter than the cell specific cycle, it monitors for PEI if there is a PEI-associated with the UE’s PF/PO, i.e. for a common PO for the UE specific and cell specific DRX cycles, otherwise the UE monitors PO directly.

# Appendix – Text proposals

## Option A: monitor PEI only if w

First Modified Subclause

### 7.2.1	Paging Early Indication reception

The UE may use Paging Early Indication (PEI) in RRC_IDLE and RRC_INACTIVE states in order to reduce power consumption. If PEI configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting PEI (except for the UEs expecting multicast session activation notification) can monitor PEI using PEI parameters in system information according to the procedure described below.

If lastUsedCellOnly is configured in system information of a cell, the UE monitors PEI in the cell only if the UE most recently received RRCRelease without noLastCellUpdate in this cell. Otherwise (i.e., if lastUsedCellOnly is not configured in system information of a cell), the UE monitors PEI in the camped cell.

The UE monitors one PEI occasion per DRX cycle. A PEI occasion (PEI-O) is a set of PDCCH monitoring occasions (MOs) and can consist of multiple time slots (e.g. subframes or OFDM symbols) where PEI can be sent (TS 38.213 [4]). In multi-beam operations, the UE assumes that the same PEI is repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the PEI is up to UE implementation.

The time location of PEI-O for UE's PO is determined by a reference point and an offset:

-	The reference point is the start of a reference frame determined by a frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O, provided by pei-FrameOffset in SIB1;

-	The offset is a symbol-level offset from the reference point to the start of the first PDCCH MO of this PEI-O, provided by firstPDCCH-MonitoringOccasionOfPEI-O in SIB1.

If one PEI-O is associated with POs of two PFs, the two PFs are consecutive PFs calculated by the parameters PF_offset, T, Ns, and N. The first PF of the PFs associated with the PEI-O is provided by (SFN for PF) - floor (iPO/Ns)*T/N, where SFN for PF is determined in clause 7.1, iPO is defined in clause 10.4a in TS 38.213[4], T, Ns, and N are determined in clause 7.1.

If the UE is configured with a UE specific DRX cycle shorter than cell specific DRX cycle, the UE monitors PEI only if the UE’s PF is common with the cell specific DRX cycle PF and therefore has PEI-O configured associated with its PO, otherwise it monitors PO directly.

End of Changes

## Option B

First Modified Subclause

### 7.2.1	Paging Early Indication reception

The UE may use Paging Early Indication (PEI) in RRC_IDLE and RRC_INACTIVE states in order to reduce power consumption. If PEI configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting PEI (except for the UEs expecting multicast session activation notification) can monitor PEI using PEI parameters in system information according to the procedure described below.

If lastUsedCellOnly is configured in system information of a cell, the UE monitors PEI in the cell only if the UE most recently received RRCRelease without noLastCellUpdate in this cell. Otherwise (i.e., if lastUsedCellOnly is not configured in system information of a cell), the UE monitors PEI in the camped cell.

The UE monitors one PEI occasion per DRX cycle if the UE is not configured with a UE specific DRX cycle shorter than the cell specific cycle. A PEI occasion (PEI-O) is a set of PDCCH monitoring occasions (MOs) and can consist of multiple time slots (e.g. subframes or OFDM symbols) where PEI can be sent (TS 38.213 [4]). In multi-beam operations, the UE assumes that the same PEI is repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the PEI is up to UE implementation.

End of Changes