3GPP TSG-RAN WG2 Meeting #120 												   		R2-2211940

Toulouse, France, November 14-18, 2022

Agenda Item:	8.18.2

Source: 	SONY

Title:	DL SDT triggering and procedures

Document for:	Discussion and Decision

# Introduction

RAN2 already completed the specification of Mobile Originated-Small Data Transmission (MO-SDT) for Rel-17 UEs in RRC_INACTIVE state. The benefits of MO-SDT were to reduce the signalling overhead, latency, and power consumption at the UE when there is an initial uplink packet as well as subsequent UL/DL data transmissions.

With the same benefits as MO-SDT, a new work item (WI) for Mobile Terminated-Small Data Transmission (MT-SDT) has been approved in RAN [1], for Rel-18 UEs in RRC_INACTIVE state, to receive an initial DL packet as well as subsequent DL/UL data transmissions.

Objective of the WI [1]:

- Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3]

- MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RA-SDT and CG-SDT as the UL response;

- MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE.

Note: Data transmission in DL within paging message is not in scope of this WI.

Also, a Study Item on Expanded and improved NR positioning was approved [2]. The study item has several objectives, in which one of them is related to supporting Low Power High Accuracy Positioning (LPHAP):

- Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

In this contribution, we discuss MT-SDT triggering mechanism and procedures for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE state. In addition, we discuss aspect for MT-SDT to support positioning methods relevant for LPHAP devices and scenarios.

## DL SDT triggering and procedures

Triggering mechanism: RAN has already narrowed down the triggering mechanism to be based on paging message for MT-SDT UEs in RRC_INACTIVE state. Therefore, RAN2 discussion should concentrate on how the paging message can be tailored to carry the triggering information and the format of the message.

When a UE is in RRC_INACTIVE state, the UE is usually paged with an Inactive RNTI (I-RNTI) allocated by the last serving gNB. If successful, the UE resumes from RRC_INACTIVE to RRC_CONNECTED state.

The current paging in ASN.1 is encoded as follow [4]:

#### –	Paging

The Paging message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: Network to UE

Paging message

-- ASN1START

-- TAG-PAGING-START

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList                                                        OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                Paging-v1700-IEs                                                        OPTIONAL

}

Paging-v1700-IEs ::=                SEQUENCE {

pagingRecordList-v1700              PagingRecordList-v1700                                                  OPTIONAL, -- Need N

pagingGroupList-r17                 PagingGroupList-r17                                                     OPTIONAL, -- Need N

nonCriticalExtension                SEQUENCE {}                                                             OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList-v1700 ::=          SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1700

PagingGroupList-r17 ::=             SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingRecord-v1700 ::=              SEQUENCE {

pagingCause-r17                     ENUMERATED {voice}      OPTIONAL    -- Need N

}

PagingUE-Identity ::=               CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

fullI-RNTI                          I-RNTI-Value,

...

}

-- TAG-PAGING-STOP

-- ASN1STOP

PagingRecord field descriptions

accessType Indicates whether the Paging message is originated due to the PDU sessions from the non-3GPP access.

pagingRecordList If the network includes pagingRecordList-v1700, it includes the same number of entries, and listed in the same order, as in pagingRecordList (i.e. without suffix).

pagingCause Indicates whether the Paging message is originated due to IMS voice. If this field is present, it implies that the corresponding paging entry is for IMS voice. If upper layers indicate the support of paging cause and if this field is not present but pagingRecordList-v1700 is present, it implies that the corresponding paging entry is for a service other than IMS voice. Otherwise, paging cause is undetermined.

In order to inform a UE that this paging message is for MT-SDT, a triggering information can be added in the paging message, so that the UE does not move to RRC_CONNECTED state but remains in RRC_INACTIVE state and prepares itself to receive MT-SDT.

Therefore, based on the above paging message, there could be few options how to indicate MT-SDT in the paging message:

- A new cause value: In the current spec, the pagingCause indicates whether the Paging message is originated due to IMS voice, otherwise, paging cause is undetermined. Therefore, it may be possible to add a new cause value in the paging record, to trigger MT-SDT arrival.

- A new flag: A flag can be added in the PagingRecord, to indicate whether this paging message is to trigger MT-SDT arrival. If the flag is set, e.g. flag=1, then there is an MT-SDT message for the UE. If the flag is not set, then the UE would perform the legacy RRC Connection process.

- Other methods….

Proposal 1: RAN2 to discuss MT-SDT triggering mechanism for UEs in RRC_INACTIVE, such as a new cause value or new flag in the paging message.

DL SDT procedures: Some of the procedures for UL SDT/MO-SDT can be reused for DL SDT/MT-SDT as there is some similarities between UL SDT and DL SDT, for example as WI captures, existing RA-SDT and CG-SDT can be used as the UL response to the paging message. In our view, the DL SDT can be described as follows:

The network has already moved a UE to INACTIVE state. At some point later, the UE receives RAN paging from the network which may trigger the MT-SDT. Assuming that the paging message contains triggering information (as discussed above) for the arrival of the MT-SDT, the UE checks at least the following conditions:

- At least one radio bearer for DL SDT is configured.

- The measured downlink RSRP/RSRP of the downlink pathloss reference is higher than the configured DL RSRP threshold. This is to exclude cell-edge UEs similar to MO-SDT in Rel-17.

- The UE can also check whether there is an UL SDT data. This may be coincident such that by the time UE received the triggering of DL SDT, there is also an UL SDT data that has just come into the UE’s buffer. The size of the UL data in the buffer should be less than Data Volume Threshold (DVT).

- If Rel-17 CG-SDT resource is configured and valid (i.e., UL time-alignment timer has not expired), the UE should select CG-SDT which includes CCCH message in the first UL transmission.

- If CG-SDT resource is not configured or not valid, the UE response to the network is to select Rel-17 RA-SDT which includes CCCH message in the UL transmission (i.e., Message 3 or Message B).

- If none of the above is met, the UE declares that the conditions to initiate MT-SDT procedure are not fulfilled

  - Then the UE initiates the procedure to move to RRC_CONNECTED state.

- After network receives the UL response via either RA-SDT and CG-SDT, the network can schedule DL data to the UE dynamically.

Note that if RAN paging does not indicate the arrival of MT-SDT, the UE assumes legacy paging procedure and responds to the network based on legacy behaviour/procedure.

Proposal 2: RAN2 to discuss DL SDT/MT-SDT procedures after a UE receives the paging message that triggers the DL SDT/MT-SDT and what can be reused from the Rel-17 UL SDT/MO-SDT procedures.

## MT-SDT for LPHAP support

At the last meeting [3] it was agreed in the positioning session to down prioritize MT-SDT in the positioning study.

However, it would be relevant to discuss some aspect related to Low Power High Accuracy Positioning (LPHAP) related to MT-SDT during the MT-SDT Workitem [1].

We see e.g., about how to configure the UE to perform positioning measurements and reporting, even though the principles of “Low Power Periodic and Triggered 5GC-MT-LR Procedures” [5], should be considered. For this LCS procedure there are a number of variants like “deferred” or scheduled location time, where the UE has been configured/pre-configurated, while in RRC Connected mode (e.g. during Release), in order to at a later stage perform a positioning activity,

But any Network triggered positioning procedure while the UE is in Idle/Inactive mode is currently not standardized, though it is part of the objectives in the Positioning study, it would be more relevant to discuss in the SDT session for Rel-18.

Observation 1: Network triggered MT-LR procedures for UEs in Idle/Inactive is currently not supported.

Observation 2: Network triggered MT-LR procedure using MT-SDT would preferably be discussed in the SDT session.

The MT-SDT procedure would be suitable when network needs to trigger and potentially also configure a UE in RRC Idle mode/RRC-Inactive mode) to perform measurement and/or measurement reporting in LPHAP (Low Power High Accuracy Positioning) scenarios.

Further, in some of scenarios the UE has been configured when in RRC Connected mode, or when being released from Connected mode to Idle/Inactive mode. In some other scenarios the UE is not configured at all, and hence needs both being triggered as well as configured in order to perform the positioning measurements and/or reporting.

Observation 3. Depending on the scenario the UE may have to be configured in Idle/Inactive mode, if not configured while released from Connected mode.

Proposal 3: Both alternatives, configuration triggered via MT-SDT or configured via RRC release should be considered.

Typically, and in most cases, the UE will be configured separately and prior to doing the actual positioning procedure. Then for initiating the positioning procedure the UE needs to be triggered.  Here we believe some wake-up mechanism would be suitable, e.g., using PEI, or paging DCI. For this likely legacy functionality can be used, but some positioning trigger indication would be needed.

Proposal 4: Further discuss wake-up mechanism to trigger UE to initiate positioning procedure or if legacy procedures can be used.

Irrespectively, some sorts of indication would be needed on the triggering message to indicate that the MT-SDT procedure is to trigger the MT-LR procedure.

Proposal 5: Introduce new parameter “positioning trigger indication”, in relation to the MT-SDT procedure to initiate/trigger the positioning procedure/measurements.

## Conclusions

In this contribution, we have discussed MT-SDT triggering mechanism and procedures for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE state. In addition, we have discussed aspect for MT-SDT to support positioning methods relevant for LPHAP devices and scenarios. We have the following observations and proposals:

Proposal 1: RAN2 to discuss MT-SDT triggering mechanism for UEs in RRC_INACTIVE, such as a new cause value and new flag in the paging message.

Proposal 2: RAN2 to discuss DL SDT/MT-SDT procedures after a UE receives the paging message that triggers the DL SDT/MT-SDT and what can be reused from the Rel-17 UL SDT/MO-SDT procedures.

Observation 1: Network triggered MT-LR procedures for UE´s in Idle/Inactive is currently not supported.

Observation 2: Network triggered MT-LR procedure using MT-SDT would preferably be discussed in the SDT session.

Observation 3. Depending on the scenario the UE may have to be configured in Idle/Inactive mode, if not configured wile released from Connected mode.

Proposal 3: Both alternatives, configuration triggered via MT-SDT or configured via RRC release should be considered.

Proposal 4: Further discuss wake-up mechanism to trigger UE to initiate positioning procedure or if legacy procedures can be used.

Proposal 5: Introduce new parameter “positioning trigger indication”, in relation to the MT-SDT procedure to initiate/trigger the positioning procedure/measurements.

## References

[1] RP-213583, “New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR,” RAN#94e.

[2] RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RANP #94e, December 2021

[3] RAN2#119bis-e meeting

[4] TS 38.331 V17.2.0

[5] Functional stage 2 description of Location Services (LCS) (Release 17)