3GPP TSG-RAN WG2 Meeting #120	R2-2212070

Toulouse, France, 14 - 18 November 2022

Agenda item:	8.4.3

Source: 	Huawei, HiSilicon

Title: 	NR-DC selective activation of SCG

Document for:	Discussion and Decision

# 1 Introduction

There were some discussions at the RAN2#119bis-e meeting with the following agreements reached for selective activation of SCG:

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs):

- a.	Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell.

- b.	Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells.

- c.	Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell.

Confirm that “CPA” selective activation of cell groups will be supported for this WI objective

Confirm that we aim to support delta configuration, i.e. that there need to be a known reference.

RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

In this contribution we discuss different aspects and open issues for selective activation of CG.

# 2	Discussion

## 2.1	Applicable Scenarios

In Rel-16 and Rel-17, the following types of CPC were introduced:

-	SN-initiated intra-SN CPC;

-	SN-initiated inter-SN CPC;

-	MN-initiated inter-SN CPC.

For Rel-18 selective activation, we should consider all types of CPC for realization of subsequent CPC. Like for non-conditional mobility since Rel-15,  SN-initiated CPC is used for coverage reasons while MN-initiated CPC is usedfor load balancing, SN initiated CPC is more applicable for the frequent cell group changes in FR2 scenario mentioned in the justification in the WID. So, it makes sense to support SN initiated CPC

However, we should consider the preparation and configuration of measurements and execution conditions for the subsequent CPC execution. Execution conditions refer to the SCG measConfig in SN initiated CPC, whereas in MN initiated CPC the execution conditions refer to the MCG measConfig. When the UE configured with SN-initiated inter-SN CPC performs a PSCell change to a new target SN, the new SCG measConfig is determined by the target UE configuration prepared by the target SN and the previously stored execution conditions for other candidate target PSCells may refer to conditional events that were released or that do not match with the candidate target PSCell. Making subsequent execution of CPC possible requires every candidate target SN to prepare the executions and corresponding measConfig towards every candidate target PSCell prepared by itself or by another candidate target SN.

This requires some inter-node coordination. For instance, the MN could indicate to each candidate target SN the list of candidate cells controlled by other target candidate SNs, so that the candidate target SN configures conditional events for them. However, as additional candidate target SNs are configured, it is necessary to require the previously prepared candidate target SN to provide additional execution conditions.

For intra-SN subsequent CPC no such coordination is needed.

In addition, RAN2 agreed the support of delta-configuration, like for LTM. However, unlike LTM, selective SCG activation can change the CU, which involves additional inter-node signalling. For LTM and for intra-SN selective SCG activation, using delta signalling is already a subject of study in itself. In order to support this also for inter-SN selective SCG activation, the work load will be increased.

Besides, unlike for inter-SN selective activation, there is no need of security key refresh so any CPC configuration could be used several times without any security issue.

For Rel-17 CPC, completing the WI in time has required accepting a number of non-optimal solutions, which may reduce the benefit of the work. The scope of the Rel-18 WI is much larger and we expect a similar situation but on a larger scale, also including the questions of co-existence of the Rel-18 feature.

The work for intra-SN selective SCG activation is still significant and it is a subset of the work for inter-SN selective activation, i.e. it is necessary for both. Therefore, we think it would be a good strategy to prioritize the work on intra-SN selective SCG activation.

Proposal 1: prioritize intra-SN selective SCG activation (work on intra-SN selective SCG activation includes procedures also needed for inter-SN SCG activation, e.g. delta signalling).

In Rel-17 SN-initiated or MN-initiated inter-SN CPC, for each candidate target PSCell selected by the MN or by the source SN, the UE candidate target configuration is fully under the control of the candidate target SN.

If the candidate target SN supports intra-SN selective SCG activation, the candidate target SN may want to configure this feature when the UE moves to one of its cells.

While the candidate target SN could do this after the UE has executed CPC, this would require a further reconfiguration, while avoiding reconfigurations for subsequent CPC is one of the objectives of the WI.

While this would need to be further checked once there is more progress, it probably won't be difficult to allow the candidate target SN, during inter-SN CPC preparation, to provide an SCG configuration that includes

Proposal 2: consider the possibility to configure intra-SN SCG selective activation inside a Rel-17 conditional reconfiguration for inter-SN CPC.

With respect to inter-SN selective SCG activation, we think it is important to first discuss the scenarios to be considered.

Proposal 3: For inter-SN selective SCG activation, discuss whether to consider MN-initiated, candidate target SN-initiated or source SN-initiated preparation.

## 2.2	Solutions

### 2.1.1	Delta signalling design

For intra-SN selective SCG mobility, it can be assumed, like for any PSCell change by RRC reconfiguration, that the UE will perform RLC re-establishment and, for AM RBs, PDCP data recovery. This is different from LTM mobility where it would be possible to avoid such actions when the DU is not changed.

However, delta signalling for the secondaryCellGroup could be similar to delta signalling for LTM, as discussed in [1], and similar CU-DU coordination may be required to support delta signalling.

Proposal 4: Attempt to have a common design of delta signalling for LTM and for selective SCG activation.

### 2.1.2	SN security keys

In Rel-17, upon reconfiguration with sync of the SCG while CPC is configured, the UE and the MN release the CPAC configuration so the CPAC configuration can only be used once. In the Rel-17 CPC preparation procedure, the MN can provide different KSN to different candidate SNs via the SN Addition/Modification Request messages and include the corresponding sk-counter in the corresponding conditional reconfigurations (the same sk-counter value is included for candidate cells belonging to the same SN as a single KSN is provided to each candidate SN).

However, in the Rel-18, the CPAC configuration including the security-related parameters could be used more than once to support the subsequent CPC procedures. When the UE switches to a SN that the UE has never accessed, the KSN derived using KgNB and the sk-counter was not previously used but if the UE switches to a target cell controlled by another node and later goes to a cell controlled by the same SN while KgNB hasn't changed, the UE will derive the same KSN, which does not comply with security requirements according to TS 33.501.

Observation 1: If, after several CPC execution, the UE selects a previously selected PSCell, or even a cell controlled by the same gNB as a previously selected PSCell, according to current key derivation principles, the same KSN will be used, which does not comply with security requirements as specified in TS 33.501.Therefore, to avoid reusing the same KSN when the UE goes back to a previously used SN, the sk-counter or the KgNB should be changed. In this case, the UE need to know which candidate cells belong to the same SN, based on the same sk-counters provided in the CPAC configuration implicitly or based on the explicit indication from the MN. For example, the following methods could be used:

Method 1: Specify a rule for the UE (and the MN) to derive an updated sk-counter when the UE goes back to an SN which has been accessed.

In Rel-17, the UE does not need to maintain the SN Counter after it has computed the KSN since the MN provides the UE with the current SN Counter value when the UE needs to compute a new KSN. In Rel-18, since the MN is not expected to reconfigure the UE after each CPC or CPA procedure, the UE could remember the sk-counters previously used and, when executing CPC to a configuration with a sk-counter that has the same value as previously used, derived a new sk-counter value from the previous value, following a specified rule.

On the network side, the MN could provide updated KSN to the candidate target SNs for the subsequent accesses, or the MN could wait until the UE has executed CPC to provide KSN to the SN that the UE has actually accessed.

Method 2: Provide multiple sk-counter values to the UE for the same CPC configuration, to be used for subsequent access.

This is similar to method 1 except that no rule is specified to derive new sk-counter values, the UE uses the values provided by the MN. On the network side, the MN could provide a corresponding list of KSN values to the candidate target SNs for the subsequent accesses, or the MN could wait until the UE has executed CPC to provide KSN to the SN that the UE has actually accessed.

Method 3: Change the derivation method of KSN so that it can be derived directly by the SN from the previous KSN value.

In the current security mechanism, the KSN used by the SN is derived by the MN and sent to the SN. The input to derive the KSN is the KgNB , which is only known by the UE and by the MN. In Rel-18, when the UE accesses a SN that was previously accessed, the input to derive the new KSN could be the previous KSN used between the UE and the SN, since the previous KSN can be stored by the UE and by the SN.

In any case, all the above methods require SA3 to assess the feasibility and security.

Proposal 5: Ask SA3 whether it is possible to re-use SN keys in inter-SN selective SCG activation and, if not possible, the feasibility of a new mechanism to avoid key reuse if the UE moves back to the same SN.

# 3	Conclusion

In this document, we have the following proposals:

Proposal 1: prioritize intra-SN selective SCG activation (work on intra-SN selective SCG activation includes procedures also needed for inter-SN SCG activation, e.g. delta signalling).

Proposal 2: consider the possibility to configure intra-SN SCG selective activation inside a Rel-17 conditional reconfiguration for inter-SN CPC.

Proposal 3: For inter-SN selective SCG activation, discuss whether to consider MN-initiated, candidate target SN-initiated or source SN-initiated preparation.

Proposal 4: Attempt to have a common design of delta signalling for LTM and for selective SCG activation.

Proposal 5: Ask SA3 whether it is possible to re-use SN keys in inter-SN selective SCG activation and, if not possible, the feasibility of a new mechanism to avoid key reuse if the UE moves back to the same SN.

# 4	References

- RP-2212069, Configuration of candidate target configurations (pre-configurations) for LTM, Huawei, HiSilicon