3GPP TSG-RAN WG2 Meeting #120  	R2-2212101

November 14-November 20, Taulouse, France

Agenda item:	8.6.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Analysis on mobility enhancements for IoT-NTN

WID/SID:	IoT_NTN_enh - Release 18

Document for:	Discussion and Decision

# 1	Introduction

Rel-18 WID [1] for IoT-NTN enhancements includes one objective for mobility enhancements as given below.

Mobility Enhancements: - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC.

Following are the agreements made in RAN2-119e and RAN2-119bis-e meeting for mobility enhancements for IoT-NTN.

RAN2-119bis

- For eMTC over NTN, for both earth-moving and earth-fixed cell scenarios, we introduce location based CHO triggering events

RAN2-119

- IoT NTN can use the mechanism for neighbour cell measurements in connected mode (specified in Rel-17 for NB-IoT). FFS if any enhancements are needed (e.g. triggers) for both NB-IoT and eMTC.

- RAN2 to continue working on a new time-based trigger for triggering intra and inter frequency measurements in connected mode, e.g. the serving cell is going to stop covering the current area, for both earth-moving and earth-fixed cell (FFS on distance-based trigger)

- CHO enhancements for eMTC NTN (i.e. time/timer based solution) are introduced based on the R17 NR NTN solution. FFS on location-based solution

- Measurement results reporting is not supported in Rel-18 NB-IoT NTN.

In this paper, we further analyze the remaining issues to be addressed for introducing connected mode measurements for IoT-NTN and extending the Rel-17 NR-NTN enhancements for eMTC.

# 2	Discussion

## 2.1 Triggering Condition for connected mode measurements for IoT-NTN.

It is agreed to further work on introducing new timer-based trigger for starting intra and inter-frequency measurements. Use of distance-based trigger requires further analysis.  For location based triggering event the main issue indicated in the last meeting is that for evaluation of location UE may need to do GNSS update that will require interruption to the current connection. However, we believe the existing GNSS validity timer can still be used for measurements and there is no need to do GNSS update during connected mode. Because, the UE, which is RRC Connected, has a valid GNSS validity duration such that is can perform uplink pre-compensation. Otherwise it would have to move to RRC Idle according to the release 17 specification. Therefore, the UE need not acquire the GNSS fix again for evaluation of location based event as the  UE has a valid location information available at the time of evaluation.

Observation 1: Evaluation of location/distance-based event can make use of available GNSS Fix which is already valid until validity timer for uplink synchronisation.

Proposal 1: Distance-based triggering condition to start connected mode measurements is supported for IoT-NTN

The connected mode measurements prior to RLF can be configured to start only after the serving cell radio condition goes below the specific threshold where the UE can do the serving cell measurements as relaxed measurements using the same principles used in idle mode with the difference that the relaxed monitoring state may be restarted or continued in connected mode.

For NTN deployment scenario with the moving cell scenario (LEO), the relaxed monitoring of the serving cell defined for the terrestrial system will not be efficient. Because the serving cell strength changes continuously due to moving cell situation so the relaxed monitoring may not even start in idle mode. The same problem will be applicable also in connected mode serving cell measurements. So the relaxed serving cell measurements used prior to starting connected mode measurements needs to be revisited for the LEO scenario. For example, serving cell measurements can be relaxed based on timer value which determines the UE mobility towards the target cell in a deterministic way can be one option.

Proposal 2: For the LEO scenario, the relaxed serving cell measurements in a connected mode need to be modified to improve the effectiveness of measurements for RLF enhancement

## CHO Modifications for eMTC connected mode mobility.

As part of Rel-17 work, NR NTN enhancements for conditional handover (CHO) introduced additional CHO execution conditions based on location and time values over the measurement event for enhanced connected mode mobility. The specification of the CHO events for NR NTN is captured in TS 38.8311 [4] as:

- CondEvent D1: Distance between UE and a reference location referenceLocation1 becomes larger than configured threshold Thresh1 and distance between UE and a reference location referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold Thresh2; - CondEvent T1: Time measured at UE becomes more than configured threshold Thresh1 but is less than Thresh1+duration;

The Rel-18 WID on IoT NTN enhancements [1] includes an objective, noted above, which propose to reuse mobility enhancements from NR NTN for eMTC NTN. In our view, it will be beneficial for RAN2 to focus on re-using the Rel-17 NR NTN CHO enhancements in eMTC NTN, i.e. location-based D1 event and time-based T1 event.

For measurement-based CHO events, i.e. CondEvent A3, CondEvent A4 and CondEvent A5, the UE makes use of RRM measurements to trigger the event conditions. However, for CondEvent D1 and CondEvent T1 the UE relies on the distance between the UE and cells reference location and a time window evaluated by the UE, respectively. Note that in RAN2#117 [5] was agreed for Rel-17 NR NTN that joint configuration of location-based and time-based CHO events will not be supported in Rel-17.

In this paper, the applicability of the events CondEvent D1 and CondEvent T1, are analysed considering cell mobility and UE mobility. The following aims to summarize the advantages and disadvantages of CondEvent D1 and CondEvent T1 events in Quasi-Earth-fixed cells (EFC) and Earth-moving cells (EMC).

### EFC

EFC scenarios are characterized by satellite beams covering one geographical area and providing service for a certain amount of time to that area. The time-based CHO event, i.e. CondEvent T1, is based on the absolute time Thresh1 measured by the UE and Duration as a duration started at T1. Given that the network can predict when the serving cell stops serving the area, time-based CHO events require low complexity for stationary UEs. However, moving UEs configured with time-based CHO events could present sub-optimal mobility performance.

The CondEvent T1 is configured as the CHO execution condition, while measurement-based NR events, e.g. A3/A4/A5, are used for MR triggering. In this context, UEs in the serving cell edge can trigger multiple MR due to low RSRP variability among NTN cells. A clever network can disregard the MR based on the UE location, but it results in undesired signalling overhead.

Observation 2: Time-based CHO event, i.e. CondEvent T1, does not reduce signalling overhead.

UEs configured with CondEvent T1 require to be signalled with time settings Thresh1 and Duration; UEs with mobility, located near the serving cell edge, can lead to a sub-optimal configuration of these parameters. Even those UEs following a predictable route require dedicated configuration of Thresh1 and Duration as compared with stationary UEs. Such UE-specific configuration is only possible if the network is aware of the UE location and its movement direction.

Observation 3: Time-based CHO events will require frequent UE location reporting and UE-specific signaling of Thresh1/Duration.

Proposal 3: The applicability of time-based CHO events for moving UEs should be further studied in RAN2.

The location-based CHO enhancements for Rel-17 NR NTN allow to evaluate CHO execution condition considering distance information. Such triggering condition can be configured with the CondEvent D1, which requires the UE to evaluate the distance between the UE location and two reference locations associated to serving cell and candidate target cell. The reference locations, among other required elements, shall be signalled to the UE as part of the ReportConfigNR information element.

In EFC deployments, moving UEs near the cell edge could be configured with CondEvent D1 to enhance mobility performance. Specially UEs following an unpredictable mobility pattern could be configured with the CondEvent D1 to trigger the CHO execution condition when the UE is sufficiently close to the target cell reference location. Note that UEs are signalled via SIB-19 with t-Service, i.e. the time that the cell stops serving the area. Thus, in EFC, moving UEs could jointly use CondEvent D1 and t-Service to enhance the HO execution while considering when the serving cell will disappear.

Proposal 4: RAN2 should further investigate the joint use of CondEvent D1 and t-Service for moving UEs in EFC.

### EMC

With fixed satellite beam bearing, NTN cells sweeps the Earth in EMC deployments. Since cells are moving at very high-speed relative to Earth, the UE mobility can be disregarded, and the deterministic movement of these cells can be exploited. Location-based CHO events can be exploited in this context. For the UE to evaluate the distance between its own location and serving/target cell reference location, UE location capabilities are required. This requirement is not exclusive of location-based CHO events since an updated UE location is needed for UL pre-compensation and for time-based CHO events (as elaborated above).

Observation 4: UE location capabilities are required, at least, for UL pre-compensation, CondEvent D1 and CondEvent T1.

Time-based CHO events in EMC scenarios present disadvantages as compared with location-based CHO events. Figure 3 shows three UEs randomly distributed within Cell-A. In such context, each UE requires dedicated CondEvent T1 parameters. Furthermore, the network will require updated UE location to estimate the optimal value of Thresh1 and Duration.

Observation 5: In EMC, time-based CHO events require UE-specific Thresh1/Duration and UE location reporting, which results in additional UL signalling as compared with location-based CHO events.

Figure 3 EMC scenario: UEs configured with CondEvent T1 requires UE-specific Thresh1 and Thresh2

The following agreement was taken as part of RAN2#119 [3]:

- CHO enhancements for eMTC NTN (i.e. time/timer based solution) are introduced based on the R17 NR NTN solution. FFS on location-based solution

The objectives in the WI [6] for Rel-18 NR NTN, captures the following:

- Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3]

Time-based CHO events may lead to increased UL signalling as compared with location-based CHO events. Since in IoT NTN signalling overhead is even more critical than in NR NTN, location-based CHO events, which require less UL signalling, shall be introduced for Rel-18 eMTC NTN.

Proposal 5: CHO enhancements for Rel-18 eMTC NTN should be based on the R17 NR NTN solutions, considering, both time-based and location-based CHO events.

For eMTC-NTN, CHO execution condition can be defined as combination of A3/A4/A5 event and location or timer based events for EMC scenario.  This combined evaluation is needed to ensure that execution considers the radio condition changes at UE along with conditions linked to NTN cell mobility. In case of power limited and stationary UE the target cell radio measurements can be skipped if the radio conditions observed over successive execution for the same time/distance trigger remains the same and also the result of CHO execution. The UE may internally decide to consider only the timer/distance-based trigger without radio measurements depending on the estimated stability of these measurements. The UE may resume the radio measurements for evaluation if the CHO execution based on time/distance trigger was not successful earlier.

Proposal 6: RAN2 to consider mechanism where CHO condition is evaluated only against the time/distance-based event skipping radio condition evaluation dynamically based on the history of measurements and CHO execution results.

# 3	Conclusion

Based on the analysis related to the applicability of mobility enhancement features to IOT-NTN, we make the following observations and proposals.

Observation 1: Evaluation of location/distance-based event can make use of available GNSS Fix which is already valid until the validity timer for uplink synchronization.

Proposal 1: Distance-based triggering condition to start connected mode measurements is supported for IoT-NTN.

Proposal 2: For the LEO scenario, the relaxed serving cell measurements in a connected mode need to be modified to improve the effectiveness of measurements for RLF enhancement.

Observation 3: Time-based CHO event, i.e. CondEvent T1, does not reduce signaling overhead.

Observation 4: Time-based CHO events will require frequent UE location reporting and UE-specific signaling of Thresh1/Duration.

Proposal 3: The applicability of time-based CHO events for moving UEs should be further studied in RAN2.

Proposal 4: RAN2 should further investigate the joint use of CondEvent D1 and t-Service for moving UEs in EFC.

Observation5: UE location capabilities are required, at least, for UL pre-compensation, CondEvent D1 and CondEvent T1.

Observation 6: In EMC, time-based CHO events require UE-specific Thresh1/Duration and UE location reporting, which results in additional UL signaling as compared with location-based CHO events.

Proposal 5: CHO enhancements for Rel-18 eMTC NTN should be based on the R17 NR NTN solutions, considering, both time-based and location-based CHO events.

Proposal 6: RAN2 to consider mechanism where CHO condition is evaluated only against the time/distance-based event skipping radio condition evaluation dynamically based on the history of measurements and CHO execution results.

# References

[1] RP-221806, WID on IoT NTN enhancements, 3GPP TSG RAN Meeting #96, June 6 - 9, 2022.

[2] R2-2105224, Enhancements for Re-establishment time reduction, 3GPP RAN2 meeting #114

[3] R2-2208701, Report from Break-out session on R17 NTN, REDCAP and CE 3GPP TSG-RAN WG2 Meeting #119 Online, August 2022.

[4] 3GPP Technical Specification 38.331, version 17.1.0, June 2022.

[5] R2-2203512 Report from Break-out session on R17 NTN, REDCAP and CE 3GPP TSG-RAN WG2 Meeting #117 Online, Feb 21st - Mar 3rd, 2022.

[6] RP-221819, WID on NR NTN enhancements, 3GPP TSG RAN Meeting #96, June 6 - 9, 2022.