3GPP TSG-RAN WG2 Meeting #120	R2-2212113

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.3.2

Source:	Intel Corporation

Title:	Considerations of Cell DTX and DRX

WID/SID:	FS_Netw_Energy_NR

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#119b-e, the followings are agreed for Cell DTX and DRX:

=>	Let’s start with understanding solution in the context of connected

•	Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX / DRX OFF duration.

•	Example 2: gNB is expected to turn off its transmission / reception only for data traffic during Cell DTX / DRX OFF duration (i.e. gNB will still transmit / receive reference signals).

•	Example 3: gNB is expected to turn off its dynamic transmission / reception during Cell DTX / DRX OFF duration (i.e. gNB is expected to still perform periodic transmission / reception, including SPS, CG-PUSCH, SR, RACH, and SRS).

•	Example 4: gNB is expected to only transmit reference signals (e.g. CSI-RS for measurement).

=>	RAN2 assumes that the options above are possible for gNB DTX/DRX behavior and discuss UE RAN2 behavior/impact during the DTX/DRX.

=>	For the purpose of our discussion we will focus on a single UE behavior at any point in time.  FFS if we allow multiple configuration of NW DRX/DTX behaviors.

=>	Periodic DTX is assumed as a baseline.  The gNB provides indication to UE about NW DTX mode/configuration via dedicated dynamic L1/L2 signaling.

=>	Dynamic L1/L2 group signalling from NW to provide NW DTX mode/configuration is also considered in RAN2

=>	It is beneficial to align UE DRX with network DTX and DRX alignment among multiple UEs.  Details are FFS, including UE transmission/reception behavior during DTX.  RAN2 to study the alignment.

During the meeting, RAN2 further discussed the aspects of the Cell DTX and DRX which would require discussion in the next meeting in R2-2211066 [1].

In this contribution, the procedure and signalling aspects and the UE behaviour of the Cell DTX/DRX are further discussed.

# 2	Discussion

## 2.1	Procedure and signalling aspects

As in the TR, the general understanding of the solution is that the configuration for the Cell DTX/DRX are provided by the RRC signalling and when to initiate the use of the configuration can be indicated by dynamic L1/L2 signalling and this is as described below in the TR:

A periodic Cell DTX/DRX (i.e., active and non-active periods) can be configured by gNB via RRC signalling. <<Omitted text>>

The Cell DTX mode/configuration can also be indicated to the UE via dynamic L1/L2 signalling.

The configuration for the Cell DTX/DRX contains RRC parameters that define the Cell DTX/DRX pattern. Like UE C-DRX pattern, the baseline is that “on-duration” of the Cell DTX/DRX pattern is periodic.  On the other hand, unlike UE C-DRX which is configured based on the UE QOS requirement (with some offset to spread the scheduling load) and UE power saving, whether/when to turn on the periodic pattern also depends on the load of the network. The periodic pattern configured by RRC should only be started when the network needs to perform network energy saving mode (e.g. due to low load) so that network has more chance of energy saving and at some point, the network may need to turn off the network energy saving mode by stop applying the periodic pattern configured by the RRC (e.g. when the load becomes normal).

To achieve maximum network energy saving, there is a need for the network to indicate the start and stop applying of the Cell DTX/DRX quickly, but without releasing the Cell DTX/DRX configuration.  Hence having a L1/L2 indication to start and stop applying the Cell DRX/DRX seems to be beneficial.  As such configuration is not just for a UE but most likely to all or a group of UEs in a cell, an efficient method to signal such indication to prevent signalling overhead is needed. Therefore, a group configuration update mechanism seems useful. Without such a group configuration update mechanism, the network has to signal the change to each of the UE in RRC Connected mode via dedicated signalling and hence does not allow the network to bring its desired network energy efficiency in a timely manner.  This may also result in huge signalling overhead and thus does not provide an efficient operation.

Observation#1: Unlike UE C-DRX which is configured based on the UE QOS requirement (with some offset to spread the scheduling load) and UE power saving, whether/when to turn on the periodic cell DTX/DRX pattern depends also on the load of the network. The periodic pattern configured by RRC should only be started when the network needs to perform network energy saving mode (e.g. due to low load) so that network has more chance of energy saving and at some point, the network may need to turn off the network energy saving mode by stopping the application of the periodic pattern configured by the RRC.

Observation#1-1: To achieve maximum network energy saving, there is a need for the network to indicate the start and stop applying of the Cell DTX/DRX quickly and frequently without releasing the Cell DTX/DRX configuration

Proposal#1: RAN2 to agree to have a L1/L2 indication to start and stop applying the Cell DTX/DRX pattern. This signalling allows for faster and frequently change between applying and not applying Cell DTX/DRX pattern without releasing the Cell DTX/DRX configuration provided by the gNB via RRC in the UE.

When the cell goes through DTX/DRX, it will be applicable for all or group of UEs in the cell.  Hence a group indication will be more efficient over providing UE dedicated indications for all the UEs in the cell.

Observation#2: As such L1/L2 indication is not just for a UE but most likely to all or a group of UEs in a cell, an efficient method to signal such indication to prevent signalling overhead and to bring its desired network energy efficiency in a timely manner.

Proposal#2: UE group L1/L2 indication is supported for the indication of the Cell DTX/DRX in Proposal 1 on top of the agreed dedicated L1/L2 indication.

As on whether to support multiple configurations of Cell DTX/DRX, the UE needs to be configured with at least one Cell DTX/DRX configuration (i.e. turn on Cell DTX/DRX) when the gNB load is low in addition to the normal configuration.  This normal configuration applies when no DTX/DRX configuration is applied (i.e. turn off Cell DTX/DRX), e.g. when the gNB load is high and/or QoS requirements of the UEs are not satisfied. Typical gNB operation may serve different loading conditions, such as low, moderate, or high load and correspondingly system resource utilization would vary. Without further evaluation on whether there is a need for more than one Cell DTX/DRX configuration for, say moderate load (i.e. more than 1 Cell DTX/DRX configurations for different gNB load levels) and while further granularity is needed for different QoS requirement of the UEs in the cell, it is difficult to decide whether more than 1 Cell DTX/DRX configurations are needed.

Observation#3: Whether there is a need to have for more than 1 Cell DTX/DRX configurations for gNB load other than low load, more discussion is needed in the WI phase with RAN1 based on RAN1 evaluations.

Proposal#3: To determine whether more than 1 Cell DTX/DRX configurations are needed to handle different gNB load levels, more discussion and evaluation is needed. This evaluation can be worked on in the WI phase with RAN1.

The procedure overview of the Cell DTX/DRX is shown below in Figure 1 for UE in RRC Connected mode:

Figure 1: High level view of the Cell DTX/DRX

Consider UE enters connected mode indicating it supports Cell DTX/DRX

Step 1: gNB configures the UE with one or more Cell DTX/DRX configurations (e.g. periodic pattern) in advance. If multiple configurations are supported, each configuration is identified by a configuration ID

Step 2: When the load is low and the Cell DTX/DRX configuration can meet the QoS of the UEs or group of UEs in the cell, the gNB triggers the UE to start applying Cell DTX/DRX configurations (i.e. the Cell DTX.DRX pattern) using the dynamic (group) L1/L2 indication. For the case multiple configurations are supported, the indication includes the configuration ID.

Step 3: When the load is normal and/or the Cell DTX/DRX configuration is no longer acceptable to the QoS of the UEs or group of UEs in the cell, the gNB indicates to the UE to stop applying Cell DRX/DRX configurations (i.e. the Cell DTX/DRX pattern) using the dynamic (group) L1/L2 indication.

It is proposed that RAN2 include the above procedure overview and steps in the TR.

Proposal#4: Include the above Figure 1 and procedure for Cell DTX/DRX in the TP Section 6.1.1.x of the draft TR

## 2.2	UE behaviours

In the last RAN2 meeting, the following examples of gNB behaviours are included into the draft TR as follow:

- Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods.

- Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals)

- Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

- Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

Example 1 provides the maximum network energy gain since no transmission and reception are occurring during the Cell DRX/DRX non-active periods. As indicated in Section 2.1, to perform the Cell DTX/DRX configuration, the configuration also has to meet the UEs’ QoS and RRM requirement in the cell (e.g. all the UEs or group of UEs in the cell have delay tolerant traffic QoS (e.g. background traffic that are delay tolerant) and the UEs do not need to perform much RRM and scheduling measurement (e.g. stationary or pseudo-stationary)).

Observation#4: Example 1 provides the maximum network energy gain and seems to be beneficial if the configuration also meets the UEs’ QoS and RRM requirement in the cell.

Examples 2 and 4 are assuming that all the UEs or group of UEs in the cell need to perform RRM and/or DL/UL scheduling measurement due to mobility.  It is unclear how much network energy gain can be achieved with network still having to transmit and/or receive reference signal. Some RAN1 evaluations are needed.

Observation#5: For Examples 2 and 4, it is unclear how much network energy gain can be achieved with network still having to transmit and/or receive reference signal. RAN1 evaluations are needed.

For Example 3, the gNB can already turn off its dynamic data transmission/reception by implementation without Cell DTX/DRX and the UE can avoid monitoring for dynamic data transmission and reception if the UE’s C-DRX is aligned with the Cell DTX/DRX pattern. Hence, we do not see the need of Example 3.

Observation#6: For Example 3, gNB can already do that without Cell DTX/DRX and the UE can avoid monitoring for dynamic data transmission and reception if the UE’s C-DRX is aligned with the Cell DTX/DRX pattern

Based on Observation#4 to #6, it is proposed that RAN2 focus on implementing Example 1:

Proposal#5: RAN2 to initially focus on Example 1 in the TR for Cell DTX/DRX and assume that gNB turns off all the transmissions and reception during the non-active period.

When Example 1 is expected and periodic pattern configuration is configured to the UE and UE is indicated to apply the periodic cell DTX/DRX pattern, UE is not required to monitor the occasions and shall not transmit on the occasions that fall outside of the Cell DTX/DRX active time. In some cases, some critical signal/channel transmission periodicity e.g., PDCCH monitoring for receiving system information, PDCCH monitoring for receiving paging early indication, TRS, SSB, SRS occasions may not end in the Cell DTX/DRX active period. For such cases, it is expected that gNB may align these critical signal/channel transmission/reception occasions during the active time.  Such alignment can be performed when gNB indicates to the UE to apply the cell DTX/DRX pattern and the UE can apply the reconfiguration (provided in Step 1 in Figure 1 together with the Cell DTX/DRX configuration) to identify the valid monitoring occasions in DL and/or transmission occasions in UL of those critical signal/channel. For example, the reconfiguration may include one or more shifts to be applied to one or more monitoring or transmission occasions of existing configuration (i.e., new occasions are derived from existing configuration) or may include explicit configurations related to occasions to arrive at the transmission or reception occasions within the Cell DTX/DRX active period.

Observation#7: For UE behaviour in Example 1, the UE is not required to monitor the occasions and shall not transmit on the occasions that fall outside gNB’s active time. In addition, the UE may need to apply additional configurations or rules (provided in Step 1 in Figure 1 together with the Cell DTX/DRX configuration) to identify the valid monitoring occasions in DL and/or transmission occasions in UL to align to the active time of the Cell DTX/DRX (e.g., changes to the TRS monitoring occasions, shifting UL signalling such as SR over PUCCH, SRS etc.).

For normal PDCCH monitoring for dynamic transmission and reception, the UE performs C-DRX and monitors PDCCH monitoring only during the on-duration.  This may not align to the active period of the Cell DTX/DRX and thus there is also a need for ensuring alignment.

Observation#8: Alignment of the UE’s C-DRX configuration can be part of the additional configurations or rules to identify the valid monitoring occasions in DL.

Proposal#6: For UE behaviour in Example 1 in the draft TR [2]:

- UE is not required to monitor the occasions and shall not transmit on the occasions during Cell DTX/DRX non-active period (i.e., fall outside of the gNB’s active time).

- If configured by the gNB in Step 1 of Figure 1, UE may need to apply additional configurations or rules to identify the valid monitoring occasions in DL and/or transmission occasions in UL to align to the active time of the Cell DTX/DRX (e.g., changes to the TRS monitoring occasions, shifting UL signalling such as SR over PUCCH, SRS, aligning of the C-DRX on-duration with the Cell DTX/DRX active period)

# 4	Conclusion

It is requested that RAN2 agree to the proposals and observations below:

Observation#1: Unlike UE C-DRX which is configured based on the UE QOS requirement (with some offset to spread the scheduling load) and UE power saving, whether/when to turn on the periodic cell DTX/DRX pattern depends also on the load of the network. The periodic pattern configured by RRC should only be started when the network needs to perform network energy saving mode (e.g. due to low load) so that network has more chance of energy saving and at some point, the network may need to turn off the network energy saving mode by stopping the application of the periodic pattern configured by the RRC.

Observation#1-1: To achieve maximum network energy saving, there is a need for the network to indicate the start and stop applying of the Cell DTX/DRX quickly and frequently without releasing the Cell DTX/DRX configuration

Proposal#1: RAN2 to agree to have a L1/L2 indication to start and stop applying the Cell DTX/DRX pattern. This signalling allows for faster and frequently change between applying and not applying Cell DTX/DRX pattern without releasing the Cell DTX/DRX configuration provided by the gNB via RRC in the UE.

Observation#2: As such L1/L2 indication is not just for a UE but most likely to all or a group of UEs in a cell, an efficient method to signal such indication to prevent signalling overhead and to bring its desired network energy efficiency in a timely manner.

Proposal#2: UE group L1/L2 indication is supported for the indication of the Cell DTX/DRX in Proposal 1 on top of the agreed dedicated L1/L2 indication.

Observation#3: Whether there is a need to have for more than 1 Cell DTX/DRX configurations for gNB load other than low load, more discussion is needed in the WI phase with RAN1 based on RAN1 evaluations.

Proposal#3: To determine whether more than 1 Cell DTX/DRX configurations are needed to handle different gNB load levels, more discussion and evaluation is needed. This evaluation can be worked on in the WI phase with RAN1.

Proposal#4: Include the procedure for Cell DTX/DRX in the TP Section 6.1.1.x of the draft TR [2]

Observation#4: Example 1 provides the maximum network energy gain and seems to be beneficial if the configuration also meet the UEs’ QoS and RRM requirement in the cell.

Observation#5: For Examples 2 and 4, it is unclear how much network energy gain can be achieved with network still having to transmit and/or receive reference signal. RAN1 evaluations are needed.

Observation#6: For Example 3, gNB can already do that without Cell DTX/DRX and the UE can avoid monitoring for dynamic data transmission and reception if the UE’s C-DRX is aligned with the Cell DTX/DRX pattern

Proposal#5: RAN2 to initially focus on Example 1 in the TR for Cell DTX/DRX and assume that gNB turns off all the transmissions and reception during the non-active period.

Observation#7: For UE behaviour in Example 1, the UE is not required to monitor the occasions and shall not transmit on the occasions that fall outside gNB’s active time. In addition, the UE may need to apply additional configurations or rules (provided in Step 1 in Figure 1 together with the Cell DTX/DRX configuration) to identify the valid monitoring occasions in DL and/or transmission occasions in UL to align to the active time of the Cell DTX/DRX (e.g., changes to the TRS monitoring occasions, shifting UL signalling such as SR over PUCCH, SRS etc.).

Observation#8: Alignment of the UE’s C-DRX configuration can be part of the additional configurations or rules to identify the valid monitoring occasions in DL.

Proposal#6: For UE behaviour in Example 1 in the draft TR [2]:

- UE is not required to monitor the occasions and shall not transmit on the occasions during Cell DTX/DRX non-active period (i.e., fall outside of the gNB’s active time).

- If configured by the gNB in Step 1 of Figure 1, UE may need to apply additional configurations or rules to identify the valid monitoring occasions in DL and/or transmission occasions in UL to align to the active time of the Cell DTX/DRX (e.g., changes to the TRS monitoring occasions, shifting UL signalling such as SR over PUCCH, SRS, aligning of the C-DRX on-duration with the Cell DTX/DRX active period)

The updated TP for 6.1.1 Cell DTX/DRX is provided in Section 5.

# 5	TP updates for 6.1.1 Cell DTX/DRX [2]

### 6.1.1	Cell DTX/DRX

#### 6.1.1.x	Higher layer procedures

Cell DTX/DRX is applied to at least UEs in RRC_CONNECTED state. A periodic Cell DTX/DRX (i.e., active and non-active periods) can be configured by gNB via RRC signalling. The procedure overview of the Cell DTX/DRX is shown below in Figure 1 for UE in RRC Connected mode:

Figure 1: High level view of the Cell DTX/DRX

Consider UE enters connected mode indicating it supports Cell DTX/DRX

Step 1: gNB configures the UE with one or more Cell DTX/DRX configurations (e.g. periodic pattern) in advance. If multiple configurations are supported, each configuration is identified by a configuration ID

Step 2: When the load is low and the Cell DTX/DRX configuration can meet the QoS of the UEs or group of UEs in the cell, the gNB triggers the UE to start applying Cell DTX/DRX configurations (i.e. the Cell DTX.DRX pattern) using the dynamic (group) L1/L2 indication. For the case multiple configurations are supported, the indication includes the configuration ID.

Step 3: When the load is normal and/or the Cell DTX/DRX configuration is no longer acceptable to the QoS of the UEs or group of UEs in the cell, the gNB indicates to the UE to stop applying Cell DRX/DRX configurations (i.e. the Cell DTX/DRX pattern) using the dynamic (group) L1/L2 indication.

Below examples on Cell DTX/DRX behaviour during non-active periods are assumed to be possible options, and the UE behaviour/impact will be studied:

- Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods.

- Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals)

- Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

- Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

The study will focus on UE behavior for Example 1. With Example 1, UE is not required to monitor the occasions and shall not transmit on the occasions during Cell DTX/DRX non-active period (i.e., fall outside gNB’s active time). At any point in time the cell activates a single DTX/DRX configuration. Multiple sets of Cell DRX/DTX configuration can be considered if it can be shown to have benefits.

The Cell DTX mode/configuration can also be indicated to the UE via dynamic L1/L2 signalling. The dynamic L1/L2 signalling supports UE dedicated and UE group common indication.

It is beneficial to align UE DRX with Cell DTX and DRX alignment among multiple UEs. If configured by the gNB in Step 1 of Figure 1, UE may need to apply additional configurations or rules to identify the valid monitoring occasions in DL and/or transmission occasions in UL to align to the active time of the Cell DTX/DRX (e.g., changes to the TRS monitoring occasions, shifting UL signalling such as SR over PUCCH, SRS, aligning of the C-DRX on-duration with the Cell DTX/DRX active period)

# 6	References

[1] R2-2211066 Report of [POST119bis][303][NES] TP on NW DTX/DRX, Huawei/Apple

[2] R2-2211067 TR on CellDTX/DRX to TR38.864