3GPP TSG-RAN WG2 #120	Tdoc R2-2212147

Toulouse, France, 14 – 18 November, 2022

Agenda Item:	6.24.1

Source:	Ericsson

Title:	On FR2 FBG5

Document for:	Discussion

# 1	Introduction

In this document, we discuss aspects on introducing new FR2 band combination fallback group FBG5 (with new BW classes R2-R12), and new field for Max Aggregated Bandwidth, as requested by RAN4 in LS [1].

# 2	Discussion

## 2.1	Advertising additional perCC capabilities with existing signalling

During RAN2#119e-bis discussions, there seemed to be some confusion on what possibilities existing signalling offers for the UE to indicate additional or “better” UE capabilities for a cell/CC of a lower-order BC (i.e. a BC with fewer CC:s than the advertised “parent” BC). In NR UE capability signalling, there are two options:

- The UE can advertise the lower-order BC explicitly with associated FeatureSetCombinations and linked FeatureSets.

- The UE can for the “parent” BC

  - indicate additional FeatureSetCombinations, but with fewer linked FeatureSets (for inter-band and intra-band non contiguous BCs), or

  - indicate additional FeatureSets, but with fewer linked FeatureSetsPerCC (for (intra-band contiguous BCs).

This is specified in TS 38331, subclause 6.3.3, FeatureSetCombination IE description:

NOTE 1:  The UE may advertise fallback band-combinations in which it supports additional functionality explicitly in two ways: Either by setting FeatureSet IDs to zero (inter-band and intra-band non-contiguous fallback) and by reducing the number of FeatureSet-PerCC Ids in a Feature Set (intra-band contiguous fallback). Or by separate BandCombination entries with associated FeatureSetCombinations.

NOTE 2:  The UE may advertise a FeatureSetCombination containing only fallback band combinations. That means, in a FeatureSetCombination, each group of FeatureSets across the bands may contain at least one pair of FeatureSetUplinkId and FeatureSetDownlinkId which is set to 0/0.

However, it should be clear that “additional functionality” in NOTE1 above means “better capabilities”. I.e., an additional FS/FSpCC entry can enable additional functionality for fewer/narrower carriers. But it cannot reduce/narrow down what the “first” FS/FSpCC allows already.

If companies have concerns with the quoted text, the wording can be adjusted, but we ask RAN2 to confirm these existing options.

RAN2 to confirm the existing two options for a UE to advertise additional functionality for lower-order band combinations as indicated in NOTE1 and NOTE2.

## 2.2	Usefulness of new max aggregated BW field

In [1], RAN4 propose to introduce a new IE Max Aggregated bandwi

In general, efficient signalling of UE-supported band combinations are beneficial for the air interface signalling, but also for the UE to compose the signalling and for the gNB to analyse the signalling.  As Network vendor, we can confirm that the computational effort in gNB to carve through all BBandCombinations/BandParameters and related FeatureSetCombination/FeatureSets/FeatureSetPerCC is also an issue.

In NR, FeatureSetCombination/FeatureSets/FeatureSetPerCC is an efficient mechanism to reduce UE capability signalling size by reusing (linking to) a FeatureSetCombination in multiple Band Combinations of different bands. This reduced the need for UE to advertise multiple instances of the same band combination. Instead, the number of channel bandwidths and flexible bandwidth support means that the number of FeatureSetCombinations/FeatureSets/FeatureSetsPerCC is the main contributor to the signalling size and complexity.

In Appendix, we present data collected at recent IODT activity. The data shows that this UE, to advertise an aggregated BW of 300 MHz for a 4-cc FR1 band combination, indicates 21 FeatureSet Entries.  Having an IE to limit max aggregated BW will for sure simplify the capability reporting and the processing thereof in the gNB.

In FR2 FBG5, since there are fewer bandwidths (100 and 200 MHz), the signalling gain with a new Max Aggregated BW is consequently less. The tables below show the FeatureSets to indicate support of R12 with a Max Aggregated Bandwidth of 1200 MHz and 1600 MHz, respectively.

|  | CC1 | CC2 | CC3 | CC4 | CC5 | CC6 | CC7 | CC8 | CC9 | CC10 | CC11 | CC12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FS1 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| FS2 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 200 |  |
| FS3 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 200 | 200 |  |  |
| FS4 | 100 | 100 | 100 | 100 | 100 | 100 | 200 | 200 | 200 |  |  |  |
| FS5 | 100 | 100 | 100 | 100 | 200 | 200 | 200 | 200 |  |  |  |  |
| FS6 | 100 | 100 | 200 | 200 | 200 | 200 | 200 |  |  |  |  |  |
| FS7 | 200 | 200 | 200 | 200 | 200 | 200 |  |  |  |  |  |  |

Table 1. FeatureSets to indicate Max Aggr bandwidth 1200 MHz for R12

|  | CC1 | CC2 | CC3 | CC4 | CC5 | CC6 | CC7 | CC8 | CC9 | CC10 | CC11 | CC12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FS1 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 200 | 200 | 200 | 200 |
| FS2 | 100 | 100 | 100 | 100 | 100 | 100 | 200 | 200 | 200 | 200 | 200 |  |
| FS3 | 100 | 100 | 100 | 100 | 200 | 200 | 200 | 200 | 200 | 200 |  |  |
| FS4 | 100 | 100 | 200 | 200 | 200 | 200 | 200 | 200 | 200 |  |  |  |
| FS5 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 |  |  |  |  |

Table 2 . FeatureSets to indicate Max Aggr bandwidth 1600 MHz for R12

With a new Max aggregated BW field, UE would indicate support of R12 with e.g. these FeatureSets for 1200 and 1600 Max Aggregated bandwidth, respectively:

|  | CC1 | CC2 | CC3 | CC4 | CC5 | CC6 | CC7 | CC8 | CC9 | CC10 | CC11 | CC12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FS1 | 100 | 100 | 100 | 100 | 100 | 100 | 200 | 200 | 200 | 200 | 200 | 200 |

|  | CC1 | CC2 | CC3 | CC4 | CC5 | CC6 | CC7 | CC8 | CC9 | CC10 | CC11 | CC12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FS1 | 100 | 100 | 100 | 100 | 200 | 200 | 200 | 200 | 200 | 200 | 200 | 200 |

Another advantage of introducing a new field for the Max aggregated BW is that the gNB implementation could use this as an indication that no more FeatureSets

As discussed above we see clear advantages of introducing a new field for Max aggregated bandwidth as proposed by RAN4 in their LS [1] for FR2 FBG5.

Introduce new field for Max aggregated bandwidth for FR2 FBG5 BC capability signalling.

As we illustrated above, the gain would be even more substantial for FR1. We therefore ask RAN2 to consider introduce new field for Max aggregated bandwidth also for FR1.

RAN2 to consider to introduce new field for Max aggregated bandwidth for FR1 BC capability signalling.

## 2.3	BW consideration with gNBs

Clearly, the new bandwidth classes R2-R12 are not supported by existing gNBs, they need the existing FR2 FBGs and BW classes On the other hand, and as we discussed before, all gNBs of a network could be upgraded before supporting UEs expects to enter the market, hereby avoiding any problem with non-supporting gNBs. Still, from signalling size and complexity aspect, RAN2 should consider to introduce a capability filter, such that FR2 legacy BW classes (and related FeatureSetCombinatins/FeatureSets/FeatureSeetsPerCC) are omitted (unless UE supports anything better with the legacy FR2 BW classes) when FBG5 BW class is reported.

Introduce capability filter such that FR2 legacy BW classes are omitted from BC capability signalling if FBG5 BW class is reported.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm the existing two options for a UE to advertise additional functionality for lower-order band combinations as indicated in NOTE1 and NOTE2.

Proposal 2	Introduce new field for Max aggregated bandwidth for FR2 FBG5 BC capability signalling.

Proposal 3	RAN2 to consider to introduce new field for Max aggregated bandwidth for FR1 BC capability signalling.

Proposal 4	Introduce capability filter such that FR2 legacy BW classes are omitted from BC capability signalling if FBG5 BW class is reported.

# References

[1]		R2-2209347		LS on new contiguous BW classes for legacy networks	RAN4 (To: RAN2)

[2]		R2-2211009		Summary of email discussion [AT119bis-e][010][NR17] FBG5 BW Classes												Qualcomm Incorporated

# Appendix

DL: n48(2A)n77(2A)	Inter-band and intra-band non-contiguous CA

BC#96    FSC#82    n48 a/a    n48 a/-    n77 a/-    n77 a/-    BCS'11101'

FSE#1     n5/5       n5/0       n8/0       n11/0      1x(30kHz 100MHz 4L) / 1x(30kHz 100MHz 1L)    1x(30kHz 100MHz 4L) / -    1x(30kHz 80MHz 4L) / -    1x(30kHz 20MHz 4L) / -

FSE#2     n5/5       n5/0       n9/0       n9/0       1x(30kHz 100MHz 4L) / 1x(30kHz 100MHz 1L)    1x(30kHz 100MHz 4L) / -    1x(30kHz 50MHz 4L) / -    1x(30kHz 50MHz 4L) / -

FSE#3     n5/5       n8/0       n5/0       n11/0      1x(30kHz 100MHz 4L) / 1x(30kHz 100MHz 1L)    1x(30kHz 80MHz 4L) / -    1x(30kHz 100MHz 4L) / -    1x(30kHz 20MHz 4L) / -

FSE#4     n5/5       n8/0       n8/0       n10/0      1x(30kHz 100MHz 4L) / 1x(30kHz 100MHz 1L)    1x(30kHz 80MHz 4L) / -    1x(30kHz 80MHz 4L) / -    1x(30kHz 40MHz 4L) / -

FSE#5     n5/5       n9/0       n5/0       n9/0       1x(30kHz 100MHz 4L) / 1x(30kHz 100MHz 1L)    1x(30kHz 50MHz 4L) / -    1x(30kHz 100MHz 4L) / -    1x(30kHz 50MHz 4L) / -

FSE#6     n5/5       n10/0      n8/0       n8/0       1x(30kHz 100MHz 4L) / 1x(30kHz 100MHz 1L)    1x(30kHz 40MHz 4L) / -    1x(30kHz 80MHz 4L) / -    1x(30kHz 80MHz 4L) / -

FSE#7     n5/5       n11/0      n5/0       n8/0       1x(30kHz 100MHz 4L) / 1x(30kHz 100MHz 1L)    1x(30kHz 20MHz 4L) / -    1x(30kHz 100MHz 4L) / -    1x(30kHz 80MHz 4L) / -

FSE#8     n8/6       n5/0       n5/0       n11/0      1x(30kHz 80MHz 4L) / 1x(30kHz 80MHz 1L)    1x(30kHz 100MHz 4L) / -    1x(30kHz 100MHz 4L) / -    1x(30kHz 20MHz 4L) / -

FSE#9     n8/6       n5/0       n8/0       n10/0      1x(30kHz 80MHz 4L) / 1x(30kHz 80MHz 1L)    1x(30kHz 100MHz 4L) / -    1x(30kHz 80MHz 4L) / -    1x(30kHz 40MHz 4L) / -

FSE#10    n8/6       n8/0       n5/0       n10/0      1x(30kHz 80MHz 4L) / 1x(30kHz 80MHz 1L)    1x(30kHz 80MHz 4L) / -    1x(30kHz 100MHz 4L) / -    1x(30kHz 40MHz 4L) / -

FSE#11    n8/6       n8/0       n8/0       n9/0       1x(30kHz 80MHz 4L) / 1x(30kHz 80MHz 1L)    1x(30kHz 80MHz 4L) / -    1x(30kHz 80MHz 4L) / -    1x(30kHz 50MHz 4L) / -

FSE#12    n8/6       n9/0       n8/0       n8/0       1x(30kHz 80MHz 4L) / 1x(30kHz 80MHz 1L)    1x(30kHz 50MHz 4L) / -    1x(30kHz 80MHz 4L) / -    1x(30kHz 80MHz 4L) / -

FSE#13    n8/6       n10/0      n5/0       n8/0       1x(30kHz 80MHz 4L) / 1x(30kHz 80MHz 1L)    1x(30kHz 40MHz 4L) / -    1x(30kHz 100MHz 4L) / -    1x(30kHz 80MHz 4L) / -

FSE#14    n8/6       n11/0      n5/0       n5/0       1x(30kHz 80MHz 4L) / 1x(30kHz 80MHz 1L)    1x(30kHz 20MHz 4L) / -    1x(30kHz 100MHz 4L) / -    1x(30kHz 100MHz 4L) / -

FSE#15    n9/7       n5/0       n5/0       n9/0       1x(30kHz 50MHz 4L) / 1x(30kHz 50MHz 1L)    1x(30kHz 100MHz 4L) / -    1x(30kHz 100MHz 4L) / -    1x(30kHz 50MHz 4L) / -

FSE#16    n9/7       n8/0       n8/0       n8/0       1x(30kHz 50MHz 4L) / 1x(30kHz 50MHz 1L)    1x(30kHz 80MHz 4L) / -    1x(30kHz 80MHz 4L) / -    1x(30kHz 80MHz 4L) / -

FSE#17    n9/7       n9/0       n5/0       n5/0       1x(30kHz 50MHz 4L) / 1x(30kHz 50MHz 1L)    1x(30kHz 50MHz 4L) / -    1x(30kHz 100MHz 4L) / -    1x(30kHz 100MHz 4L) / -

FSE#18    n10/2      n5/0       n8/0       n8/0       1x(30kHz 40MHz 4L) / 1x(30kHz 40MHz 1L)    1x(30kHz 100MHz 4L) / -    1x(30kHz 80MHz 4L) / -    1x(30kHz 80MHz 4L) / -

FSE#19    n10/2      n8/0       n5/0       n8/0       1x(30kHz 40MHz 4L) / 1x(30kHz 40MHz 1L)    1x(30kHz 80MHz 4L) / -    1x(30kHz 100MHz 4L) / -    1x(30kHz 80MHz 4L) / -

FSE#20    n11/8      n5/0       n5/0       n8/0       1x(30kHz 20MHz 4L) / 1x(30kHz 20MHz 1L)    1x(30kHz 100MHz 4L) / -    1x(30kHz 100MHz 4L) / -    1x(30kHz 80MHz 4L) / -

FSE#21    n11/8      n8/0       n5/0       n5/0       1x(30kHz 20MHz 4L) / 1x(30kHz 20MHz 1L)    1x(30kHz 80MHz 4L) / -    1x(30kHz 100MHz 4L) / -    1x(30kHz 100MHz 4L) / -