3GPP TSG-RAN WG2 Meeting #120                        	R2-2212316

Toulouse, France, November 14-18, 2022

Agenda item:	6.8

Source:	Samsung

Title:	Discussion on postponed issue for slice specific cell reselection

Document for:	Discussion & Decision

# 1	Introduction

This contribution discusses an issue for slice specific cell reselection which was postponed during RAN2#119bis-e meeting. The issue is about the slice availability for NSAG-Frequency pair only present in dedicated signalling

# 2	Discussion

## 2.1	Slice availability for NSAG-Frequency pair present only in dedicated signalling

A UE performs slice based cell reselection using various information as given below in TS 38.304:

The UE derives reselection priorities for slice-based cell reselection by using:

-	NSAGs and their priorities provided by NAS,

-	sliceInfoList and or sliceInfoListDedicated per frequency with nsag-CellReselectionPriority per NSAG, if provided in system information and/or dedicated signalling,

-	cellReselectionPriority per frequency provided in system information and/or dedicated signalling.

SliceInfoList broadcasted in SIB16 of the serving cell can contain slice specific cell reselection priorities and the slice availability lists, while sliceInfoListDedicated in the RRC Release can contain only slice specific cell reselection priorities. If the UE is provided with sliceInfoListDedicated, it ignores the cell reselection priorities in system information. But the UE provided with slice specific cell reselection priorities for certain NSAGs per frequency in dedicated signalling, still relies on the slice availability list in the SIB16 to decide whether a certain NSAG is supported in a cell while calculating the cell reselection priority.

Observation 1: A UE provided with slice based cell reselection priorities in RRCRelease uses the slice availability lists in SIB16.

In the current specifications, the UE behaviour during various scenarios like when NAS doesn’t provide NSAGs and their priorities or RRC doesn’t provide cellReselectionPriority for a frequency or when RRC provides cellReselectionPriority through both system information and dedicated signalling are clearly specified. But the UE behaviour due to the absence of slice availability for some NSAGs or frequencies which are present in dedicated signalling and not present in system information is not specified.

We observe that there are three cases where UE can’t determine the slice availability for the NSAGs and frequencies whose priorities are provided through dedicated signaling.

- SIB16 is not broadcasted in the serving cell but the UE has received freqPriorityListDedicatedSlicing in RRC Release.

- SIB16 is broadcasted, but a frequency in FreqPriorityListDedicatedSlicing is not present in FreqPriorityListSlicing in SIB16.

- FreqPriorityListSlicing is available in SIB16 for all the frequencies in FreqPriorityListDedicatedSlicing, but an nsag-id for a frequency in FreqPriorityListDedicatedSlicing is not present in the FreqPriorityListSlicing in SIB16 for that frequency.

These scenarios can happen commonly, especially during the new deployment of a network or the new deployment of a particular slice in the network.

During RAN2119#e meeting, we discussed them in the offline discussion R2-2208733. There were quite divergent views during the discussion where some companies thought that gNB should ensure that slice based cell reselection priorities should be provided in RRC Release only for the NSAGs and frequencies configured through SIB16, while some of the companies though such a restriction is neither feasible nor desirable. There were also two views on the UE behavior in case when the NSAG-frequency pair in dedicated signaling is not configured in the system information. While some companies have expressed that slice based cell reselection should not be performed for such NSAG-frequency pairs as UE can’t determine the slice availability, there were some companies which thought that UE should perform slice based cell reselection considering that such an NSAG is available in all cells in the frequency, thereby bypassing the slice availability check in the cell for the NSAG-frequency pair.

So we postponed the issue in RAN2#119-e as below

5: P1, P2, and P3 in R2-2208519 are postponed. Can discuss if the gNB implementation always provides dedicated slice information only for frequencies/NSAGs in SIB16? If not, what should be the applicable UE behaviour?

Now let us consider if the gNB implementation can always provide dedicated slice priorities only for frequencies/NSAGs in system information of all the cells where the UE can move while dedicated slice priorities are used. It is obvious that the minimum requirement for such an implementation is that the gNB which sends RRCRelease should know all the cells in which the UE can move when the dedicated slice priorities are valid. From TS 38.300 V17.2.0, NG-RAN awareness of NSAG information as configured by OAM or exchanged with NG-RAN nodes is supported for (immediate) neighbor cells only.

Awareness in the NG-RAN of the NSAG information supported in the list(s) of neighbour cells may be configured by OAM, or exchanged with neighbour NG-RAN nodes.

In fact, gNB may not be even aware of the cells other than its neighbor cells.So it is not practical for the gNB to know about the slice information of the cells other than its neighbor cells.

Observation 2: A gNB sending RRC Release may not be aware of the cells other than its neighbor cells.

Validity timer for dedicated slice priority is of the order of minutes or even hours, even up to 3 hours. It is obvious that a NR UE can travel quite a distance within this duration and move away from the neighbor cells of the cell that send RRC Release.

Observation 3: Normally, a UE can move to cells which are not the neighbors of the cell that send RRC Release while dedicated slice priority is valid.

From the above two observations, it is clear that it is not possible or practical for the gNB to always provide the dedicated slice information only for frequencies/NSAGs in system information of all the cells where UE could move.

So we propose that RAN2 decides the UE behavior when NSAG-Frequency pair in the dedicated information is not present in the system information, i.e. the behavior when the slice availability list is not available for the NSAG-frequency pair in the dedicated information from the below two options.

Proposal 1: When the NSAG-Frequency pair configured in dedicated slice information is not available in the SIB16, consider the below options

- UE doesn’t use the NSAG-Frequency pair for deriving slice based cell reselection priority in this cell.

- UE derives the slice based cell reselection priority assuming all the cells in the frequency support the NSAG.

It is quite common during initial deployments that a new frequency is added in some location first and then expanding to other areas. Similarly, during initial deployments of slices, for some frequency, slices may be deployed in one area first and later in another area. It may be also possible that some slices are available only in some location, even after the initial deployment. For e.g. a slice available in a city needn’t be present in a nearby town and the UE moving to such a location is quite possible. Hence we think option a) is more suitable. A TP according to the option a) is provided in section 5.

Proposal 2: RAN2 is requested to select option a) and adapt the given TP, when the NSAG-Frequency pair configured in dedicated slice information is not available in the SIB16.

# 3	Conclusion

In the previous sections, we have discussed certain issues with slice specific cell reselection priorities from RRC Release and have made following observations:

Observation 1: A UE provided with slice based cell reselection priorities in RRCRelease uses the slice availability lists in SIB16.

Observation 2: A gNB sending RRC Release may not be aware of the cells other than its neighbor cells.

Observation 3: Normally, a UE can move to cells which are not the neighbors of the cell that send RRC Release while dedicated slice priority is valid.

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: When the NSAG-Frequency pair configured in dedicated slice information is not available in the SIB16, consider the below options

- UE doesn’t use the NSAG-Frequency pair for deriving slice based cell reselection priority in this cell.

- UE derives the slice based cell reselection priority assuming all the cells in the frequency support the NSAG.

Proposal 2: RAN2 is requested to select option a) and adapt the given TP, when the NSAG-Frequency pair configured in dedicated slice information is not available in the SIB16.

# 4	References

- TS 38.304 NR User Equipment (UE) procedures in Idle mode and RRC Inactive state V17.2.0

- R2-2208519 Issues with slice specific cell reselection

- R2-2208733	Report of [AT119-e][241][Slicing] Cell reselection corrections to RAN slicing (Qualcomm)

- RAN2#119#e meeting report

# 5	Annex A: Text Proposal

START OF CHANGE

#### 5.2.4.11	Re-selection priorities for slice-based cell reselection

The UE derives reselection priorities for slice-based cell reselection by using:

-	NSAGs and their priorities provided by NAS,

-	sliceInfoList and or sliceInfoListDedicated per frequency with nsag-CellReselectionPriority per NSAG, if provided in system information and/or dedicated signalling,

-	cellReselectionPriority per frequency provided in system information and/or dedicated signalling.

The UE considers an NR frequency to support all slices of an NSAG if

-	the nsag-ID and TA of the NSAG as provided by NAS are indicated for the NR frequency (see TS 38.331) in the system information of the serving cell (and also in the dedicated slice specific information when it is available).

The UE considers a cell on an NR frequency to support all slices of an NSAG if

-	the nsag-ID and TA of the NSAG as provided by NAS are indicated for the NR frequency (see TS 38.331) in the system information of the serving cell (and also in the dedicated slice specific information when it is available); and

-	the cell is either listed in the sliceAllowedCellListNR (if provided in the used slice-based cell reselection information) or the cell is not listed in the sliceExcludedCellListNR (if provided in the used slice-based cell reselection information); or

-	Neither sliceAllowedCellListNR nor sliceExcludedCellListNR is configured in the used slice-based cell reselection information

The UE shall derive reselection priorities for slice-based cell reselection according to the following rules:

-	Frequencies that support at least one prioritized NSAG received from NAS have higher reselection priority than frequencies that support none of the NSAG(s) received from NAS.

-	Frequencies that support at least one NSAG provided by NAS are prioritised in the order of the NAS-provided priority for the NSAG with highest priority supported on the frequency.

-	Among the frequencies (one or multiple) that support the highest prioritised NSAG(s) with the same NAS-provided priorities, the frequencies are prioritized in the order of their highest nsag-CellReselectionPriority given for these NSAG(s). If no nsag-CellReselectionPriority is given for a NSAG at a frequency, the lowest priority value is used (i.e, lower than any of the network configured values for these frequencies).

-	Frequencies that support none of the NSAG(s) provided by NAS are prioritized in the order of their cellReselectionPriority;

For a UE performing slice-based cell reselection, if the highest ranked cell or best cell in a frequency fulfils the inter- freqeuency cell reselection criteria (see clause 5.2.4.5) based on reselection priority for the frequency and NSAG derived according to this clause or fulfils intra-frequency and equal priority inter-frequency cell reselection criteria (see clause 5.2.4.6), but this cell does not support the NSAG according to this clause,

-	if this cell supports any other NSAG(s) according to this clause, the UE shall re-derive a reselection priority for the frequency by considering the NSAG(s) supported by this cell (rather than those of the corresponding NR frequency);

-	Otherwise, the UE shall re-derive a reselection priority for the frequency as if none of the NSAG(s) provided by NAS is supported.

This re-derived reselection priority is used for a maximum of 300 seconds, or until new information of NSAG(s) and their priorities are received from NAS. UE shall ensure the cell reselection criteria above are fulfilled based on the newly derived priorities.