3GPP TSG-RAN WG1 Meeting #120 	R2-2212478

Toulouse, France, November 14-18, 2022

Agenda item:	    8.19

Source: 		Qualcomm Incorporated

Title: 	Discussion of RAN2 response to LS R2-2211135/S2-2209879 on UL burst sending time adjustment

Document for:	Discussion

# Introduction

In this contribution we discuss the LS by SA2 to RAN2 in R2-2211135/S2-2209879 on UL sending time adjustment and provide views on the proposed response.

# Problem Description

In order to minimize the User Plane buffering delay of low latency applications, AF and NG-RAN attempt to align their understanding of the burst arrival time, correcting for any timing discrepancy in their synchronization in the process. When AF gets the feedback, that may be reactive or proactive as explained below, it can adjust the application to adapt the BAT time to the transmission opportunity. This can be done at both NG-RAN side for DL traffic and UE side for UL traffic.

For low latency communication, SA2 has agreed to support the following two methods to assist the RAN in adapting DL scheduling to low-latency traffic arrival time:

- Reactive feedback: Based on the observed user plane burst arrivals, RAN can adapt DL resource scheduling to ensure low latency of delivery.

- Proactive feedback: Burst Arrival Time is provided from AF to RAN as part of TSCAI in control signalling. RAN can then adapt resource scheduling to ensure low latency between Burst Arrival Time and RAN scheduled DL resources.

SA2 is then considering whether to extend DL reactive mechanisms to UL, and propose whether some feedback from RRC is required as described in clause 8.4 of TR 23.700-25 as there is no BAT in TSCAI to the RAN in Reactive feedback.

# Extending DL Reactive Feedback Case to UL

Before providing views, it is useful to attempt to clarify how the proposed reactive UL scheme works focusing on the aspects of the system affecting NG-RAN, this is adapted for 23.700-25 with the aspects specific to uplink in red:

- AF requests 5GS to report BAT offset, i.e., the timing of the packet reception at the UE User Plane.

- SMF provides the notification control parameter to the NG-RAN as described in TS 23.501, and in addition includes the indication of " burst arrival time adaptation" to the QoS profile.

- Indication of " burst arrival time adaptation" is set in the TSCAI, and the NG-RAN determines that the PDB can no longer be guaranteed for a QoS Flow upon receiving a report from UE via RRC, the NG-RAN notifies the SMF as described in TS 23.501 and in addition may include a BAT offset reported by the UE to the N2 SM information that is sent to SMF, eventually forwarded via PCF/TSCTSF/NEF to AF

  - If the NG-RAN receives the indication for "burst arrival time adaptation", the NG-RAN indicates the parameter to the UE via RRC signalling. The NG-RAN indicates a threshold for the BAT offset reports to the UE.

  - If the UE receives the indication for "burst arrival time adaptation" from NG-RAN, the UE determines a relative BAT offset value in reference to the current Burst Arrival Time experienced by UE (i.e. in reference to when UE currently receives bursts) and the scheduling UL time slot at UE (e.g. in Configured Grants, as defined in TS 38.321 [11]). The UE sends the BAT offset to RAN when the time offset value reaches the configured threshold, and NG-RAN sends the BAT offset value to SMF.

# Feasibility of the Measurement

The feasibility of the measurement for a single UE comes down to whether the UE can measure the time difference between receiving a burst from the application and the UL grant (likely configured grant) scheduled to transmit the burst. We would first like to state a set of assumptions to simplify the analysis:

Assumption 1: The Burst consists of one packet only.

Assumption 2: A CG is used to transmit the burst.

Assumption 3: The low-latency flow being analysed here cannot be deprioritized wrt to the CG allocated to serve it, i.e., the burst never sees excessive latency due to LCH prioritization.

Assumption 4: Similar to assumption 3, the burst cannot be partially transmitted due to PHY or MAC control signal multiplexing.

The four assumptions before are to remove any randomness coming from RAN in measuring the burst. The rest of the analysis herein will proceed based on these assumptions.

From a RAN point of view, there is nothing technical stopping the UE from timestamping the time when the packet arrives at the PDCP entity, and reporting it to gNB via RRC signalling. The UE and gNB are synchronized using the same timing (disregarding small discrepancies due to things like propagation delay). The gNB can then calculate the offset between the CG and the packet arrival time at the UP, which is the required quantity.

Observation 1: UE can measure the absolute timing it receives a burst and indicate that to the gNB which can calculate the required offset under Assumption 1 as a condition.

If the UE was tasked with measuring the offset itself, that can still be done by an internal signal between the MAC and PDCP. While such a measurement is not standardized, it is not hard for the UE to do that as an implementation option. There are some reservations on that cross-layer information exchange, however in principle, the UE can measure an offset between MAC and PDCP, but a new capability needs to be defined in order to ensure that a UE configured with that behaviour can actually perform it.

Observation 2: UE can measure the timing offset between receiving the packet and the grant as an implementation, but a new capability needs to be defined for the behaviour to be configured by the NW.

# Overall Proposal Assessment

Having commented on the narrow question of single UE feasibility, we would like to also give some views on the efficacy of the whole proposal. In the end, the intention is to instruct the AF to change the BAT on the UE UP in-order to reduce delay. It is unclear why the UE has to do that by sending measurements to RAN which has to relay those measurements all the way to the AF, which in turn overlays some instructions back to the application located at the UE. This method requires many hops of signalling and translation steps between UE, RAN, CN and AF to generate an instruction back to the application, located at the UE. Thus, we comment that the proposal by SA2 does the job by introducing intensive control signalling. In the RAN2 scope, this packet-based event RRC control signalling overhead is intensive and not recommended when the task can be done using less signalling.

Observation 3: The overall task can be performed by the UE internally informing the application of the grant timing or current offset.

The amount of control signalling, specifically, RRC signalling needed for this task unnecessarily reduces the capacity of the channel, thus, it is advised that from a RAN2 point of view this proposal is substituted with simple internal UE signalling, as in its current form, the amount of signalling is not justified for the task.

Observation 4: The amount of RRC signalling in the proposal is not justified for the overall task, which can be performed internally at the UE.

Proposal 1: RAN2 to consider the proposed LS response included.

# Proposed LS Response to SA2

RAN2 would like to thank SA2 for the question related to uplink low-latency traffic.

For the questions asked by SA2, RAN2 provides the answers as follows:

Under the following Assumptions:

Assumption 1: The Burst consists of one packet only.

Assumption 2: A CG is used to transmit the burst.

Assumption 3: The low-latency flow being analyzed here cannot be deprioritized with respect to the CG allocated to serve it, i.e., the burst never sees excessive latency due to LCH prioritization.

Assumption 4: Similar to assumption 3, the burst cannot be partially transmitted due to PHY or MAC control signal multiplexing.

It is possible for the UE to measure the absolute timing the packet is received to the PDCP or the offset between the timing of receiving a packet at the PDCP and the CG occasion to transmit that packet.

However, for the overall task, the UE can also simply inform the Application running on the UE with the time of the grant occasion or time between packet reception and the grant occasion internally, thus, RAN2 views the proposed scheme as having unnecessary intensive signalling between RAN, CN, AF and UE, and proposes the task be handled internally within UE implementation, since the UE has all the information needed to perform that internal task.

# Conclusion

Observation 1: UE can measure the absolute timing it receives a burst and indicate that to the gNB which can calculate the required offset under Assumption 1 as a condition.

Observation 2: UE can measure the timing offset between receiving the packet and the grant as an implementation, but a new capability needs to be defined for the behaviour to be configured by the NW.

Observation 3: The overall task can be performed by the UE internally informing the application of the grant timing or current offset.

Observation 4: The amount of RRC signalling in the proposal is not justified for the overall task, which can be performed internally at the UE.

Proposal 1: RAN2 to consider the proposed LS response included.

# Annex LS from SA2 S2-2209879

1. Overall Description:

SA2 has concluded the KI#6 “Adapting downstream scheduling based on RAN feedback for low latency communication” as described in clause 8.4 of TR 23.700-25. Both Proactive feedback and Reactive feedback shall be supported in normative phase.  Reactive feedback is based on the observed user plane burst arrivals and does not need the AF to provide Burst Arrive Time (i.e. there can be no BAT in TSCAI to the RAN) in control signalling. Proactive feedback is based on the BAT provided to RAN in the TSCAI.

There is an editor’s note regarding UL adaptation in Reactive feedback:. ” Editor’s note: the need for UL BAT adaptation and the associated RRC signalling as described above is to be confirmed by RAN2.” The background is that SA2 is considering that the adaptation mechanism used for DL can also be extended for UL in Reactive feedback case. For this, feedback from UE over RRC may be required as described in clause 8.4 of TR 23.700-25 as there is no BAT in TSCAI to the RAN in Reactive feedback.

SA2 would like to ask if RAN2 see feasible to extend the adaptation mechanism also to the UL case based on UE feedback to RAN using RRC signalling.

2. Actions:

To RAN2 group.

ACTION: 	SA2 asks RAN2 group to inform SA2 if RAN2 considers it feasible to extend the reactive adaptation mechanism also to the UL case based on UE feedback to RAN using RRC signalling.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #154		Nov 14 – 18, 2022				Toulouse

TSG-SA2 Meeting Adhoc   	Jan 16 – 20, 2023				Elbonia

# References

[1]  R2-2211135: LS S2-2209879 on UL burst sending time adjustment