3GPP TSG-RAN WG2 Meeting #120	 R2-2212508

Toulouse, FRANCE

14-18 November 2022

Agenda Item:	8.9.2

Source:	Xiaomi

Title:	Further discussion on U2U relay discovery and relay selection

Document for:	Discussion

# Introduction

L2 and L3 U2U relay is to be supported in REL18. In this contribution, we discuss the common parts of L2/L3 U2U relay, i.e. relay discovery, relay selection and relay reselection.

# Discussion

In general the baseline for U2U NR sidelink relaying is based on the scenario and assumptions captured in TR38.836 as agreed in the RAN WID [1]. The following discusses details regarding the discovery message transmission, AS conditions for a Relay UE acting as Relay, Relay UE selection AS criteria, Relay UE reselection and some AS aspects of discovery message content.

## 2.1 U2U Relay Discovery Transmission

From RAN2#119bis-e meeting [3] the following agreement on Discovery message transmission was made.

Agreement:

Proposal 2.3a [20/20]:    Discovery message transmission at the remote UE is conditioned on at least upper layer indication.

As agreed the UE higher layer may initiate a U2U relay UE discovery message transmission at a Remote UE. This higher layer trigger may be influenced by one or many other factors and triggers including triggers from the Access Stratum. In U2N relay, the Remote UE can trigger Relay UE discovery based on Uu channel quality, since the poor Uu channel quality indicates potential RLF. In the U2U relaying scenario, a Remote UE should therefore trigger discovery if an existing source Remote UE to target Remote UE direct PC5 connection (i.e. no relay UE) fails or is about to fail. One straightforward factor to indicate such a potential PC5 connection failure is the PC5 channel quality falling below certain threshold. In this instance the PC5 channel quality can be obtained via the SL-RSRP in case of ongoing sidelink communication, or in the case of no sidelink communication the PC5 channel quality may be determined through the SD-RSRP.

Proposal 1: The Remote UE upper layer may trigger a U2U Relay UE discovery transmission based on AS conditions. These AS triggers may be signalled to the upper layer for the triggering of the discovery message.

Proposal 2: When two remote UEs have an existing direct PC5 link then U2U Relay UE discovery transmission shall be triggered when the Remote UE, determines that the PC5 channel quality is below a threshold. FFS whether the threshold is configurable or pre-configured.

Proposal 3: When two remote UEs have an existing PC5 link then the PC5 channel quality used to determine when to trigger a discovery message transmission may be based on the SL-RSRP in case of ongoing sidelink communication, or in the case of no sidelink communication the PC5 channel quality may be determined by SD-RSRP.

In addition it may not be possible to detect whether the threshold is breached or not and in particular this may include the triggering of RLF, which may be the indication of the existing PC5 link no longer being viable, this also may be an AS trigger indication to trigger transmission of U2U Relay UE discovery transmission, by the Remote UE.

Proposal 4: Detection of RLF of the PC5 direct link between two Remote UEs can also trigger discovery message transmission.

Moreover, unlike the Uu interface, a reference signal may not always available on the sidelink and so a UE may not obtain the measurement result for the sidelink. T400 expiry and the maximum number of consecutive HARQ DTX were introduced to trigger sidelink RLF in this case. Therefore, sidelink RLF may occur before UE can detect PC5 channel quality becomes lower than the threshold. U2U relay UE discovery procedure should be triggered in advance of RLF in the case the sidelink measurement is not available. Otherwise, UE can only trigger U2U relay discovery after sidelink RLF based on sidelink measurements occurs, which may delay the relay selection and extend the service interruption.

Proposal 5: U2U relay discovery should be triggered in advance of sidelink RLF in case the sidelink measurement result is not available, in order to minimise any service interruption. The expiry of T400 can be signalled to the upper layer in order to trigger Discovery message transmission.

In this scenario where the AS trigger for sending a discovery message is based on detection of the PC5 channel quality or RLF for a PC5 channel directly between two Remote UEs falling below a threshold, it is unclear as to whether one or both remote UEs may initiate the Discovery procedure message transmission. The determination as to whether either one of or both of the remote UEs triggers discovery message transmission can be considered a decision of the upper layers of each Remote UE and as such is outside of RAN2’s scope of consideration.

Proposal 6: In the case where detection of the PC5 channel quality for a PC5 channel directly between two Remote UEs falls below a threshold, the determination as to whether one or both of the remote UEs triggers discovery is a decision of the upper layers and as such is outside of RAN2’s scope.

## 2.2 AS criteria to act as U2U relay UE

In Rel-17 U2N relay, a minimum and a maximum threshold are defined for a Relay UE to determine whether it can be a Relay UE or not. When both thresholds are configured, only when the relay UE’s Uu RSRP is within the interval of the minimum and maximum threshold, can the UE act as a candidate relay UE. This is to ensure the relay UE is neither too close to nor too far away from the network to guarantee both the coverage and the communication quality.

When it comes to U2U relaying, according to SA2 U2U relay discovery both Model A and Model B are supported. For Model A, the U2U Relay UE broadcasts an announcement message which may include the user information of Remote UEs that the Relay UE has or can make contact with. For Model B, the U2U Relay UE broadcasts the discovery solicitation message and the discovery response message to the target/source remote UEs respectively.

Similar as U2N relaying, it makes sense for U2U relaying to define some AS criteria to determine whether a UE can serve as a Relay UE or not for both Model A and Model B. Considering the different operating schemes, different principles may need to be introduced for the different models.

For Model A, since the candidate relay UE is to broadcast the announcement message, the SL-RSRP/SD-RSRP between the relay UE and at least a certain number of remote UE should be larger than a minimum threshold to ensure the transmission quality for each hop to those remote UEs. For Model B, since upon reception of the discovery message, relay UE is able to derive the L2 ID information of both the source remote UE and target remote UE, the SL-RSRP/SD-RSRP of each hop should be considered separately, i.e., the SL-RSRP/SD-RSRP between the source/target remote UE and relay UE is above the minimum threshold.

Proposal 7: For Model A, a UE can act as candidate relay UE when the SL-RSRP/SD-RSPR between this UE and a certain number of remote UE is above the minimum threshold.

Proposal 8: For Model B, a UE can act as candidate relay UE when the SL-RSRP/SD-RSPR between this UE and source/target remote UE is above the minimum threshold.

## 2.3 Relay selection

From RAN2#119bis-e meeting [3], the following agreement regarding Relay UE selection was made.

Proposal 7.1a:    Relay selection triggers include at least 1) Upper layer trigger; 2) PC5 signal strength conditions.  RAN2 further discuss details for trigger 2).

The SA2 TR [2] captures a number of scenarios and proposed solutions relating to Relay UE selection both for source Remote UE and target Remote UE, and depending on the discovery model used. For example for model B discovery a source Remote UE cannot be sure of the ability for any single Relay UE to be able to reliably reach a target Remote UE, subsequently the target Remote UE may potentially receive multiple solicitation messages, each from different Relay UEs and each originating from the same source Remote UE.

In addition to these indirect discovery messages received via candidate Relay UEs, the target Remote UE may potentially also receive the discovery message direct from the source Remote UE, e.g. direct, if it is in proximity.

How the target Remote UE responds to these multiple direct and indirectly received messages has been considered in [2] as follows.

6.1.1 … It may exist a situation where multiple UE-to-UE relays can be used to reach the target UE or the target UE may also directly receive the Direct Communication Request or Solicitation message from the source UE. The target UE may choose which one to reply according to e.g. signal strength, local policy (e.g. traffic load of the UE-to-UE relays), Relay Service Code if there is any or operator policies (e.g. always prefer direct communication or only use some specific UE-to-UE relays).

As indicated, in order to support the target Remote UE a number of assistance parameters may assist the Relay UE selection and to which discovery solicitation message or messages to respond to and hence potentially a down selection of Relay UE(s). Some of these assistance parameters relate to Non-Access Stratum information and some relate to Access Stratum information including the following.

- PC5 Signal strength

- Local policy/ preference

- Traffic load of potential U2U Relay UE

- Relay Service Code

- Operator policies (direct over U2U relay)

- Specific Relay UE or Relay UEs for communication

It is for RAN2 to resolve which Access Stratum parameters may be used in the selection process.

In U2N relay, only the U2N relay UE whose PC5 signal strength is above the threshold is considered for relay (re)selection. A similar procedure could be introduced during U2U relay selection. It was agreed at R2#119bis-e that the PC5 signal strength may be an AS selection trigger, but remains FFS as to how it would be specified and utilised.

Proposal 9: Candidate U2U Relay UE is considered to meet the AS criteria for selection if its PC5 link quality measurement is above a configured or pre-configured threshold. FFS regarding how to set the Threshold.

Further, specific Relay UE selection behaviours can be considered separately for each discovery model type.

For Model A, it is the source remote UE to select the relay UE from amongst potential candidate Relay UEs if multiple announcement messages including the user information of the target remote UE are received. The mechanism from U2N can be reused, i.e., the SL-RSRP/SD-RSRP between the source remote UE and the relay UE is above the minimum threshold. If there are multiple relay UE satisfying the condition, it is up to source remote UE’s implementation to select one relay UE.

Proposal 10: For Model A, source Remote UE selects a Relay UE for which the SL-RSRP/SD-RSRP is above the minimum threshold.

For Model B, either the source remote UE or the target remote UE can select the relay UE. If relay UE selection is performed by the target remote UE, the target remote UE may only forward the discovery response to the selected relay UE and the selected relay UE delivers this response to the source remote UE. If relay selection is performed by the source remote UE, which means the target remote UE forwards the discovery response messages to all or a subset of the relay UEs from which the discovery message is received, then upon reception of multiple response messages from the same target remote UE, the source remote UE performs final Relay UE selection. No matter whether the source remote UE or the target remote UE selects the relay UE, the general principle is the same, i.e., the SL-RSRP/SD-RSRP between the source/target remote UE and the relay UE is above the minimum threshold.

Proposal 11: For Model B, target remote UE can select the relay UE for which the SL-RSRP/SD-RSRP is above the minimum threshold and forwards the discovery response message to the selected relay UE.

Proposal 12: For Model B, source remote UE can select the relay UE for which the SL-RSRP/SD-RSRP is above the minimum threshold if multiple discovery response messages from the same target remote UE are received from multiple Relay UEs.

Whether the target Remote UE responds to only the directly received solicitation message (if received), or to one or more discovery messages received via a Relay UE(s) or to both the directly received solicitation message and one or more of the indirect solicitation messages received via the Relay UEs according to [2] is down to target Remote UE including using locally configured rules.

However, when multiple candidate Relay UEs are detected at the target Remote UE it is clear that in cases where the Relay UE could be a viable selection candidate Relay UE e.g. it passes both AS and the upper layer criteria, then the target Remote UE could be expected to send a solicitation response message. This is so that where more than one viable candidate Relay UEs exists then the source Remote UE may actually receive response discovery messages from a choice of Relay UEs. In particular, if there is only one candidate Relay UE sending discovery response messages (when more than one may be viable) then this one candidate may not be optimal for the source Remote UE, but in this case it has no choice.

Proposal 13: A target Remote UE receiving multiple discovery solicitation messages via Relay UEs from the same source Remote UE should respond to all discovery solicitation messages that are from viable candidate Relay UEs, as determined following successful AS and higher layer checks, in order to assist the source Remote UE in the final Relay UE selection.

Proposal 14: A source Remote UE receiving multiple discovery response messages from Relay UEs with a PC5 link quality measurement above the PC5 link quality threshold, may rank these Relay UEs. The Relay UE ranking may be according to their respective PC5 link quality measurement, and forwarded to the Remote UE higher layer for Relay UE selection.

In addition to the PC5 channel quality measurement as an Access Stratum selection criteria the Remote UE may also consider the respective Relay UEs’ load as a factor for Relay UE selection. If multiple U2U Relay UEs fulfil the AS and NAS requirements as viable candidate Relay UEs, a simple solution is to select the U2U Relay UE with the lowest load. If there are multiple lowest load relay UEs, the remote UE may use a ranking of Relay UE candidates based on Relay UE load or additionally a ranking of their respective PC5 channel quality measurement to select the Relay UE or may make a Relay UE selection based on other parameters.

Proposal 15: If multiple U2U Relay UEs fulfil the AS and NAS selection requirement, the Remote UE selects the U2U relay UE with the lowest load.

In addition when a direct link between two Remote UEs exists it may not be possible to detect whether a minimum PC5 link threshold is breached or not and in particular this may result in the triggering of RLF, indicating that the existing PC5 link is no longer viable, this should be considered as an AS trigger for Relay selection by the source Remote UE.

Proposal 16: Detection of RLF of the PC5 direct link between two Remote UEs can trigger relay selection.

2.4 Relay UE Reselection

RAN2#119bis-e made the following agreements in regards to Relay UE reselection triggers.

Proposal 7.1b (modified):       Relay reselection triggers include at least 1) Upper layer trigger; 2) PC5-RLF detection at the remote UE; 3) PC5-RLF indication received from the relay; 4) PC5 signal strength conditions; 5) PC5 link release message from relay to remote.  RAN2 further discuss details for trigger 4), potentially including T400 expiry.  FFS if some of the conditions could be indicated to upper layer instead of directly causing reselection.

Relay UE reselection triggers may be considered as similar to the discovery transmission triggers, wherein discovery may be necessary to identify and locate valid alternative candidate Relay UEs. Specifically it can be observed that triggers for discovery message transmission relating to PC5 channel quality for two Remote UEs in direct communication, as captured in section 2.1 may be adapted in the U2U relaying scenario and applied for Relay UE reselection triggers.

Further, to the above agreement [3], the PC5 signal strength measurement may re-use the existing measurements applied in REL17 for the U2N relaying scenarios. Namely the PC5 channel quality may be based on the SL-RSRP in case of ongoing sidelink communication, or in the case of no sidelink communication by SD-RSRP.

Proposal 17: For the PC5 signal strength measurement in determining whether to trigger Relay UE reselection, the PC5 channel quality may be based on the SL-RSRP in case of ongoing sidelink communication, or in the case of no sidelink communication the PC5 channel quality may be determined by SD-RSRP.

When considering the PC5 signal strength conditions, and unlike the Uu interface, a reference signal may not always be available on the sidelink and so a UE may not obtain the measurement result for the sidelink. T400 expiry and the maximum number of consecutive HARQ DTX were introduced to trigger sidelink RLF in cases where no sidelink measurement was available. Therefore, sidelink RLF may occur before UE can detect PC5 channel quality becomes lower than a threshold. U2U relay reselection should be triggered in advance of RLF in the case the sidelink measurement is not available. Otherwise, UE can only trigger U2U relay reselection after sidelink RLF occurs, which may delay the relay selection and extend the service interruption.

Proposal 18: for a Remote UE U2U relay reselection should be triggered in advance of sidelink RLF in case the sidelink measurement result is not available. The expiry of T400 can be signalled to the upper layer in order to trigger Relay UE reselection.

Proposal 19: A Relay UE may send an indication to a Remote UE on detection of T400 expiry on a PC5 link in order to enable the Remote UE to trigger Relay UE reselection.

In addition to the above triggers the Remote UE may also consider indication by the Relay UE of the Relay UE’s load as a trigger for Relay UE reselection. The Relay UE can trigger the indication on the Relay load reaching a threshold, where the actual Relay UE loading may be causing delay to one or more of the traffic channels currently supported by the Relay UE. The resulting action by the Remote UE may be to trigger a reconfiguration or to trigger a Relay UE reselection. An indication including an indication of the load enables the Remote UE to make an informed choice based on its understanding of the actual traffic conditions.

Proposal 20: Relay UE triggers an indication to the Remote UE when it’s loading reaches a threshold. The Indication includes information relating to the relative loading of the Relay UE. FFS how to measure the actual load and the corresponding load indication.

Alternatively and additionally the Relay UE may explicitly indicate e.g. via a PC5-S link release message, that the Remote UE should trigger a Relay UE reselection. The resulting behaviour for the Remote UE may vary depending on the explicit indication cause sent by the Relay UE. In one example this may be a notably different behaviour to a RLF indication (for the second hop) wherein the ongoing service may be maintained up to the point where the Remote UE successfully reselects an alternative Relay UE.

Proposal 21: The Relay UE sends an explicit indication to trigger the Relay UE reselection by the Remote UE. In one scenario the indication may enable the Remote UE to continue with the existing sidelink service until the new Relay UE reselection is completed.

## 2.5 Discovery Message

Regarding the content carried in U2U relay discovery message particularly to aid Relay UE selection where multiple Relay UEs are candidates to establish a U2U relay link, SA2 TR [2], proposed to consider traffic load as well as a number of other factors of the relay UE during relay UE selection as following.

6.1.1 … When a UE-to-UE relay receives a Direct Communication Request or a Solicitation message with the relay_indication set, then it shall decide whether to forward the message (i.e. modify the message and broadcast it in its proximity), according to e.g. Relay Service Code if there is any, Application ID, authorization policy (e.g. relay for specific ProSe Service), the current traffic load of the relay, the radio conditions between the source UE and the relay UE, etc. It may exist a situation where multiple UE-to-UE relays can be used to reach the target UE or the target UE may also directly receive the Direct Communication Request or Solicitation message from the source UE. The target UE may choose which one to reply according to e.g. signal strength, local policy (e.g. traffic load of the UE-to-UE relays), Relay Service Code if there is any or operator policies (e.g. always prefer direct communication or only use some specific UE-to-UE relays).

Therefore, traffic load should be included in or with the discovery message. RAN2 can further discuss how to define the load of the relay UE. One straight forward way to define the load is based on relay UE’s CBR result.

Proposal 22: Load information of U2U relay UE can be included in or with the discovery message for example in the AS information transmitted with the U2U relay discovery message, FFS on how to determine the load information.

# Conclusion

Based on the discussion in section 2, we have following observation and proposals

Discovery Transmission

Proposal 1: The Remote UE upper layer may trigger a U2U Relay UE discovery transmission based on AS conditions. These AS triggers may be signalled to the upper layer for the triggering of the discovery message.

Proposal 2: When two remote UEs have an existing direct PC5 link then U2U Relay UE discovery transmission shall be triggered when the Remote UE, determines that the PC5 channel quality is below a threshold. FFS whether the threshold is configurable or pre-configured.

Proposal 3: When two remote UEs have an existing PC5 link then the PC5 channel quality used to determine when to trigger a discovery message transmission may be based on the SL-RSRP in case of ongoing sidelink communication, or in the case of no sidelink communication the PC5 channel quality may be determined by SD-RSRP.

Proposal 4: Detection of RLF of the PC5 direct link between two Remote UEs can also trigger discovery message transmission.

Proposal 5: U2U relay discovery should be triggered in advance of sidelink RLF in case the sidelink measurement result is not available, in order to minimise any service interruption. The expiry of T400 can be signalled to the upper layer in order to trigger Discovery message transmission.

Proposal 6: In the case where detection of the PC5 channel quality for a PC5 channel directly between two Remote UEs falls below a threshold, the determination as to whether one or both of the remote UEs triggers discovery is a decision of the upper layers and as such is outside of RAN2’s scope.

AS Criteria to Act As a Relay UE

Proposal 7: For Model A, a UE can act as candidate relay UE when the SL-RSRP/SD-RSPR between this UE and a certain number of remote UE is above the minimum threshold.

Proposal 8: For Model B, a UE can act as candidate relay UE when the SL-RSRP/SD-RSPR between this UE and source/target remote UE is above the minimum threshold.

Relay Selection

Proposal 9: Candidate U2U Relay UE is considered to meet the AS criteria for selection if its PC5 link quality measurement is above a configured or pre-configured threshold. FFS regarding how to set the Threshold.

Proposal 10: For Model A, source Remote UE selects a Relay UE for which the SL-RSRP/SD-RSRP is above the minimum threshold.

Proposal 11: For Model B, target remote UE can select the relay UE for which the SL-RSRP/SD-RSRP is above the minimum threshold and forwards the discovery response message to the selected relay UE.

Proposal 12: For Model B, source remote UE can select the relay UE for which the SL-RSRP/SD-RSRP is above the minimum threshold if multiple discovery response messages from the same target remote UE are received from multiple Relay UEs.

Proposal 13: A target Remote UE receiving multiple discovery solicitation messages via Relay UEs from the same source Remote UE should respond to all discovery solicitation messages that are from viable candidate Relay UEs, as determined following successful AS and higher layer checks, in order to assist the source Remote UE in the final Relay UE selection.

Proposal 14: A source Remote UE receiving multiple discovery response messages from Relay UEs with a PC5 link quality measurement above the PC5 link quality threshold, may rank these Relay UEs. The Relay UE ranking may be according to their respective PC5 link quality measurement, and forwarded to the Remote UE higher layer for Relay UE selection.

Proposal 15: If multiple U2U Relay UEs fulfil the AS and NAS selection requirement, the Remote UE selects the U2U relay UE with the lowest load.

Proposal 16: Detection of RLF of the PC5 direct link between two Remote UEs can trigger relay selection.

Relay Reselection

Proposal 17: For the PC5 signal strength measurement in determining whether to trigger Relay UE reselection, the PC5 channel quality may be based on the SL-RSRP in case of ongoing sidelink communication, or in the case of no sidelink communication the PC5 channel quality may be determined by SD-RSRP.

Proposal 18: for a Remote UE U2U relay reselection should be triggered in advance of sidelink RLF in case the sidelink measurement result is not available. The expiry of T400 can be signalled to the upper layer in order to trigger Relay UE reselection.

Proposal 19: A Relay UE may send an indication to a Remote UE on detection of T400 expiry on a PC5 link in order to enable the Remote UE to trigger Relay UE reselection.

Proposal 20: Relay UE triggers an indication to the Remote UE when it’s loading reaches a threshold. The Indication includes information relating to the relative loading of the Relay UE. FFS how to measure the actual load and the corresponding load indication.

Proposal 21: The Relay UE sends an explicit indication to trigger the Relay UE reselection by the Remote UE. In one scenario the indication may enable the Remote UE to continue with the existing sidelink service until the new Relay UE reselection is completed.

Discovery Message

Proposal 22: Load information of U2U relay UE can be included in or with the discovery message for example in the AS information transmitted with the U2U relay discovery message, FFS on how to determine the load information.

# Reference

[1] 	RP-221947 SR for NR_SL_relay_enh, LG Electronics (Rapporteur)

[2]	TR 23.700-33 V1.1.0 (2022-10)  Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2 (Release 18)

[3]	RAN2-119bis-e-Positioning-Relay-2022-10-19-2000_EOM, Session chair draft minutes

[4]	R2-2207126	Discovery and Relay (re-)selection for UE-to-UE relay, Qualcomm