3GPP TSG-RAN WG2 Meeting #120	R2-2212546

Toulouse, France, Nov 14 – Nov 18, 2022

Agenda Item		:	8.4.2.3

Source		:	LG Electronics Inc.

Title		:	Discussion on dynamic cell switch

Document for		:	Discussion and Decision

1.	Introduction

In Rel-18, L1L2 triggered mobility (LTM) is one of the important features for further NR mobility enhancements, which is introduced in order to reduce latency, overhead and interruption time [1]. In this paper, for LTM, we discuss configuration, execution, and failure handling.

2.	Discussion

2.1	Candidate cell configuration and management

In RAN3#117bis, the following FFS is noted:

FFS on whether the gNB-DU can suggest candidate cells after the gNB-CU initiates the L1/L2 inter-cell mobility configuration. Three Options are left for further discussion: Opt 1. DU cannot suggest any candidate cells. Opt 2. DU suggest candidate cells within the list provided by CU. Opt 3. DU suggest candidate cells outside the list provided by CU.

To enable the option 3 in the above FFS, UE should be able to perform L1 measurement to a cell that is neither a serving cell or a candidate cell and report the L1 measurement results to gNB-DU. We believe in Rel-18 the L1 measurement framework is not enhanced to perform L1 measurements on cells that are neither serving nor candidate cells. In our understanding, the network uses L3 measurement reports (i.e. MeasurementReport messages) when determining which candidate cell will be configured for LTM because configuring a candidate cell is not as urgent as executing LTM. We think the option 3 is on the table because RAN3 may not know what kind of measurement will be used for the network to determine candidate cells to be configured for LTM. To avoid this confusion, RAN2 needs to clarify this issue and inform RAN3 about it.

Proposal 1. The network determines candidate cells to be configured for LTM based on L3 measurement reports.

Proposal 2. If Proposal 1 is agreed, RAN2 informs about it to RAN3.

2.2	LTM execution aspects

RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering. FFS if the MAC CE can indicate TCI state(s) (or other beam info) to activate for the target Cell(s), dep on RAN1 progress.

From the above agreements made in RAN2#119bis, the LTM execution could be performed by one-step or two-step procedure.

In the one-step procedure, upon receiving the cell switch command via MAC CE, UE executes the LTM by transmitting an UL signal to the target cell indicated by the cell switch command. An example procedure of one-step LTM execution is decribed in Figure 1.

Figure 1. An example procedure of one-step LTM execution.

In the two-step procedure, upon receiving the cell switch command via MAC CE, UE starts PDCCH monitoring to the target cell indicated by the cell switch command. The source cell indicates the LTM execution to the target cell if an acknowledgement corresponding to the cell switch command is received. Then, the target cell transmits a DCI to UE. UE exchanges user data with the source cell until a DCI is received from the target cell. Upon receiving a DCI from the target cell, UE switches to the target cell. An example procedure of two-step LTM execution is decribed in Figure 2.

Figure 2. An example procedure of two-step LTM execution.

Observation 1. The LTM execution could be performed by one-step or two-step procedure:

- One-step procedure: Upon receiving the cell switch command via MAC CE, UE executes the LTM by transmitting an UL signal to the target cell indicated by the cell switch command.

- Two-step procedure: Upon receiving the cell switch command via MAC CE, UE starts PDCCH monitoring to the target cell indicated by the cell switch command. The source cell indicates the LTM execution to the target cell if an acknowledgement corresponding to the cell switch command is received. Then, the target cell transmits a DCI to UE. UE exchanges user data with the source cell until a DCI is received from the target cell. Upon receiving a DCI from the target cell, UE switches to the target cell.

In order to execute the LTM based on the two-step procedure, UE may needs to set up dual protocol stack to the source and the target cells, which is similar to Rel-16 DAPS based mobility. We think the two-step LTM execution would lead to high UE complexity and requirement for UE to capable of dual L1/L2 protocols. Further, the two-step LTM execution may require Rel-17 ICBM as prerequisite, which conflicts with RAN2 agreement that Rel-17 is not a prerequisite. On the other hand, the two-step LTM execution is simpler and may be applicable to all Rel-18 LTM scenarios. Therefore, RAN2 consider the one-step LTM execution as a baseline.

Proposal 3. The one-step LTM execution is considered as a baseline. Upon receiving the cell switch command, UE executes the LTM by transmitting an UL signal to the target cell.

In the previous meeting RAN2#119bis, RAN2 made the following agreement:

RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this.

From the above agreement, UE performs RACH-less LTM if TA of the candidate cell and beam information applicable for the candidate cell upon receiving the cell switch command, UE performs RACH-based LTM otherwise.In the case of RACH-based LTM, the target cell can know that UE execute an LTM by receiving RA preamble. On the other hand, in the case of RACH-less LTM, UE needs to transmit an UL signal to the target cell in order to indicate LTM execution to the target cell.

For example, the UE can transmit the Scheduling Request (SR) to the target cell immediately, where SR indicates completion of LTM while requesting resources for UL transmission. Furthermore, if the network can provide the UL grant applicable to the target cell via the cell switch command, the UE could perform the PUSCH transmission to the target cell even without transmitting SR, where the PUSCH indicates LTM completion while including UL data. This implies that the cell switch command including UL grant may reduce the latency due to UL transmission by skipping SR transmission. Since the latency reduction is important for DL as well as for UL in dynamic switching scenario, RAN2 needs to discuss how to reduce the latency due to the UL transmission for LTM.

Proposal 4. RAN2 considers the following signaling options for indicating the mobility completion:

- A. UE transmits Scheduling Request to the target cell.

- B. UE transmits PUSCH to the target cell if UL grant applicable to the target cell is indicated by the cell switch command.

- C. UE transmits a new UL signal to the target cell. FFS new UL signal.

2.3	LTM failure handling

RAN2 needs to further discuss how UE detects a failure of LTM. It is straightforward that a new timer (e.g., T304-like) is used to detect the failure as in legacy handover failure detection. That is, UE starts a timer (e.g. T304-like) to detect LTM failure when transmitting an UL signal to the target cell. The timer stops upon receiving the acknowledgement corresponding to the UL signal or receiving the first DL signal from the target cell. UE detects LTM failure at timer expiry.

Proposal 5. RAN2 to discuss how UE detects a failure of LTM, e.g. timer-based failure detection.

If UE initites the RRC re-establishment procedure due to LTM failure, it causes not only a large interruption but also unnecessary reconfiguration after the re-establishment. UE may utilize configured candidate cells for faster recovery. For example, upon failure of LTM, UE may attempt to access one of other candidate cells. In this case, if UE arbitrarily selects a target cell among the configured candidate cells, the UE may select a target cell that is not preferred by the network for recovery. For instance, some candidate cells may not be fully prepared for fast recovery while other candidates may have been fully prepared for fast recovery. If UE attempts to access a candidate cell that is not fully prepared for fast recovery, the recovery may be delayed or even fail.

Observation 2. UE based LTM failure recovery, i.e., UE attempts to access one of other candidate cells upon detecting LTM failure, may lead to select a candidate cell that is not fully prepared for fast recovery. Consequently, the recovery may be delayed or even failed by UE based LTM failure recovery.

The network indication based LTM failure recovery can be considered as an alternative to UE based LTM failure recovery. There are two approaches of the network indication based LTM failure recovery.

Approach 1: Upon detecting LTM failure, UE indicates the failure to the source cell. If the source cell successfully receives the LTM failure indication from UE, then the network sends a new cell switch command indicating one of other candidate cells.

Figure 3. An example of Approach 1.

Approach 1 is simple and straightforward. The latency due to recovery could be reduced by re-attempting LTM to a candidate cell indicated by a new cell switch command instead of performing the RRC re-establishment procedure. Since in general the LTM is triggered when the quality of the candidate link improves as that of the source link deteriorates, it is reasonable to assume that the source link quality is not good after receiving the cell switch command. So, it is likely that the UE would fail to indicate the LTM failure to the network due to poor source link quality.

Approach 2: The network indicates one or more secondary candidate cells to UE before or by cell switch command. Upon detecting LTM failure, UE attempts to access a secondary candidate cell.

Figure 4. An example of Approach 2.

Approach 2 is faster than Approach 1 because UE immediately attempts to access a secondary candidate cell without signaling between UE and the network in Approach 2. For Approach 2, RAN2 needs to further study the follwings, e.g., how the network indicates secondary candidate cells. For example, secondary candidate cells for LTM failure recovery could be included in an RRC Reconfiguration for LTM preparation or in a cell switch command.

Proposal 6. RAN2 considers the following options for network controlled LTM failure recovery:

- Approach 1. Upon detecting LTM failure, UE indicates the failure to the source cell. If the source cell successfully receives the LTM failure indication from UE, then the network sends a new cell switch command indicating one of other candidate cells.

- Approach 2. The network indicates one or more secondary candidate cells to UE before or by cell switch command. Upon detecting LTM failure, UE attempts to access a secondary candidate cell.

3.	Conclusion

In this paper, we have made the following proposals:

Proposal 1. The network determines candidate cells to be configured for LTM based on L3 measurement reports.

Proposal 2. If Proposal 1 is agreed, RAN2 informs about it to RAN3.

Observation 1. The LTM execution could be performed by one-step or two-step procedure:

- One-step procedure: Upon receiving the cell switch command via MAC CE, UE executes the LTM by transmitting an UL signal to the target cell indicated by the cell switch command.

- Two-step procedure: Upon receiving the cell switch command via MAC CE, UE starts PDCCH monitoring to the target cell indicated by the cell switch command. The source cell indicates the LTM execution to the target cell if an acknowledgement corresponding to the cell switch command is received. Then, the target cell transmits a DCI to UE. UE exchanges user data with the source cell until a DCI is received from the target cell. Upon receiving a DCI from the target cell, UE switches to the target cell.

Proposal 3. The one-step LTM execution is considered as a baseline. Upon receiving the cell switch command, UE executes the LTM by transmitting an UL signal to the target cell.

Proposal 4. RAN2 considers the following signaling options for indicating the mobility completion:

- A. UE transmits Scheduling Request to the target cell.

- B. UE transmits PUSCH to the target cell if UL grant applicable to the target cell is indicated by the cell switch command.

- C. UE transmits a new UL signal to the target cell. FFS new UL signal.

Proposal 5. RAN2 to discuss how UE detects a failure of LTM, e.g. timer-based failure detection.

Observation 2. UE based LTM failure recovery, i.e., UE attempts to access one of other candidate cells upon detecting LTM failure, may lead to select a candidate cell that is not fully prepared for fast recovery. Consequently, the recovery may be delayed or even failed by UE based LTM failure recovery.

Proposal 6. RAN2 considers the following options for network controlled LTM failure recovery:

- Approach 1. Upon detecting LTM failure, UE indicates the failure to the source cell. If the source cell successfully receives the LTM failure indication from UE, then the network sends a new cell switch command indicating one of other candidate cells.

- Approach 2. The network indicates one or more secondary candidate cells to UE before or by cell switch command. Upon detecting LTM failure, UE attempts to access a secondary candidate cell.

4.	References

[1] RP-222332 Revised WID on Further NR mobility enhancements, 3GPP TSG RAN Meeting #97-e, 12 – 16 September, 2022