3GPP TSG RAN WG2 #120	R2-2212554

Toulouse, France, November 14th – 18th, 2022

Source:	Sharp

Title:	Signaling procedures to enable sidelink positioning

Agenda Item:	8.2.2

Document for:	Discussion and Decision

Introduction

In RAN2#119bis-e, followings were agreed [1].

Agreements: Proposal 1 (modified): RAN2 agrees to support unicast SLPP/RSPP session-based operation and to study the applicability of groupcast/broadcast to SLPP/RSPP group operation. FFS if groupcast/broadcast operation, if supported, would be session-based or sessionless. Proposal 3 (modified): RAN2 agrees to support at least unicast SLPP/RSPP “centralized” operation in the sense used in R2-2210911, i.e., operation where one UE performs range and/or position calculations based on measurement/location information relating to itself and/or other UEs. RAN2 will follow SA2 on which UE(s) can perform the calculation and related RAN1 definitions.

In this contribution, we share the view on SL positioning.

Discussions

Deference between positioning and ranging

In our understanding, for positioning, the multiple measurement results are needed for the target position calculation. There are two possibilities. One is that a certain node collects all the other nodes’ measurement results so that such node can calculate all the target UEs’ locations based on the collected measurement results, then it would send the calculated location to each UE that needs to know its own location. The other is that each UE that needs to know its own location collects necessary measurement results from other nodes. Therefore, the measurement results should be reported centralized or distributed for one or more nodes.

On the other hand, for ranging, the purpose is to obtain the distance or direction to the target, but since measurement of TOA or AOA are almost equivalent to the calculation of distance or direction that is the result of Ranging. Therefore, it does not make sense to transfer measurement results from a UE that performs measurement to another UE that performs the calculation of ranging result (i.e., distance or direction).

Therefore, for the procedure design, the positioning and ranging should be discussed separately.

Proposal 1

- For the signaling procedure design, the positioning and ranging should be considered separately.

Session-less operation

Figure 1 Positioning for V2X scenario with multiple RSUs

For positioning in the V2X use-case with RSU, one possible scenario is shown in Figure 1. On roads which a lot of cars pass through, RSUs may be deployed with a certain spacing between one another. Each car calculates its location by using signal and data from multiple RSUs. For example, the multiple RSUs may transfer the assistance information (e.g. SL-PRS configuration, RSU’s location information) to a vehicle UE. In this case, the vehicle UEs may not need to exchange signal or data with other vehicle UEs. In this use case with one-way signaling from a UE-type RSU to a vehicle UE, session-based operation may cause a signaling overhead, while session-less operation may be more beneficial to reduce the overhead. Therefore, the session-less operation should be supported for positioning.

Proposal 2

- For positioning, the session-less operation should be supported.

Figure 2 Ranging for V2X scenario

On the other hand, for ranging between 2 UEs, information just has to be exchanged between one specific UE and only one of the other UE for one ranging calculation. For example, when UE1 perform ranging to obtain target UE(UE2)’s distance or direction, UE1 may transfer request for SL ranging related signal/information, and UE1 may perform ranging calculation based on the SL ranging related signal/information provided by UE2. In this case, the session-less operation may not reduce the signaling overhead. Rather, the session-based operation is suitable, because the UE1 may need to be aware of whether UE2 has the ability of SL ranging or not anyway before the requesting.

Proposal 3

- For ranging, the session-less operation is not necessary.

Centralized/Distributed operation

For positioning, the method such as SL-TDOA or SL-Multi-RTT may need to measure the SL-PRS from multiple anchor UEs to obtain the location of the target UE. In other words, the signaling procedure needs to cover the cases where the UE has to exchange signal/data with more than one UEs. Therefore, it is meaningful to discuss whether support the centralized operation or distributed operation.

On the other hand, for the method such as SL-Single-RTT or SL-AoA which is used for ranging may be done with measuring of the SL-PRS from single anchor UEs to obtain the distance or direction. In this case, considering the signaling between two nodes should be sufficient. Therefore, the centralized operation or distributed operation does not need to be considered for ranging.

Proposal 4

- The discussion of the centralized/distributed operation is limited for positioning, and no need to consider for ranging.

Conclusion

In this contribution, we have the following proposals:

Proposal 1

- For the signaling procedure design, the positioning and ranging should be considered separately.

Proposal 2

- For positioning, the session-less operation should be supported.

Proposal 3

- For ranging, the session-less operation is not necessary.

Proposal 4

- The discussion of the centralized/distributed operation is limited for positioning, and no need to consider for ranging.

References

”RAN2 Chairman’s Notes,” RAN2 #119bis-e, October 2022.