3GPP TSG-RAN WG2 Meeting #120	R2-2212643

Toulouse, France, 14th – 18th November, 2022

Source: 	vivo

Title:  	Further discussion on SON for NPN

Agenda Item:	8.13.7

Document for:	Discussion and Decision

- Introduction

In RAN2#119bis-e meeting [1], the following agreements regarding NPN were achieved:

Agreements:

1	SNPN ID (e.g.,NID ID) checking is needed before sending the availability indication for corresponding SON and MDT report. The details can be discussed case by case. FFS PNI-NPN ID checking.

2	Include the NPN ID into SON/MDT report, whether SNPN ID or PNI-NPN ID related info should be included can be discussed per use case.

3	RAN2 prioritizes the use cases of RLF report and logged MDT enhancement for NPN.

In this contribution, we will provide further analyses and proposals on the support of SON/MDT enhancement for NPN regarding the following respects:

- Network ID checking;

- Logged MDT configuration for NPN;

- Additional information on RLF report for NPN;

- Resource wastes due to non-real-time retrieval.

- Discussion

  - Network ID checking

In the last meeting, the preliminary agreements about network checking are as follows:

1 SNPN ID (e.g.,NID ID) checking is needed before sending the availability indication for corresponding SON and MDT report. The details can be discussed case by case. FFS PNI-NPN ID checking.

For discussion on the details of SNPN ID checking, the corresponding procedure of SNPN ID checking before sending RLF report is discussed first in this section. Considering mobility is only supported in the same SNPN, to prevent reports recorded in an SNPN from being reported to another SNPN or PLMN, in the first step, UE shall store registered SNPN (i.e, PLMN ID and NID) when determining the content of the RLF report in VarRLF-Report; before including the available indication of RLF report in the RRC complete message (e.g., RRCConfigurationComplete, RRCReestablishmentComplete), UE shall perform SNPN checking, i.e., if the registered SNPN is same as stored SNPN in VarRLF-Report and if the UE has radio link failure or handover failure information available in VarRLF-Report, UE shall include the available indication of RLF report in the RRC complete message. Since PLMN ID checking procedure before sending the availability indication corresponding report of use cases of RLF report and logged MDT is the same, the above details are also applicable in logged MDT.

Proposal 1: The following steps can be adopted for SNPN ID checking before sending the availability indication for RLF report:

- UE shall store registered SNPN (i.e, PLMN ID and NID) in VarRLF-Report when determining the content of RLF report;

- If the registered SNPN is the same as the stored SNPN in VarRLF-Report and if the UE has radio link failure or handover failure information available in VarRLF-Report, UE shall include the available indication of RLF report in the RRC Complete message.

Referring to PLMN ID checking in the Spec, besides PLMN ID checking being needed before sending the availability indication, it is also needed before transmitting the information. So we think SNPN ID checking is needed before sending the information for the corresponding SON and MDT report.

Proposal 2: SNPN ID checking is needed before sending the information for the corresponding SON and MDT reports.

For PNI-NPN, since mobility between the public network and CAG Member Cells can be supported if the cell is part of either the selected PLMN or the registered PLMN or PLMN of the Equivalent PLMN list, no enhancement is needed for PLMN checking for PNI-NPN.

Proposal 3: PNI-NPN ID checking is not needed before sending the availability indication and the information for the corresponding SON and MDT report.

  - Logged MDT configuration for NPN

For logged MDT configuration, we found that related enhancement was discussed in RAN3 as well and the following agreement was achieved by RAN3 [2]:

Area scope: Introduce a CAG list for MDT area scope.

Furthermore, RAN3 also discussed whether to introduce SNPN ID for MDT area scope configuration during RAN3#117bis-e meeting in [3], we think it is unnecessary to repeat the discussion of MDT area scope configuration in RAN2 and RAN2 can wait for the agreements of the discussion of RAN3.

Proposal 4: RAN2 to wait for RAN3 progress on MDT area scope configuration.

  - Additional information on RLF report for NPN

In the last meeting, several companies proposed that NPN ID should be included in the RLF report, but the relevant agreements were not reached. Only CGI is used to indicate cell ID related information based on the current Spec. Based on CGI, the network does not know whether the information reported by UE is related to the public network or the private network. For network sharing scenarios, the resources configured for the public and private networks may be independent per the network perspective, and the network needs to know whether this radio link failure is caused by the public or private network to optimize the corresponding configuration. Therefore, the NPN information (e.g., NID or CAG list) of the cell should be included in the existing RLF report.

Proposal 5: The NPN information of the cell, e.g., NID or CAG list, should be included in the RLF report.

  - Resource waste due to non-real-time retrieval

In RAN2#119bis-e meeting, the issue of the UE storage resource waste due to non-real-time retrieval of SNPN-related reports was proposed by some companies. Taking the RLF report as an example, the scenario is described as follows, assuming that a UE has RLF reports for this SNPN network, once the UE is connected to other networks before sending these reports to the specific SNPN network, the stored RLF report cannot be transferred to the original SNPN network, resulting in UE storage resource wastes. In our understanding, this issue exists in the public network as well since UE can only store one set of RLF information at a time and the new information will overwrite the previous one, if UE moves to another network that is not equivalent to the network where the RLF occurred, the UE cannot report this report either. Furthermore, we think this issue can be solved by the network, i.e., the network can request the reports once receiving the available indication.

Proposal 6: It is up to the network implementation to solve the issue of resource waste due to the non-real-time retrieval of SNPN-related reports.

- Conclusions

Based on the analyses given above, we have the following proposals:

Proposal 1: The following steps can be adopted for SNPN ID checking before sending the availability indication for RLF report:

- UE shall store registered SNPN (i.e, PLMN ID and NID) in VarRLF-Report when determining the content of RLF report;

- If the registered SNPN is the same as the stored SNPN in VarRLF-Report and if the UE has radio link failure or handover failure information available in VarRLF-Report, UE shall include the available indication of RLF report in the RRC Complete message.

Proposal 2: SNPN ID checking is needed before sending the information for the corresponding SON and MDT reports.

Proposal 3: PNI-NPN ID checking is not needed before sending the availability indication and the information for the corresponding SON and MDT report.

Proposal 4: RAN2 to wait for RAN3 progress on MDT area scope configuration.

Proposal 5: The NPN information of the cell, e.g., NID or CAG list, should be included in the RLF report.

Proposal 6: It is up to the network implementation to solve the issue of resource waste due to the non-real-time retrieval of SNPN-related reports.

- Reference

- Chairman_notes of 3GPP TSG RAN WG2 meeting #119bis-e, Online

- Chairman_notes of 3GPP TSG RAN WG3 meeting #117bis-e, Online

- R3-225910 Summary of Offline Discussion on CB: #SONMDT4_NPN