3GPP TSG-RAN WG2 Meeting #120	 										    R2-2212655

Toulouse, France, 14th - 18th November 2022

Source: 	Xiaomi

Title:  	Discussion on NR-DC with selective activation of the cell groups

Agenda Item:	8.4.3 NR-DC with selective activation cell of groups

Document for:	Discussion and Decision

Introduction

A New WID on Further NR mobility enhancements [1] has been approved. In the WID, one of the objectives is to specify mechanism and procedures of NR-DC with selective activation of the cell groups, shown as below:

2. To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements: - To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4] Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

In previous meetings, the following agreements have been reached for NR-DC with selective activation of the cell groups [2-3].

Agreement_RAN2#119-e:

The selective activation of cell groups should correspond to support of subsequent conditional changes (CPC) after a cell group change (normal or conditional). CPA FFS.

Initial focus on SCG

There is interest to support delta configuration, to reduce the signalling overhead (FFS if some other objective should be achieved)

FFS how many subsequent conditional changes are targeted (and what is the impact of such assumption).

FFS whether there is a security issue: e.g. to determine vertical or horizontal key derivation, e.g. security parameters re-used as part of subsequent CG switch (for the case when UE goes back to a previous cell, maybe in another SN), and FFS on the procedure/method with which the UE derives the SN security, e.g. based on a prior MN config (without RRC CPC config at the time of SN switch).

Agreement_RAN2#119bis-e:

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs):

- a.	Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell.

- b.	Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells.

- c.	Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell.

Confirm that “CPA” selective activation of cell groups will be supported for this WI objective

Confirm that we aim to support delta configuration, i.e. that there need to be a known reference.

RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

In this contribution, we provide our understandings for the remaining issues of selective activation of the cell groups.

Discussion

Scenarios

Considering different initiating node (MN or SN), R17 CPC can be divided into MN initiated and SN initiated CPC. And the initiating node (MN or SN) indicates a list of candidate target PSCells and associated execution conditions. Both MN initiated and SN initiated CPC can be discussed for R18 SCG selective activation:

Proposal 1: RAN2 can consider the following scenarios of SCG selective activation:

- Scenario 1: MN initiated SCG selective activation

- Scenario 2: SN initiated SCG selective activation

Procedures

In previous meeting, RAN2 agreed the baseline procedure to support selective activation of SCG. There are some remaining issues for the procedure as follows:

Issue 1: FFS if UE keeps all configurations or if those are indicated by the network,

Issue 2: FFS support of nested configs

For Issue 1, it has been agreed that “after finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC”. Hence, keeping other candidate target configuration has been agreed, and we only need to discuss whether UE needs to keep current serving cell configuration and associated conditions after SCG addition or change.

Proposal 2: RAN2 needs to discuss whether UE needs to keep current serving cell configuration and associated conditions after SCG addition or change.

To support subsequent secondary cell group change, UE keeps CPC/CPA configurations based on the indication provided by network. Without the indication, UE cannot distinguish between legacy CPC/CPA configuration and R18 CPC/CPA configuration for subsequent SCG change/addition. A common indication can be used for all candidate configuration if legacy CPC/CPA and R18 selective activation cannot be configured simultaneously. A separate indication for each candidate configuration also can be considered, which can be used to support configuring legacy CPC/CPA and R18 selective activation simultaneously.

Proposal 3: To support subsequent secondary cell group change, UE keeps CPC/CPA configurations based on the indication provided by network.

For Issue 2, keeping candidate configuration for R18 selective activation is enough and the support of nested configurations, i.e. CPC/CPA configuration including CPC configurations is not needed. If it is supported, the additional work of the WI is needed and it is needed to discuss how to transmit the candidate configuration for applied nested configurations.

Proposal 4: RAN2 doesn’t support nested configurations for SCG selective activation.

Because all candidate SCGs are associated with the serving MCG and MCG directly or indirectly transmitted CPC/CPA configurations to UE, which means CPC/CPA configurations rely on the serving MCG, UE should release CPC/CPA configuration after MCG change even though selective activation of the cell groups has been configured.

Proposal 5: In SCG selective activation, UE should release CPC/CPA configurations after MCG change.

  - CPA

In last meeting, RAN2 confirmed that “CPA” selective activation of cell groups will be supported for this WI objective. However, how to achieve “CPA” selective activation of cell groups is unclear. RAN2 need to discuss the issue and the following options can be considered:

Option 1: using same conditional configuration for CPA and CPC.

Option 2: using different configurations for CPA and CPC.

Option 3: using two set of conditions for CPA and CPC, i.e. one for CPA and one for CPC

In legacy CPA and CPC, MN initiated inter-SN CPC and CPA can be configured with event A4 and SN initiated inter-SN CPC can be configured with event A3/event A5. Hence, A4 can be configured for subsequent CPC and CPA. Due to the support of delta configuration, all candidate SCG configuration can be the delta of reference whether it is used for CPA or CPC. According to above analyses, CPA and CPC can use the same conditions (event A4) and candidate SCG configuration. Furthermore, Option 1 may be a simple and effective method for “CPA” selective activation of cell groups with lower impact on specification. Option 1 can be supported.

Proposal 6: For selective activation of SCG, it is not needed to distinguish between CPC and CPA and the same conditional configuration can be used for CPC and CPA configuration.

Because “CPA” selective activation of cell groups has been supported, UE should also keep conditional configuration after SCG release.

Proposal 7: In SCG selective activation, UE should keep CPC/CPA configurations after SCG release.

Delta configuration

To reduce the signalling overhead, RAN2 has supported delta configuration, and there need to be a known reference. In legacy CPC procedure, UE can calculate the configuration of the candidate target SCG with reference to current configuration of serving SCG. So, the reference is the current configuration of SCG. In selective activation of cell groups, the SCG configuration will not be release and reconfigured but the current SCG configuration has been changed after SCG change. Therefore, the full configuration of candidate SCG cannot be acquired based on current SCG configuration and delta configuration of candidate SCG in CPC configuration. According to the differentiation between Rel-17 CPC and Rel-18 SCG selective activation, legacy principle for delta configuration cannot be reused in Rel-18. Which configuration can be the known reference for the configuration of selective activation needs to be discussed and the following options can be investigated:

Option 1: Network can provide a separate reference (SCG) configuration for all candidate SCG configurations.

Option 2: The reference is current SCG configuration, and the newly added candidate SCG configuration can be delta of the current SCG configuration, at the time when the candidate SCG configuration is provided.

For option1, UE only needs to store the reference (SCG) configuration which is provided by network, full configuration of each candidate SCG can be calculated referencing to the same reference (SCG) configuration. However, option 2 will require UE store a long list of “the current serving cell configuration” for each candidate SCG configuration. Hence, Option 1 may be a better way.

Proposal 8: Network can provide a separate reference (SCG) configuration for all candidate SCG configurations.

For inter-SN CPC and CPA, MN should provide the reference configuration to UE and all candidate T-SNs. T-SNs generate the candidate SCG configuration as the delta of the reference configuration and transmits it to MN. If MN updates the reference configuration, MN need to inform all candidate T-SNs to reconfigure the SCG configuration, and then MN provides the updated reference configuration and CPC/CPA configurations to UE.

Proposal 9: For inter-SN CPC and CPA, MN should provide the latest reference configuration to all candidate T-SNs.

Security

Security of NR-DC with selective activation of the cell groups has been discussed in previous meetings but no agreement has been reached for it. The scope of NR-DC with selective activation has been limited to SCG. So, we need to investigate whether there is a security issue in SCG selective activation with high priority. The potential issue of SCG selective activation is that UE may reuse the same security parameters for subsequent SCG switch.

For intra-SN, the same SN key can be reused for subsequent SCG change and there is no security issue without SN change.

Observation 1: There is no security issue for intra-SN SCG selective activation.

For inter-SN mobility, there are two cases need to be investigated about security parameters re-used:

Case 1: The same sk-Counter value is provided for each candidate SCG configuration.

Case 2: Different sk-Counter values are provided for each candidate SCG configuration.

For case 1, Key reuse will occur for every time UE perform subsequent CPC. Even though all candidate SCGs are configured with different sk-Counter values (Case 2), when UE goes back to a previous cell, it will also face the key reuse unless it is not allowed that UE perform subsequent CPC to the same candidate SCG more than once. Due to the unpredictable movement of UE, it is difficult for network to configure appropriate sk-Counter values for each candidate SCG to satisfy the monotonically increment principle of SN Counter as specified in TS 33.501 [4]. Hence, RAN2 needs to ask SA3 whether case 2 can be supported and work well.

Observation 2: For inter-SN SCG selective activation, if all candidate SCGs are configured with different sk-Counter values and UE is not allowed to perform subsequent CPC to the same candidate SCG more than once, there is no security issue and the SN key will not be reused.

Observation 3: In the case of each candidate SCG with different sk-Counter values, it is difficult for network to configure appropriate sk-Counter values for each candidate SCG to satisfy the monotonically increment principle of SN Counter.

In RAN2#119bis-e meeting, “RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security” has been agreed. So, only UE autonomous-based mechanisms can be used to solve the issue. Network and UE can automatically refresh the sk-Counter using the same criterion after SCG change. i.e. increment by 1.

For case1, the fresh sk-Counter can be used for the subsequent CPC. For case 2, the fresh sk-Counter can be used for the prospective CPC where UE access to the selected candidate SCG again.

Proposal 10: Network and UE can automatically refresh the sk-Counter using the same criterion after SCG change. i.e. increment by 1 to solve the security issue in inter-SN SCG selective activation.

Proposal 11: RAN2 needs to ask SA3 whether the simultaneous existence of multiple candidate SCGs with different sk-Counter values can be supported to achieve subsequent CPC and to evaluate the feasibility of the method in P10.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Proposal 1: RAN2 can consider the following scenarios of SCG selective activation:

- Scenario 1: MN initiated SCG selective activation

- Scenario 2: SN initiated SCG selective activation

Proposal 2: RAN2 needs to discuss whether UE needs to keep current serving cell configuration and associated conditions after SCG addition or change.

Proposal 3: To support subsequent secondary cell group change, UE keeps CPC/CPA configurations based on the indication provided by network.

Proposal 4: RAN2 doesn’t support nested configurations for SCG selective activation.

Proposal 5: In SCG selective activation, UE should release CPC/CPA configurations after MCG change.

Proposal 6: For selective activation of SCG, it is not needed to distinguish between CPC and CPA and the same conditional configuration can be used for CPC and CPA configuration.

Proposal 7: In SCG selective activation, UE should keep CPC/CPA configurations after SCG release.

Proposal 8: Network can provide a separate reference (SCG) configuration for all candidate SCG configurations.

Proposal 9: For inter-SN CPC and CPA, MN should provide the latest reference configuration to all candidate T-SNs.

Observation 1: There is no security issue for intra-SN SCG selective activation.

Observation 2: For inter-SN SCG selective activation, if all candidate SCGs are configured with different sk-Counter values and UE is not allowed to perform subsequent CPC to the same candidate SCG more than once, there is no security issue and the SN key will not be reused.

Observation 3: In the case of each candidate SCG with different sk-Counter values, it is difficult for network to configure appropriate sk-Counter values for each candidate SCG to satisfy the monotonically increment principle of SN Counter.

Proposal 10: Network and UE can automatically refresh the sk-Counter using the same criterion after SCG change. i.e. increment by 1 to solve the security issue in inter-SN SCG selective activation.

Proposal 11: RAN2 needs to ask SA3 whether the simultaneous existence of multiple candidate SCGs with different sk-Counter values can be supported to achieve subsequent CPC and to evaluate the feasibility of the method in P10.

Reference

[1] RP-221799, MediaTek Inc., “Revised WID on Further NR mobility enhancements”

[2] R2-2209301, “Report of 3GPP TSG RAN WG2 meeting #119-e”

[3] R2-xxxxxx, “Report of 3GPP TSG RAN WG2 meeting #119bis-e”

[4] TS 33.501, “Security architecture and procedures for 5G system”