3GPP TSG-RAN WG2 Meeting #120	R2-2212716

# Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3725 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous RRC corrections for SL enhancement |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | F |  |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Issue #1: Whether the “Mode 1” limitation is needed for TX UE to report SL DRX on/off indication in groupcast case In the last RAN2#119bis-e meeting, the issue#1 is postponed as below: [Proposal 10] “Mode 1” condition for GC for clause 5.8.3.2 “Initiation” in R2-2209878 is agreed. (9/11) - Postponed. The usage of SL DRX on/off indication in groupcast case is justified as follows: On one hand, the “Mode 1” limitation is firstly introduced because the gNB is responsible for the SL DRX configuration for unicast case and TX UE in RRC CONNECTED and Mode 1 RA. However, it is not applicable to groupcast case since the TX UE decides the SL DRX configuration for RX UE. On the other hand, the SL DRX on/off indication for the associated Destination L2 ID is beneficial for alignment of Uu DRX of TX UE and SL DRX of RX UE. In such case, the gNB can provide proper Uu DRX configuration to TX UE. Moreover, this should be common for mode 1 and mode 2 as long as the TX UE is in RRC CONNECTED. As above, some procedure text in clause 5.8.3.3 are updated to remove the “Mode 1” limitation for TX UE to report SL DRX on/off indication in groupcast case. Issue #2: Clarification on field SearchSpaceLinkingId based on RAN1 LS in R2-2211155(R1-2210735) According to RAN1 LS in R2-2211155(R1-2210735), RAN1 has made the following conclusion: There is no consensus in RAN1 on the support of PDCCH repetition for search space sets configured with SL DCI format 3_0 and/or 3_1 in Rel-17. Since the current RAN1 specification does not cover the case for SL DCI format 3_0 and/or 3_1 with PDCCH repetition, some clarification is made on field description of SearchSpaceLinkingId in order to be consistent with the RAN1 specification. Issue #3: Clarification on the default CBR parameter in field sl-DefaultTxConfigIndex In the last RAN2#119bis-e meeting, the following Case 3 is postponed: - Case 1: partial sensing, R17 normal pool, R17 default CBR – partial - Case 2a: random selection, R17 normal pool, R17 default CBR – random - Case 2b: random selection, R16/17 exceptional pool, R16 default CBR - Case 3: full sensing, R16/17 normal pool, R16 default CBR or invalid case? - Case 1, 2a, 2b are confirmed. Case 3 will be revisited next meeting. As in R16 sidelink, the default CBR is applicable for the normal resource pool (with full sensing operation) and the exceptional pool when the UE has no available CBR measurement results. Therefore, for the above Case 3, the R17 UE behaviour can be inherited from legacy mechanism i.e., as long as the UE is performing full sensing regardless on the R16/R17 normal pool, if the UE has no available CBR measurement results, the R16 default CBR is applied. Some clarification is made on field description of sl-DefaultTxConfigIndex correspondingly. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: In clause 5.8.3.3, some procedure text is updated to remove the “Mode 1” limitation for TX UE to report SL DRX on/off indication in groupcast case. Change #2: In the field description of SearchSpaceLinkingId, clarify that this parameter is not applicable to search space configured with dci-FormatsSL for monitoring format 3_0 and/or format 3_1. Change #3: In the field description of sl-DefaultTxConfigIndex, clarify that this field is applicable only to R16/R17 exceptional pool or when the UE is performing full sensing on the R16/17 normal pool. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality NR Sidelink enhancement Inter-operability: If the UE implements the CR but not the network, there is no inter-operability issue. If the network implements the CR but not the UE, there is no inter-operability issue. If one UE implements the CR but not the other UE, there is no inter-operability issue. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Consequences if Change #1 is not approved: The TX UE in mode 1 cannot report SL DRX on/off indication in groupcast case, and thus the gNB may not configure proper Uu DRX configuration for the alignment of Uu DRX and SL DRX. Consequences if Change #2 is not approved: inconsistent understanding on the support of PDCCH repetition for search space sets configured with SL DCI format 3_0 and/or 3_1 may occur across RAN1 and RAN2 specifications. Consequences if Change #3 is not approved: The cases when the UE can use the R16 default CBR value configured by the sl-DefaultTxConfigIndex are unclear, which may lead to congestion control performance downgraded in such cases. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.3.3, 6.3.2, 6.3.5 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 5.8.3.3	Actions related to transmission of SidelinkUEInformationNR message

The UE shall set the contents of the SidelinkUEInformationNR message as follows:

1>	if the UE initiates the procedure to indicate it is (no more) interested to receive NR sidelink communication or to request (configuration/ release) of NR sidelink communication transmission resources or to report to the network that a sidelink radio link failure or sidelink RRC reconfiguration failure has been declared or to report to the network the sidelink DRX configuration for NR sidelink unicast reception or to report to the network the sidelink DRX assistance information or the sidelink DRX configuration reject information for NR sidelink unicast transmission or to report to the network the Destination Layer-2 ID and QoS profile associated with its interested services that sidelink DRX is applied for NR sidelink groupcast or broadcast reception or to report to the network the Destination Layer-2 ID and the sidelink DRX on/off indication for the corresponding destination for NR sidelink groupcast transmission or to indicate it is (no more) interested to receive NR sidelink discovery messages or to request (configuration/ release) of NR sidelink discovery messages transmission resources or to request (configuration/ release) of NR sidelink U2N relay communication transmission resources (i.e. UE includes all concerned information, irrespective of what triggered the procedure):

2>	if SIB12 including sl-ConfigCommonNR is provided by the PCell:

3>	if configured by upper layers to receive NR sidelink communication:

4>	include sl-RxInterestedFreqList and set it to the frequency for NR sidelink communication reception;

3>	if configured by upper layers to transmit non-relay NR sidelink communication:

4>	include sl-TxResourceReqList and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink communication resource:

5>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink communication transmission;

5>	set sl-CastType to the cast type of the associated destination identity configured by the upper layer for the NR sidelink communication transmission;

5>	set sl-RLC-ModeIndication to include the RLC mode(s) and optionally QoS profile(s) of the sidelink QoS flow(s) of the associated RLC mode(s), if the associated bi-directional sidelink DRB has been established due to the configuration by RRCReconfigurationSidelink;

5>	set sl-QoS-InfoList to include QoS profile(s) of the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink communication transmission;

5>	set sl-InterestedFreqList to indicate the frequency of the associated destination for NR sidelink communication transmission;

5>	set sl-TypeTxSyncList to the current synchronization reference type used on the associated sl-InterestedFreqList for NR sidelink communication transmission.

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from the associated peer UE.

4>	if a sidelink radio link failure or a sidelink RRC reconfiguration failure has been declared, according to clauses 5.8.9.3 and 5.8.9.1.8, respectively;

5>	include sl-FailureList and set its fields as follows for each destination for which it reports the NR sidelink communication failure:

6>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink communication transmission;

6>	if the sidelink RLF is detected as specified in clause 5.8.9.3:

7>	set sl-Failure as rlf for the associated destination for the NR sidelink communication transmission;

6>	else if RRCReconfigurationFailureSidelink is received:

7>	set sl-Failure as configFailure for the associated destination for the NR sidelink communication transmission;

3>	if SIB12 including sl-NonRelayDiscovery and if configured by upper layers to receive NR sidelink non-relay discovery messages, or if SIB12 including sl-L2U2N-Relay and if configured by upper layers to receive NR sidelink L2 U2N relay discovery messages, or if SIB12 including sl-L3U2N-RelayDiscovery and if configured by upper layers to receive NR sidelink L3 U2N relay discovery messages:

4>	include sl-RxInterestedFreqListDisc and set it to the frequency for NR sidelink discovery messages reception;

4>	if the UE is capable of L2 U2N remote UE:

5>	include sl-SourceIdentityRemoteUE and set it to the source identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

3>	if SIB12 including sl-NonRelayDiscovery and if configured by upper layers to transmit NR sidelink non-relay discovery messages, or if SIB12 including sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay discovery messages, or if SIB12 including sl-L3U2N-RelayDiscovery and if configured by upper layers to transmit NR sidelink L3 U2N relay discovery messages:

4>	include sl-TxResourceReqListDisc and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink discovery messages resource:

5>	set sl-DestinationIdentityDisc to the destination identity configured by upper layer for NR sidelink discoverymessages transmission;

5>	if the UE is acting as L2 U2N Relay UE:

6>	set sl-SourceIdentityRelayUE to the source identity configured by upper layer for NR sidelink L2 U2N relay discovery messages transmission;

5>	set sl-CastTypeDisc to the cast type of the associated destination identity configured by the upper layer for the NR sidelink discovery messages transmission;

5>	set sl-TxInterestedFreqListDisc to indicate the frequency of the associated destination for NR sidelink discovery messages transmission;

5>	set sl-TypeTxSyncListDisc to the current synchronization reference type used on the associated sl-InterestedFreqList for NR sidelink discovery messages transmission;

5>	set sl-DiscoveryType to the current discovery type of the associated destination identity configured by the upper layer for NR sidelink discovery messages transmission;

3>	if SIB12 including sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay communication and the UE is acting as L2 U2N Relay UE:

4>	include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L2 U2N relay communication resource:

5>	set sl-DestinationIdentityL2U2N to the destination identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-InterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission;

5>	set sl-LocalID-Request to request local ID for L2 U2N Remote UE;

5>	set sl-PagingIdentityRemoteUE to the paging UE ID received from peer L2 U2N Remote UE, if it is not released as in 5.8.9.8.3;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE.

4>	include ue-Type and set it to relayUE;

3>	if SIB12 including sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay communication and the UE has a selected L2 U2N Relay UE:

4>	include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows to request network to assign NR sidelink L2 U2N relay communication resource:

5>	set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-InterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE.

4>	include ue-Type and set it to remoteUE;

3>	if SIB12 including sl-L3U2N-RelayDiscovery and if configured by upper layers to transmit NR sidelink L3 U2N relay communication:

4>	include sl-TxResourceReqL3U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L3 U2N relay communication resource:

5>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink L3 U2N relay communication transmission;

5>	set sl-CastType to the cast type of the associated destination identity configured by the upper layer for the NR sidelink L3 U2N relay communication transmission;

5>	set sl-RLC-ModeIndication to include the RLC mode(s) and optionally QoS profile(s) of the sidelink QoS flow(s) of the associated RLC mode(s), if the associated bi-directional sidelink DRB has been established due to the configuration by RRCReconfigurationSidelink;

5>	set sl-QoS-InfoList to include QoS profile(s) of the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink L3 U2N relay communication transmission;

5>	set sl-TxInterestedFreqList to indicate the frequency of the associated destination for NR sidelink L3 U2N relay communication transmission;

5>	set sl-TypeTxSyncList to the current synchronization reference type used on the associated sl-InterestedFreqList for NR sidelink L3 U2N relay communication transmission.

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE.

4>	include ue-Type and set it to relayUE if the UE is acting as NR sidelink L3 U2N Relay UE or to remoteUE otherwise;

3>	if sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs:

4>	if configured by upper layers to perform NR sidelink reception:

5>	include sl-RxDRX-ReportList and set its fields (if needed) as follows for each destination for which it reports to network:

6>	set sl-DRX-ConfigFromTx to include the accepted sidelink DRX configuration of the associated destination for NR sidelink unicast communication, if received from the associated peer UE;

5>	include sl-RxInterestedGC-BC-DestList and set its fields (if needed) as follows for each Destination Layer-2 ID for which it reports to network:

6>	set sl-RxInterestedQoS-InfoList to include the QoS profile of its interested service(s) that sidelink DRX is applied for the associated destination for NR sidelink groupcast or broadcast reception;

NOTE:	It is up to UE implementation to set the QoS profile in sl-RxInterestedQoS-InfoList for NR sidelink discovery reception.

6>	set sl-DestinationIdentity to the associated destination identity configured by upper layer for NR sidelink groupcast or broadcast reception;

4>	if configured by upper layers to perform NR sidelink transmission:

5>	include sl-TxResourceReqList and/or sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it reports to network:

6>	set sl-DRX-Indication to include the sidelink DRX on/off indication for the associated destination for NR sidelink groupcast transmission;

6>	if configured with sl-ScheduledConfig:

7>	set sl-DRX-InfoFromRxList to include the sidelink DRX assistance information of the associated destination, if any, received from the associated peer UE;

7>	if the RRCReconfigurationCompleteSidelink message includes the sl-DRX-ConfigReject:

8>	set sl-Failure as drxReject-v1710 for the associated destination for the NR sidelink communication transmission;

1>	if the UE initiates the procedure while connected to an E-UTRA PCell:

2>	submit the SidelinkUEInformationNR to lower layers via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified in TS 36.331 [10], clause 5.6.28;

1>	else:

2>	submit the SidelinkUEInformationNR message to lower layers for transmission.

START OF NEXT CHANGE

### 6.3.2	Radio resource control information elements

#### –	SearchSpace

The IE SearchSpace defines how/where to search for PDCCH candidates. Each search space is associated with one ControlResourceSet. For a scheduled SCell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent (regardless of their presence conditions). For a scheduled SpCell in the case of the cross carrier scheduling, if the search space is linked to another search space in the scheduling SCell, all the optional fields of this search space in the scheduled SpCell are absent (regardless of their presence conditions) except for nrofCandidates.

SearchSpace information element

-- ASN1START

-- TAG-SEARCHSPACE-START

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639),

sl1280                                  INTEGER (0..1279),

sl2560                                  INTEGER (0..2559)

}                                                                                                   OPTIONAL,   -- Cond Setup4

duration                                INTEGER (2..2559)                                           OPTIONAL,   -- Need S

monitoringSymbolsWithinSlot             BIT STRING (SIZE (14))                                      OPTIONAL,   -- Cond Setup

nrofCandidates                          SEQUENCE {

aggregationLevel1                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16                      ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}                                                                                                   OPTIONAL,   -- Cond Setup

searchSpaceType                         CHOICE {

common                                  SEQUENCE {

dci-Format0-0-AndFormat1-0              SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-0                           SEQUENCE {

nrofCandidates-SFI                      SEQUENCE {

aggregationLevel1                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16                      ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-1                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-2                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-3                           SEQUENCE {

dummy1                                  ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}  OPTIONAL,   -- Cond Setup

dummy2                                  ENUMERATED {n1, n2},

...

}                                                                                           OPTIONAL    -- Need R

},

ue-Specific                                 SEQUENCE {

dci-Formats                                 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...,

[[

dci-Formats-MT-r16                   ENUMERATED {formats2-5}                                OPTIONAL,    -- Need R

dci-FormatsSL-r16                    ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1, formats3-0, formats3-1,

formats3-0-And-3-1}                        OPTIONAL,    -- Need R

dci-FormatsExt-r16                   ENUMERATED {formats0-2-And-1-2, formats0-1-And-1-1And-0-2-And-1-2}

OPTIONAL     -- Need R

]]

}

}                                                                                                   OPTIONAL    -- Cond Setup2

}

SearchSpaceExt-r16 ::=                   SEQUENCE {

controlResourceSetId-r16                ControlResourceSetId-r16                                    OPTIONAL,   -- Cond SetupOnly2

searchSpaceType-r16                     SEQUENCE {

common-r16                              SEQUENCE {

dci-Format2-4-r16                       SEQUENCE {

nrofCandidates-CI-r16                   SEQUENCE {

aggregationLevel1-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16-r16                  ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-5-r16                      SEQUENCE {

nrofCandidates-IAB-r16                  SEQUENCE {

aggregationLevel1-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16-r16                  ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-6-r16                       SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

...

}

}                                                                                                   OPTIONAL,    -- Cond Setup3

searchSpaceGroupIdList-r16                      SEQUENCE (SIZE (1.. 2)) OF INTEGER (0..1)           OPTIONAL,    -- Need R

freqMonitorLocations-r16                        BIT STRING (SIZE (5))                               OPTIONAL     -- Need R

}

SearchSpaceExt-v1700 ::=            SEQUENCE {

monitoringSlotPeriodicityAndOffset-v1710 CHOICE {

sl32                                     INTEGER (0..31),

sl64                                     INTEGER (0..63),

sl128                                    INTEGER (0..127),

sl5120                                   INTEGER (0..5119),

sl10240                                  INTEGER (0..10239),

sl20480                                  INTEGER (0..20479)

}                                                                                                   OPTIONAL,   -- Cond Setup5

monitoringSlotsWithinSlotGroup-r17       CHOICE {

slotGroupLength4-r17                     BIT STRING (SIZE (4)),

slotGroupLength8-r17                     BIT STRING (SIZE (8))

}                                                                                                   OPTIONAL,   -- Need R

duration-r17                             INTEGER (4..20476)                                         OPTIONAL,   -- Need R

searchSpaceType-r17             SEQUENCE{

common-r17                      SEQUENCE {

dci-Format4-0-r17               SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format4-1-r17               SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format4-2-r17               SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format4-1-AndFormat4-2-r17  SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-7-r17               SEQUENCE {

nrofCandidates-PEI-r17          SEQUENCE {

aggregationLevel4-r17       ENUMERATED {n0, n1, n2, n3, n4}                         OPTIONAL,   -- Need R

aggregationLevel8-r17       ENUMERATED {n0, n1, n2}                                 OPTIONAL,   -- Need R

aggregationLevel16-r17      ENUMERATED {n0, n1}                                     OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL    -- Need R

}

}                                                                                                   OPTIONAL,   -- Need R

searchSpaceGroupIdList-r17          SEQUENCE (SIZE (1.. 3)) OF INTEGER (0.. maxNrofSearchSpaceGroups-1-r17)  OPTIONAL,  -- Cond DedicatedOnly

searchSpaceLinkingId-r17            INTEGER (0..maxNrofSearchSpacesLinks-1-r17)                     OPTIONAL    -- Cond DedicatedOnly

}

-- TAG-SEARCHSPACE-STOP

-- ASN1STOP

SearchSpace field descriptions

common Configures this search space as common search space (CSS) and DCI formats to monitor.

controlResourceSetId The CORESET applicable for this SearchSpace. Value 0 identifies the common CORESET#0 configured in MIB and in ServingCellConfigCommon. Values 1..maxNrofControlResourceSets-1 identify CORESETs configured in System Information or by dedicated signalling. The CORESETs with non-zero controlResourceSetId are configured in the same BWP as this SearchSpace except commonControlResourceSetExt which is configured by SIB20. If the field controlResourceSetId-r16 is present, UE shall ignore the controlResourceSetId (without suffix).

dummy1, dummy2 This field is not used in the specification. If received it shall be ignored by the UE.

dci-Format0-0-AndFormat1-0 If configured, the UE monitors the DCI formats 0_0 and 1_0 according to TS 38.213 [13], clause 10.1.

dci-Format2-0 If configured, UE monitors the DCI format 2_0 according to TS 38.213 [13], clause 10.1, 11.1.1.

dci-Format2-1 If configured, UE monitors the DCI format 2_1 according to TS 38.213 [13], clause 10.1, 11.2.

dci-Format2-2 If configured, UE monitors the DCI format 2_2 according to TS 38.213 [13], clause 10.1, 11.3.

dci-Format2-3 If configured, UE monitors the DCI format 2_3 according to TS 38.213 [13], clause 10.1, 11.4

dci-Format2-4 If configured, UE monitors the DCI format 2_4 according to TS 38.213 [13], clause 11.2A.

dci-Format2-5 If configured, IAB-MT monitors the DCI format 2_5 according to TS 38.213 [13], clause 14.

dci-Format2-6 If configured, UE monitors the DCI format 2_6 according to TS 38.213 [13], clause 10.1, 10.3. DCI format 2_6 can only be configured on the SpCell.

dci-Format2-7 If configured, UE monitors the DCI format 2_7 according to TS 38.213 [13], clause 10.1, 10.4A.

dci-Format4-0 If configured, the UE monitors the DCI format 4_0 with CRC scrambled by MCCH-RNTI/G-RNTI according to TS 38.213 [13], clause [10.1].

dci-Format4-1-AndFormat4-2 If configured, the UE monitors the DCI format 4_1 and 4_2 with CRC scrambled by G-RNTI/G-CS-RNTI according to TS 38.213 [13], clause [11.1].

dci-Format4-1 If configured, the UE monitors the DCI format 4_1 with CRC scrambled by G-RNTI/G-CS-RNTI according to TS 38.213 [13], clause [10.1].

dci-Format4-2 If configured, the UE monitors the DCI format 4_2 with CRC scrambled by G-RNTI/G-CS-RNTI according to TS 38.213 [13], clause [10.1].

dci-Formats Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1.

dci-FormatsExt If this field is present, the field dci-Formats is ignored and dci-FormatsExt is used instead to indicate whether the UE monitors in this USS for DCI format 0_2 and 1_2 or formats 0_1 and 1_1 and 0_2 and 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 10.1). This field is not configured for operation with shared spectrum channel access in this release.

dci-Formats-MT Indicates whether the IAB-MT monitors the DCI formats 2-5 according to TS 38.213 [13], clause 14.

dci-FormatsSL Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1 or for format 3-0 or for format 3-1 or for formats 3-0 and 3-1. If this field is present, the field dci-Formats is ignored and dci-FormatsSL is used.

duration Number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the UE applies the value 1 slot, except for DCI format 2_0. The UE ignores this field for DCI format 2_0. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset). For SCS 480 kHz and SCS 960 kHz, duration-r17 is used, and the configured duration is restricted to be an integer multiple of L slots and smaller than periodicity, where L is the configured length of the bitmap monitoringSlotsWithinSlotGroup-r17. If duration-r17 is absent, the UE assumes the duration in slots is equal to L. The maximum valid duration is periodicity-L. For IAB-MT, duration indicates number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the IAB-MT applies the value 1 slot, except for DCI format 2_0 and DCI format 2_5. The IAB-MT ignores this field for DCI format 2_0 and DCI format 2_5. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset).

freqMonitorLocations Defines an association of the search space to multiple monitoring locations in the frequency domain and indicates whether the pattern configured in the associated CORESET is replicated to a specific RB set, see TS 38.213, clause 10.1. Each bit in the bitmap corresponds to one RB set, and the leftmost (most significant) bit corresponds to RB set 0 in the BWP. A bit set to 1 indicates that a frequency domain resource allocation replicated from the pattern configured in the associated CORESET is mapped to the RB set.

monitoringSlotPeriodicityAndOffset Slots for PDCCH Monitoring configured as periodicity and offset. For SCS 15, 30, 60, and 120 kHz and if the UE is configured to monitor: - DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. - DCI format 2_0, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213 [13], clause 10). - DCI format 2_4, only the values 'sl1', 'sl2', 'sl4', 'sl5', 'sl8' and 'sl10' are applicable. For SCS 480 kHz and if the UE is configured to monitor: - DCI format 2_0, only the values 'sl4', 'sl8', 'sl16', 'sl20', 'sl32', 'sl40', 'sl64', and 'sl80' are applicable. - DCI format 2_1, only the values ′sl4′, ′sl8′, and ′sl16′ are applicable. - DCI format 2_4, only the values 'sl4', 'sl8', 'sl16', 'sl20', 'sl32', 'sl40' are applicable. For SCS 960 kHz and if the UE is configured to monitor: - DCI format 2_0, only the values 'sl4', 'sl8', 'sl16', 'sl20', 'sl32', 'sl40', 'sl64', and 'sl80' are applicable. - DCI format 2_1, only the values ′sl4′, ′sl8′, and ′sl16′ are applicable. - DCI format 2_4, only the values 'sl8', 'sl16', 'sl32', 'sl40', 'sl64', 'sl80' are applicable. For SCS 480 kHz and SCS 960 kHz, and the configured periodicity and offset are restricted to be an integer multiple of L slots, where L is the configured length of the bitmap provided by monitoringSlotsWithinSlotGroup-r17, i.e. for a given periodicity, the offset has a range of {0, L, 2*L, …, L*FLOOR(1/L*(periodicity-1))}. For IAB-MT, If the IAB-MT is configured to monitor DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. If the IAB-MT is configured to monitor DCI format 2_0 or DCI format 2_5, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213, clause 10). If monitoringSlotPeriodicityAndOffset-r17 is present, any previously configured monitoringSlotPeriodicityAndOffset is released, and if monitoringSlotPeriodicityAndOffset is present, any previously configured monitoringSlotPeriodicityAndOffset-r17 is released.

monitoringSlotsWithinSlotGroup Indicates which slot(s) within a slot group are configured for multi-slot PDCCH monitoring. The first (leftmost, most significant) bit represents the first slot in the slot group, the second bit represents the second slot in the slot group, and so on. A bit set to '1' indicates that the corresponding slot is configured for multi-slot PDCCH monitoring (see TS 38.213 [13], clause 10). The number of slots for multi-slot PDCCH monitoring is configured according to clause 10 in TS 38.213 [13].

monitoringSymbolsWithinSlot The first symbol(s) for PDCCH monitoring in the slots configured for (multi-slot) PDCCH monitoring (see monitoringSlotPeriodicityAndOffset and duration). The most significant (left) bit represents the first OFDM in a slot, and the second most significant (left) bit represents the second OFDM symbol in a slot and so on. The bit(s) set to one identify the first OFDM symbol(s) of the control resource set within a slot. If the cyclic prefix of the BWP is set to extended CP, the last two bits within the bit string shall be ignored by the UE or IAB-MT. For DCI format 2_0, the first one symbol applies if the duration of CORESET (in the IE ControlResourceSet) identified by controlResourceSetId indicates 3 symbols, the first two symbols apply if the duration of CORESET identified by controlResourceSetId indicates 2 symbols, and the first three symbols apply if the duration of CORESET identified by controlResourceSetId indicates 1 symbol. See TS 38.213 [13], clause 10. For IAB-MT: For DCI format 2_0 or DCI format 2_5, the first one symbol applies if the duration of CORESET (in the IE ControlResourceSet) identified by controlResourceSetId indicates 3 symbols, the first two symbols apply if the duration of CORESET identified by controlResourceSetId indicates 2 symbols, and the first three symbols apply if the duration of CORESET identified by controlResourceSetId indicates 1 symbol. See TS 38.213 [13], clause 10.

nrofCandidates-CI The number of PDCCH candidates specifically for format 2-4 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. The network configures only one aggregationLevel and the corresponding number of candidates (see TS 38.213 [13], clause 10.1).

nrofCandidates-PEI The number of PDCCH candidates specifically for format 2-7 for the configured aggregation level.

nrofCandidates-SFI The number of PDCCH candidates specifically for format 2-0 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. The network configures only one aggregationLevel and the corresponding number of candidates (see TS 38.213 [13], clause 11.1.1). For a search space configured with freqMonitorLocations-r16, only value ′n1′ is valid.

nrofCandidates Number of PDCCH candidates per aggregation level. The number of candidates and aggregation levels configured here applies to all formats unless a particular value is specified or a format-specific value is provided (see inside searchSpaceType). If configured in the SearchSpace of a cross carrier scheduled cell, this field determines the number of candidates and aggregation levels to be used on the linked scheduling cell (see TS 38.213 [13], clause 10).

searchSpaceGroupIdList-r16, searchSpaceGroupIdList-r17 List of search space group IDs which the search space is associated with. The network configures at most 2 search space groups per BWP where the group ID is either 0 or 1 if searchSpaceGroupIdList-r16 is included. The network configures at most 3 search space groups per BWP where the group ID is either 0, 1 or 2 if searchSpaceGroupIdList-r17 is included. And if searchSpaceGroupIdList-r17 is included, searchSpaceGroupIdList-r16 is ignored.

searchSpaceId Identity of the search space. SearchSpaceId = 0 identifies the searchSpaceZero configured via PBCH (MIB) or ServingCellConfigCommon and may hence not be used in the SearchSpace IE. The searchSpaceId is unique among the BWPs of a Serving Cell. In case of cross carrier scheduling, search spaces with the same searchSpaceId in scheduled cell and scheduling cell are linked to each other. The UE applies the search space for the scheduled cell only if the DL BWPs in which the linked search spaces are configured in scheduling cell and scheduled cell are both active. For an IAB-MT, the search space defines how/where to search for PDCCH candidates for an IAB-MT where each search space is associated with one ControlResearchSet and for a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent.

SearchSpaceLinkingId This parameter is used to link two search spaces of same type in the same BWP. If two search spaces have the same SearchSpaceLinkingId UE assumes these search spaces are linked to PDCCH repetition REF. When PDCCH repetition is monitored in two linked search space (SS) sets, the UE does not expect a third monitored SS set to be linked with any of the two linked SS sets. The two linked SS sets have the same SS set type (USS/CSS). The two linked SS sets have the same DCI formats to monitor. For intra-slot PDCCH repetition: The two SS sets should have the same periodicity and offset (monitoringSlotPeriodicityAndOffset), and the same duration. For linking monitoring occasions across the two SS sets that exist in the same slot: The two SS sets have the same number of monitoring occasions within a slot and n-th monitoring occasion of one SS set is linked to n-th monitoring occasion of the other SS set. The following SS sets cannot be linked with another SS set for PDCCH repetition: SS set 0, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, searchSpaceBroadcast, peiSearchSpace, and sdt-SearchSpace. SS set configured by recoverySearchSpaceId cannot be linked to another SS set for PDCCH repetition. When a scheduled serving cell is configured to be cross-carrier scheduled by a scheduling serving cell, two PDCCH candidates (with the same AL and candidate index associated with the scheduled serving cell) are linked only if the corresponding two SS sets in the scheduling serving cell are linked and two SS sets in the scheduled serving cell with the same SS set IDs are also linked. This parameter is not applicable to search space configured with dci-FormatsSL for monitoring format 3-0 or for format 3-1 or for formats 3-0 and 3-1.

searchSpaceType Indicates whether this is a common search space (present) or a UE specific search space as well as DCI formats to monitor for.

ue-Specific Configures this search space as UE specific search space (USS). The UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), and SP-CSI-RNTI (if configured)

START OF NEXT CHANGE

### 6.3.5	Sidelink information elements

#### –	SL-CBR-PriorityTxConfigList

The IE SL-CBR-PriorityTxConfigList indicates the mapping between PSSCH transmission parameter (such as MCS, PRB number, retransmission number, CR limit) sets by using the indexes of the configurations provided in sl-CBR-PSSCH-TxConfigList, CBR ranges by an index to the entry of the CBR range configuration in sl-CBR-RangeConfigList, and priority ranges. It also indicates the default PSSCH transmission parameters to be used when CBR measurement results are not available, and MCS range for the MCS tables used in the resource pool.

SL-CBR-PriorityTxConfigList information element

-- ASN1START

-- TAG-SL-CBR-PRIORITYTXCONFIGLIST-START

SL-CBR-PriorityTxConfigList-r16 ::= SEQUENCE (SIZE (1..8)) OF SL-PriorityTxConfigIndex-r16

SL-CBR-PriorityTxConfigList-v1650 ::= SEQUENCE (SIZE (1..8)) OF SL-PriorityTxConfigIndex-v1650

SL-PriorityTxConfigIndex-r16 ::=    SEQUENCE {

sl-PriorityThreshold-r16             INTEGER (1..8)                                                   OPTIONAL,    -- Need M

sl-DefaultTxConfigIndex-r16          INTEGER (0..maxCBR-Level-1-r16)                                  OPTIONAL,    -- Need M

sl-CBR-ConfigIndex-r16               INTEGER (0..maxCBR-Config-1-r16)                                 OPTIONAL,    -- Need M

sl-Tx-ConfigIndexList-r16            SEQUENCE (SIZE (1.. maxCBR-Level-r16)) OF SL-TxConfigIndex-r16   OPTIONAL     -- Need M

}

SL-PriorityTxConfigIndex-v1650 ::=  SEQUENCE {

sl-MCS-RangeList-r16                SEQUENCE (SIZE (1..maxCBR-Level-r16)) OF SL-MinMaxMCS-List-r16    OPTIONAL     -- Need M

}

SL-TxConfigIndex-r16 ::=            INTEGER (0..maxTxConfig-1-r16)

-- TAG-SL-CBR-PRIORITYTXCONFIGLIST-STOP

-- ASN1STOP

SL-CBR-PriorityTxConfigList field descriptions

sl-CBR-ConfigIndex Indicates the CBR ranges to be used by an index to the entry of the CBR range configuration in sl-CBR-RangeConfigList.

sl-DefaultTxConfigIndex Indicates the PSSCH transmission parameters to be used by the UEs which do not have available CBR measurement results, by means of an index to the corresponding entry in sl-Tx-ConfigIndexList. Value 0 indicates the first entry in sl-Tx-ConfigIndexList. The field is ignored if the UE has available CBR measurement results. This field is applicable only in case the sl-TxPoolExceptional is used, or when the UE is performing full sensing in case the sl-TxPoolExceptional is not used.

sl-MCS-RangeList Indicates the minimum MCS value and maximum MCS value for the associated MCS table(s). UE shall ignore the minimum MCS value and maximum MCS value used for table of 64QAM indicated in SL-CBR-PriorityTxConfigList-r16 if SL-CBR-PriorityTxConfigList-v1650 is present.

sl-PriorityThreshold Indicates the upper bound of priority range which is associated with the configurations in sl-CBR-ConfigIndex and in sl-Tx-ConfigIndexList. The upper bounds of the priority ranges are configured in ascending order for consecutive entries of SL-PriorityTxConfigIndex in SL-CBR-PriorityTxConfigList. For the first entry of SL-PriorityTxConfigIndex, the lower bound of the priority range is 1.

SL-CBR-PriorityTxConfigList-v1650 If included, it includes the same number of entries, and listed in the same order, as in SL-CBR-PriorityTxConfigList-r16.

END OF CHANGE