3GPP TSG-RAN WG2 Meeting #120	R2-2212818

Toulouse, France, 14th – 18th November, 2022

Agenda item:	8.4.4

Source:	Samsung

Title:	Considerations on CHO with CPA/CPC

Document for:	Discussion & Decision

Introduction

In RAN2#119e, the follow agreements were made.

Agreements:

- Observation: Current RAN2 Stage-3 specifications can support CHO including target MCG and target SCG in Rel-17.

- CHO configuration referring to or including CPC/CPA configuration (intended to be applicable together) can be supported.

- FFS: When triggering CHO, UE perform CPC/CPA configuration to start CPC/CPA evaluation, FFS if CHO evaluation and CPC/CPA evaluation is concurrent or sequential.

In this contribution, we will give our considerations on evaluation mode configuration and some issues on concurrent evaluation.

Discussion

On evaluation mode

Regarding the evaluation mode, whether CHO evaluation and CPC/CPA evaluation is concurrent or sequential can depend on how the CPA/CPC candidate PSCells are prepared and how the mapping relation between candidate PCells and PSCells is determined.

In one case, the candidate PSCells of CPA/CPC can be prepared and determined with the target MN involvement. In this case, the candidate PSCells associated with a PCell can be determined by the target MN, and the corresponding CPAC configuration is provided by the target MN encapsulated in the PCell configuration.

In another case, the candidate PSCells of CPA/CPC can be prepared and determined by the source MN without target MN involvement. In this case, the source MN prepares candidate PCells and candidate PSCells by communicating with potential target MNs and potential target SNs, respectively. The association between candidate MCG and candidate SCG is determined by the source MN.

Considering these two cases, the mapping relation between candidate PCells and PSCells can be configured explicitly or implicitly with the following options.

- Option 1: implicitly indicated by the embedded configuration structure, e.g., the condRRCReconfig for a candidate PCell contains the conditionalReconfiguration for CPA/CPC.

- Option 2: explicitly indicated by linking the CondReconfigIds of multiple candidate PSCells to the CondReconfigId of a candidate PCell;

- Option 3: no indication, meaning all candidate PSCells can associate to each candidate PCell.

Proposal 1: RAN2 to discuss the following configuration options for the mapping relation between candidate PCells and candidate PSCells.

- Option 1: implicitly indicated by the embedded configuration structure, e.g., the condRRCReconfig for a candidate PCell contains the conditionalReconfiguration for CPA/CPC.

- Option 2: explicitly indicated by linking the CondReconfigIds of multiple candidate PSCells to the CondReconfigId of a candidate PCell;

- Option 3: no indication, meaning all candidate PSCells can associate to each candidate PCell.

Depending on how the mapping relation is configured, the network can configure the evaluation mode (concurrent or sequential) explicitly or implicitly based on that. Option 1 implies sequential evaluation is configured implicitly. For Option 2 and Option 3, either sequential or concurrent evaluation can be configured explicitly.

Proposal 2: The evaluation mode of CHO with CPA/CPC, i.e., concurrent or sequential, can be configured by the NW.

If configured with sequential evaluation by explicit indication or by implicit embedded configuration, the UE should evaluate CHO and CPA/CPC sequentially. The UE first evaluates configured candidate PCells. Upon the execution condition for one candidate PCell is fulfilled, the UE applies the associated CPAC configuration and starts to evaluate the associated candidate PSCells. If configured with concurrent evaluation explicitly, the UE evaluates all candidate PCells and all candidate PSCells concurrently.

Proposal 3: If the sequential evaluation is configured, the UE starts to evaluate the execution conditions for candidate PSCells only when the execution condition for the associated candidate PCell is fulfilled.

Proposal 4: If the concurrent evaluation is configured, the UE evaluates the execution conditions for all candidate PCells and candidate PSCells simultaneously.

Some issues on concurrent evaluation

    - Potential scenarios for fulfillment of execution condition

In CHO with multiple candidate SCGs,the multiple sets of MCG+SCG configuration, i.e., each for a pair of candidate PCell and candidate PSCell, will be prepared. When concurrent evaluation is configured, UE should evaluates the execution conditions for all candidate PCells and candidate PSCells simultaneously.

But CHO execution condition and CPA/CPC execution condition for the paired PCell and PSCell might not be fulfilled simultaneously, and the potential scenarios are shown in table1.

Table1: Scenarios for fulfillment of CHO/CPAC execution condition

| scenarios | Fulfillment of CHO execution condition | Fulfillment of CPA/CPC execution condition | PSCell paired with PCell |
| --- | --- | --- | --- |
| 0 | √ | √ | √ |
| 1 | √ | √ | x |
| 2 | √ | x | - |
| 3 | x | √ | - |

There are different scenarios for fulfilment of CHO/CPAC execution condition in concurrent evaluation which will have different impact on UE’s behaviour. The suggest behaviour of UE for the different scenarios are given below.

- Scenario 0: Since CHO execution condition and CPA/CPC execution condition for the paired candidate PCell and candidate PSCell are fulfilled simultaneously, UE can execute CHO and CPA/CPC concurrently.

- Scenario 1:  For CHO execution condition and CPA/CPC execution condition are fulfilled simultaneously, with unpaired candidate PCell and candidate PSCell, UE can firstly perform CHO to access the target PCell, and keep evaluating execution condition of the paired candidate PSCells of the target PCell.

- Scenario 2: If only CHO execution condition of the candidate PCell is fulfilled, UE should perform CHO firstly, and then wait for RRCReconfiguration message for SCG configuration of the target PSCell selected by T-MN.

- Scenario 3: If only CPAC execution condition of the candidate PSCells is satisfied, UE should do nothing, just keep evaluating CHO execution condition.

Proposal 5: UE’s behavior should be specified for the different scenarios for fulfillment of CHO/CPAC execution condition in concurrent evaluation.

    - On execution optimization

When concurrent evaluation is configured, the execution condition of one candidate PCell and the execution condition of one candidate PSCell are fulfilled simultaneously, and if the PCell and the PSCell is paired, UE could execute CHO and CPA/CPC concurrently and access the PCell and PSCell both with good quality. In this case, the MR-DC mobility is optimized and the throughput during handover will be ensured.

Observation 1: When concurrent evaluation is configured, CHO and CPA/CPC execution concurrently can optimize MR-DC mobility and improve the throughput during handover.

Proposal 6: The methods for facilitating CHO and CPA/CPC execution concurrently need to be discussed.

To facilitate CHO and CPA/CPC execution concurrently for the paired PCell and PSCell, the following aspects could be considered:

Specify execution condition fulfilment of CHO with CPA/CPC

For the execution condition of CHO and that of CPA/CPC can’t be fulfilled at the same time exactly, certain rules have to be specified to determine the evaluation condition is fulfilled for concurrent CHO and CPA/CPC. One can consider the execution condition for CHO with CPA/CPC is fulfilled if the execution condition of one candidate PCell and the execution condition of one candidate PSCell are fulfilled simultaneously or one after another within a certain configured duration.

Proposal 7: If the concurrent evaluation is configured, consider the execution condition for CHO with CPA/CPC is fulfilled if the execution condition of one candidate PCell and the execution condition of one candidate PSCell are fulfilled one after another within a certain configured duration.

Coordination of the execution condition settings of CHO and CPA/CPC

The time of CHO execution condition are fulfilled and that of CPA/CPC execution condition are fulfilled might be relatively close if the execution condition settings of CHO, such as the value of variable hysteresis or timeToTrigger, and that of CPA/CPC can be configured with coordination.

Observation 2: Coordination of execution condition settings of CHO and that of CPA/CPC needs to be considered.

If the execution condition settings of CHO and CPA/CPC is configured in one node, the coordination of them could be feasible. Or there might need information exchange between the nodes.

There are two options for proper execution condition settings for CHO and CPA/CPC with coordination:

- Option1: S-MN determines both of the CHO execution condition and the CPA/CPC execution condition.

- Option2: T-MN determines the CPA/CPC execution condition according to the CHO execution condition.

Proposal 8:   The solutions of proper execution condition settings for CHO and CPA/CPC need to be discussed.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: RAN2 to discuss the following configuration options for the mapping relation between candidate PCells and candidate PSCells.

- Option 1: implicitly indicated by the embedded configuration structure, e.g., the condRRCReconfig for a candidate PCell contains the conditionalReconfiguration for CPA/CPC.

- Option 2: explicitly indicated by linking the CondReconfigIds of multiple candidate PSCells to the CondReconfigId of a candidate PCell;

- Option 3: no indication, meaning all candidate PSCells can associate to each candidate PCell.

Proposal 2: The evaluation mode of CHO with CPA/CPC, i.e., concurrent or sequential, can be configured by the NW.

Proposal 3: If the sequential evaluation is configured, the UE starts to evaluate the execution conditions for candidate PSCells only when the execution condition for the associated candidate PCell is fulfilled.

Proposal 4: If the concurrent evaluation is configured, the UE evaluates the execution conditions for all candidate PCells and candidate PSCells simultaneously.

Proposal 5: UE’s behavior should be specified for the different scenarios for fulfillment of CHO/CPAC execution condition in concurrent evaluation.

Observation 1: When concurrent evaluation is configured, CHO and CPA/CPC execution concurrently can optimize MR-DC mobility and improve the throughput during handover.

Proposal 6: The methods for facilitating CHO and CPA/CPC execution concurrently need to be discussed.

Proposal 7: If the concurrent evaluation is configured, consider the execution condition for CHO with CPA/CPC is fulfilled if the execution condition of one candidate PCell and the execution condition of one candidate PSCell are fulfilled one after another within a certain configured duration.

Observation 2: Coordination of execution condition settings of CHO and that of CPA/CPC needs to be considered.

Proposal 8:   The solutions of proper execution condition settings for CHO and CPA/CPC need to be discussed.

Reference

[1] RP-222332 New WID on Further NR mobility enhancements