3GPP TSG RAN WG2 #120	R2-2212841

Toulouse, France, November 14th – 18th, 2022

Source: 	MediaTek Inc.

Title: 	Recommendations for SSB/SIB1-less techniques

Agenda item:	8.3.3

Document for:	Discussion and Decision

Introduction

In this contribution, evaluation for SSB/SIB1-less SCell is conducted on the selected NWES schemes for light or medium cell load conditions (15% - 30%/30% - 50% RU). Recommendation is then provided based on the evaluated NW energy saving gains.

Network Energy Saving Techniques for Light Load or Medium Load Condition

In this section, we evaluate and reocommend NWES schemes for low load condition (15% - 30% RU) or medium load condition (30% - 50% RU)

SSB/SIB1-less SCell

When a cell is used as a capacity booster rather than providing coverage, the cell could be defined as a secondary cell (SCell), and a gNB may turn off SSB or SIB1 transmission on the SCell for network energy savings.

For evaluating the potential benefits, the following settings are considered:

- Scenario: Urban Macro (ISD: 500m) in FR1 of 30kHz with PCell and one SCell; multi-carrier case (inter-band CA)

  - Baseline: 100M 2CCs with SSB and SIB1 in both CCs

  - Improved: 100M 2CCs with SSB and SIB1 disabled in Scell

- SSB: 20 RBs, 4 symbols, 2 SSBs per slot, and 4 consecutive slots at periodicity of 20 ms

- SIB1: 48 RBs, 8 consecutive slots at periodicity of 20 ms, multiplexing pattern 1 with SSB

- Traffic: Video traffic with DRX (cycle, on-duration, inactivity timer) = (160, 8, 100) ms

  - Note: Video traffic considered since CA is typically for scenarios with higher data activity

- RU: Light load (15% - 30%)

In the following table, there show the network energy saving gains for Cat 1 and Cat 2 types of BS’s, and we have:

Observation 3: For CA use cases with higher data activity, disabling SSB and/or SIB1 for SCell achieves very limited energy saving gains, i.e., <8% for Cat 1 BS and < 1% for Cat 2 BS.

| NW load condition | Technique | NW energy saving gain | Evaluation Settings |
| --- | --- | --- | --- |
| Light (15% - 30% RU) | SSB/SIB1-less SCell | Cat1: 8% Cat2: 1% | Traffic: Video Baseline: 100M 2CCs with SSB and SIB1 in both CCs Improved: 100M 2CCs with SSB and SIB1 disabled in Scell |

Proposal 1: Disabling SSB and/or SIB1 for SCell is NOT pursued for network energy saving.

Conclusion

By the above evaluations and discussion, the following table summarizes our recommendation for NWES scheme – SSB/SIB1-less technique for further specification in work phase:

| NW load condition | Technique | NW energy saving gain | Evaluation Settings |
| --- | --- | --- | --- |
| Not recommended for NWES work phase |  |  |  |
| Light (15% - 30% RU) | SSB/SIB1-less SCell | Cat1: 8% Cat2: 1% | Traffic: Video Baseline: 100M 2CCs with SSB and SIB1 in both CCs Improved: 100M 2CCs with SSB and SIB1 disabled in Scell |

Proposal 1: Disabling SSB and/or SIB1 for SCell is NOT pursued for network energy saving.

Reference

RP-213554, 	“New SI: Study on network energy savings for NR”, RAN#94-e, Rapporteur: Huawei

RP-221443,	“SID revision: Study on network energy savings for NR”, RAN#96-e, Rapporteur: Huawei

RP-221442,	“Status report for WI: Study on network energy savings for NR”, RAN#96-e, Rapporteur: Huawei

RP-222546,	“Status report for SI Study on network energy savings for NR”, RAN#97-e, Rapporteur: Huawei

“Draft minimutes RAN1#110-bis-e”, online available @ https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_110b-e/Report