3GPP TSG-RAN WG2 Meeting #120	R2-2213105

Toulouse, November 14-18, 2022

Title:	[Draft] Reply LS on resource efficiency for MBS reception in RAN sharing scenario

Response to:	R2-2211168/R3-226084

Release:	Rel-18

Work Item:	NR_MBS_enh

Source:	CATT (to be RAN2)

To:	RAN3

Cc:	SA2

Contact person:	Erlin Zeng

erlin.zeng@CATT.CN

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachment: 	none

1 Overall description

RAN2 thanks RAN3 for their LS R3-226084 on resource efficiency for MBS reception in RAN sharing scenario. RAN2 discussed the questions therein and would like to provide the following feedback.

Question (1): Does RRC support in Rel-17 configuration of an MBS broadcast session, which is associated with multiple TMGIs?

Answer to Question (1): In the RRC ASN.1, the PTM configuration (e.g. g-RNTI/MRB list/mtch-SchedulingInfo/pdsch-Config, etc.) is configured per TMGI value, for a list of TMGIs. So, it is supported that multiple TMGIs are provided with the same PTM configuration.

Question (2):  RRC supports the indication whether a neighbour cell provides the broadcast service on MTCH. Given the size of the mbs-NeighbourCellList to which the mtch-NeighbourCell in each MBS-SessionInfo item refers to, it is possible that not all neighbour cells can be indicated. This size-limitation would presumably be more acute in RAN sharing scenarios, at the border between a shared area and a non-shared area or similar. Can it be assumed that service continuity is also supported towards a neighbour cell not indicated in the mbs-NeighbourCellList?

Answer to Question (2): It is possible to support service continuity towards a neighbour cell not indicated in the mbs-NeighbourCellList. From RAN2 point of view, the mbs-NeighbourCellList is used to enhance service continuity of MBS broadcast.

Question (3): Is there any significant limitation from RRC point of view if the TMGI as received by the 5GC contains a PLMN/SNPN ID not broadcast in SIB1?

Answer to Question (3): From RRC point of view there is no restriction that the TMGIs for the broadcast services that UE is interested to receive or is receiving should contain PLMN ID broadcasted in SIB1. RAN2 has agreed that MBS should be supported within SNPN in Rel-17, and it is FFS whether some specification change is needed (e.g., changes related to TMGI and SNPN ID).

2 Actions

To RAN3:

ACTION: 	RAN2 kindly asks RAN3 to take the above answers into account.

3 Dates of next RAN2 meetings

TSG RAN WG2 Meeting #121			27 February – 3 March 2023		Athens GR

TSG RAN WG2 Meeting #121bis-e		17 – 26 	April		   	            Online