3GPP TSG RAN WG2#120	R2-2213106

# Toulouse, 14th – 18th November, 2022

Title:	Draft LS on SPS configuration for unicast and multicast

Response to:	-

Release:	Release 17

Work Item:	NR_MBS-Core

Source:	ASUSTeK [to be RAN2]

To:	RAN1

Cc:	-

Contact Person:

Name:	Xinra Kung

E-mail Address:	Xinra_Kung@asus.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 discussed unicast and multicast SPS configuration and made following agreements for Rel-17:

Check with RAN1 whether NW can configure SPS in one BWP using sps-Config and sps-ConfigMulticastToAddModList-r17 simultaneously. Check with RAN1 whether other indexes than 0 can be used if a single DL SPS configuration for multicast is configured.

In the description for sps-Config (IE for single unicast SPS) in the current RRC spec, there’s a restriction that NW cannot configure SPS in one BWP using sps-Config and sps-ConfigToAddModList (IE for one or multiple unicast SPS) simultaneously:

sps-Config UE specific SPS (Semi-Persistent Scheduling) configuration for one BWP. Except for reconfiguration with sync, the NW does not reconfigure sps-Config when there is an active configured downlink assignment (see TS 38.321 [3]). However, the NW may release the sps-Config at any time. Network can only configure SPS in one BWP using either this field or sps-ConfigToAddModList.

Q1: RAN2 would like to ask RAN1’s view on whether similar restriction is required when configuring SPS for both unicast and multicast in one BWP, i.e., whether network cannot use sps-Config to configure unicast SPS and simultaneously use sps-ConfigMulticastToAddModList-r17 to configure multicast in one BWP.

Q2: Another question RAN2 would like to ask RAN1 is that when a single multicast SPS is configured for a group of UEs and there is no SPS configuration for unicast for at least one UE in the group, whether sps-ConfigIndex other than 0 can be configured by network for the single multicast SPS.

2. Actions:

To RAN1:

ACTION: 	RAN2 respectfully requests RAN1 to provide feedback to the above two questions.

3. Date of Next RAN2 Meetings:

RAN2#121	February 27th – March 3rd, 2023 	Athens, GR

RAN2#121-bis-e	April 17th – 26th, 2023		Online