3GPP TSG-RAN WG2 Meeting #121	R2-2300496

Athens, Greece, 27th February – 3rd March, 2023

Agenda Item:	8.17.2

Source:	Huawei, HiSilicon

Title:	Applicable scenarios for R18 MUSIM

Document for: Discussion

# Introduction

RAN2 discussed the applicable scenarios for R18 MUSIM in RAN2-119bis-e [1] and made the following agreement:

RAN2 aims to address at least the Scenario 1: the UE in network A in RRC_CONNECTED indicates (i.e. adds/removes) its preference on temporary UE capability due start/stop connection in NW B. This can be e.g. CA/DC capability restriction.

We discuss one additional scenario that is applicable for R18 MUSIM and also present our view on R18 MUSIM UE supporting R17 MUSIM solutions/features.

# Discussion

## Scenarios

The objective 1 of the R18 MUSIM revised WID [2] is:

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4]. - Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose - RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR. - Applicable UE architecture: Dual-RX/Dual-TX UE The work item shall identify whether the WI will have RAN3 or RAN4 impacts by RAN#99 [RAN2].

From the statement “enhancements for MUSIM procedures to operate the UE in RRC_CONNECTED state simultaneously in NW A and NW B” of the objective 1, it is clear that R18 MUSIM would only consider the scenarios where the UE is in RRC_CONNECTED state in both NW A and NW B.

Observation: R18 MUSIM WI only considers the scenarios where the UE is in RRC_CONNECTED in both NW A and NW B.

In addition to the agreed scenario (as described in Section 1 above), RAN2 also needs to consider the scenario in which SIM A and SIM B are both in RRC_CONNECTED. If NW B changes configuration (e.g. measurement or MIMO configuration), then UE may need to update its capability in NW A.

Figure 1: SIM A and SIM B are both in RRC_CONNECTED

Proposal 1: RAN2 to consider the following scenario for R18 MUSIM WI:

- SIM A and SIM B are in RRC_CONNECTED state and the configuration of SIM B is updated

## On supporting R17 MUSIM features

R17 MUSIM WI [3] addressed the UEs with Single-Rx/Dual-Rx/Single-Tx architecture and proposed solutions for the following issues:

- Paging Collision

- Co-ordinated Network Switching without leaving RRC_CONNECTED

- Co-ordinated Network Switching with leaving RRC_CONNECTED

- Paging with service indication for voice

R18 MUSIM UE supports dual RRC Connections and can address all the issues solved by R17 MUSIM. Hence R18 MUSIM UE does not need to support R17 MUSIM solutions/features.

Proposal 2: R18 MUSIM UE does not need to support R17 MUSIM solutions/features.

# Conclusion

In this contribution, we discussed the applicable scenarios for R18 MUSIM WI and proposed the following:

Observation: R18 MUSIM WI only considers the scenarios where the UE is in RRC_CONNECTED in both NW A and NW B.

Proposal 1: RAN2 to consider the following scenario for R18 MUSIM WI:

- SIM A and SIM B are in RRC_CONNECTED state and the configuration of SIM B is updated

Proposal 2: R18 MUSIM UE does not need to support R17 MUSIM solutions/features.

# References

- RAN2 Chair’s Notes RAN2#119bis-e, RAN2#119bis-e, e-Meeting Oct, 2022.

- 3GPP RP-223492, “Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR,” Vivo, RAN#98e, Electronic Meeting, December 12 – 16, 2022.

- 3GPP RP-201309, “WID: Support for Multi-SIM devices for LTE/NR”, RAN#88e, June 29 – July 3, 2020