3GPP TSG-RAN WG2 #121	R2-2301509

Athens, Greece, 27 February – 3 March 2023

Agenda Item:	8.5.2.3

Source:	Ericsson

Title:	Discussion on PDU Discard

Document for:	Discussion, Decision

# Introduction

In [1], a new Study Item identifying three main areas of study, i.e., XR-Awareness in RAN, XR specific power savings and XR specific capacity improvements, was agreed.

“Objectives on XR-awareness in RAN (RAN2):

- Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of.

- Study how the above information aids XR-specific traffic handling.”

In RAN2#120 the following agreements related to the study topic of PDU Discard was made

- RAN2 to support timer-based discarding of UL transmit side of PDCP PDU/SDUs of a PDU set. FFS how this is modelled in PDCP specification, can be discussed in WI phase.

In this contribution we share our view on PDU Discard.

# 2	Discussion on PDU Discard

## 2.1	Motivation for discarding

Discarding has been discussed as a method to improve performance for XR services. The motivation for discarding can be categorized into two areas:

- Discarding to improve transmission of other packets from the user;

- Discarding with the aim to reduce network load.

The second approach may be questioned for many reasons. First, it has been stated by SA4 [2] that all packet losses will impact the user experience and the goal should thus always be to deliver all packets. Discarding goes strictly against this. Secondly, looking at the scenarios where packet discarding would be an option for a given user, it is obvious that such takes place when the user has problems with the data transmission rate. Thus, if packets are discarded in favour of new packets entering the buffer for the same user, this will not improve the transmission probability for the new packets unless the data rate also improves. All in all, the actual discarding doesn’t likely help to meet the requirements for this user. This should thus not be the goal of discarding solutions.

Discarding packets from a user likely doesn’t improve that users experience.

The first approach with the goal of reducing the network load is not trying to improve the experience for the user that is subject to discarding, but instead can be beneficial for other users in the system. It has been shown in simulations [3] that the XR capacity is limited and it is easy to overload a system with XR users. Furthermore, a bad performing XR user can consume a lot of resources and impact all other users in the system, but still be unsatisfied. Removing the load caused by such a poor performing XR user should be the goal of the discarding solutions. There are however multiple options for how the discarding could be done to reduce network load and improve the overall network XR capacity. The rest of this paper analyses the different options of PDU Set discarding with the goal of reducing network load.

Discarding can help reduce network load and improve network XR capacity.

## 2.2	Flavours of PDU discarding

### 2.2.1	Timer based Discarding

One solution that has been discussed in the Rel-18 SI is to define a discard timer connected to the PDU Set. The goal of this solution is to discard all packets belonging to the PDU Set, when the discard timer expires. In this case, the timer could start when the first IP packet is received in the buffer and, when/if it expires, all IP packets associated to the PDU set would be discarded. This is the most basic discarding mechanism and needed to avoid any build-up of useless packets in buffer; however, this simplicity may lead to low gains for the network. If the timer expires and only a small fraction of the packets remains to be transmitted, dropping them will not lead to any noticeable network load reduction and thus no capacity increase. In other words, the later the timer expires, the less capacity gains are obtained. If dropping is done, the decision should be taken as early as possible, preferably even before the transmission of the data starts.

Timer based solution is the most basic but still needed discarding mechanism.

The later the dropping decision is taken, the lower the capacity increase is.

### 2.2.2	Proactive Discarding

Another mechanism to decide PDU set discarding is to evaluate the discarding decision based on the bit rate, the PDU set size, and packet delay budget or latency left (PDB – queued time). For UL the network can evaluate, using BSR information and timing indications (if provided, or else estimated), whether it can provide grants to complete the transmission within the delay budget. When that it not the case, the NW can indicate to the UE to drop the PDU set. It could be argued that the PDU set size, packet delay budget (or latency left), and statistical bit rate can also be known at the UE. Additional configuration parameters could be provided to the UE. Based on this the UE could decide when to drop a PDU set. The main drawback from this UE-based solution is that the UE buffer size would be changing without the NW knowing about it, which could cause the network to issue larger grants than needed by the UE. This could negatively affect the network capacity.

Proactive discarding can be done by using information such as the PDU Set size and delay budget left.

### 2.2.3	Evaluations of discarding solutions

Figure 1 shows that the proactive discarding is far better than using a timer solution. The green line exemplifies the proactive discarding mechanism, while the red line exemplifies the timer discarding solution where timers are configured according to the PSDB values. It is shown that the proactive discarding increases capacity, with good and early decisions it can provide a capacity increase of 10%.

Figure 1 Satisfied fraction of UEs when two different PDU Set dropping mechanisms are used: dropping based on timers or prediction.

RAN2 should specify mechanism for the UE and signalling for NW to support PDU Set dropping solutions assuming application awareness, e.g. information about the PDU Set size and the PDU Set delay budget

UL PDU Set dropping mechanisms should be designed so that they are either 1) the NW indicates the UE to drop UL PDU sets, or 2) the NW configures the UE with adequate rules and thresholds.

### 2.2.4	FEC based discarding

NOTE 2:	In case of Forward Error Coding (FEC), active discarding of PDUs when assuming that a large enough number of packets have already been transmitted for FEC to recover without the remaining PDUs is not recommended as it might trigger an increase of FEC packets (see S4aV220921 [14]).

FEC based discarding is not recommended by SA4. This is already captured in the TR 38.835 [4] with the following note:

Thus, FEC based discarding need not be considered for the WID.

FEC based discarding is not recommended by SA4.

FEC based discarding is not considered in the WID.

### 2.2.5	PDU Set Integrated Indication

Related to FEC based discarding is the PDU Set Integrated Indication (PSII) which indicates if all data in the PDU Set is needed to be delivered for it to be useful for the application. It has been proposed that such indication could be used to discard in advance data that may not be useful for the application. However discarding based on the prediction that some data may not be needed by the application is already ruled out by same reasoning as for FEC based discarding. It has also been suggested that the PSII should be used in other ways than predictive discarding. The PSII would in such scenario tell if discarding should happen based on the identification of lost packets, i.e. deliver only the packets until the first lost packet is seen and discard the rest of the packets belonging to the same PDU Set. However, such approach will not reach the goal of reducing any network load. Since such loss can only be known with a delay, after all the retransmissions occurred, most of the data in the buffer will already be transmitted to the HARQ process. Even not considering higher layer retransmissions (which would only apply for very long PSDB) the lower layer retransmissions (HARQ retransmissions) will take up a significant time. HARQ retransmissions are still essential for good performance of XR services. With the large data rates that XR services consumes, transmission errors are inevitable, but they don’t imply that the packets don’t meet the requirements. On the contrary, HARQ operation assures that most PDU Sets are delivered without losses.

Solutions using PSII will not lead to reduction in network load.

PSII is not considered in the WID.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	Discarding packets from a user likely doesn’t improve that users experience.

Observation 2	Discarding can help reduce network load and improve network XR capacity.

Observation 3	Timer based solution is the most basic but still needed discarding mechanism.

Observation 4	The later the dropping decision is taken, the lower the capacity increase is.

Observation 5	Proactive discarding can be done by using information such as the PDU Set size and delay budget left.

Observation 6	FEC based discarding is not recommended by SA4.

Observation 7	Solutions using PSII will not lead to reduction in network load.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should specify mechanism for the UE and signalling for NW to support PDU Set dropping solutions assuming application awareness, e.g. information about the PDU Set size and the PDU Set delay budget

Proposal 2	UL PDU Set dropping mechanisms should be designed so that they are either 1) the NW indicates the UE to drop UL PDU sets, or 2) the NW configures the UE with adequate rules and thresholds.

FEC based discarding is not recommended by SA4. This is already captured in the TR 38.835 [4] with the following note:

Proposal 3	FEC based discarding is not considered in the WID.

Proposal 4	PSII is not considered in the WID.

# 4 References

RP-213587, Study on XR Enhancements for NR, Nokia, RAN#94e, December 2021.

S4-220492, “LS Reply on QoS support with PDU Set granularity” SA4 Meeting #118E e-meeting, April 2022

3GPP TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17)”, V17.0.0, Dec. 2021.

3GPP TR 38.835v1.0.0(2022-12), NR; Study on XR enhancements for NR, (Release 18)