3GPP TSG RAN WG2 Meeting #121                                                       R2-2301596

# Athens, Greece, February 27th- March 3rd, 2023

Agenda item:		8.9.3

Source:			Intel Corporation

Title:		Service continuity enhancements for L2 U2N relay

Document for:	 	Discussion

# Introduction

In the last two meetings in RAN2 (#119bis-e, #120), the topic of service continuity was only briefly discussed and following agreements were made.

RAN2#120 agreement - RAN2 will investigate whether providing lossless delivery in DL and UL in the inter-gNB service continuity cases is feasible using Rel-17 mechanisms. RAN2 #119bis-e agreements - For i2i path switch procedure, introduce a new measurement event based on individual thresholds i.e., Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2. FFS if we also have an event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE, and in this case if/how to compare SL-RSRP of serving U2N relay UE and SD-RSRP of candidate U2N relay UE. - For i2i scenario, re-use the SL-RSRP or SD-RSRP measurement quantities for path switching. - For i2i scenario, serving/candidate U2N relay UEs, when SL-RSRP is unavailable, SD-RSRP is used as the measurement quantity. Wording can be revisited if it is determined that L2IDs for U2U and U2N are always different (so that candidate U2N relay UEs would never have SL-RSRP available). - For i2d path switch scenario, re-use the existing T304 timer. - For d2i and i2i path switch scenarios, re-use the existing T420 timer.

In the above-quoted agreements, d2i refers to direct-to-indirect and i2i refers to indirect-to-indirect path switching. In this document, we discuss RAN2 aspects related to inter-gNB path switching and indirect-to-indirect intra-gNB path switching, i.e support for scenarios A-D for service continuity in Release 18, while considering the agreements and open aspects from previous RAN2 meetings.

# Discussion

## Decision of Path Switch Trigger and Path Type

In the last two RAN3 meetings (#117e, #118), open aspects on path switching were discussed for inter-gNB scenario. It has been agreed in RAN3 that “Source gNB decides to trigger path switching for the L2N remote UE” , and “Source gNB selects the target path type (direct or indirect)”. In the meeting discussion in RAN2, there didn’t seem to be any opposing view to these agreements made in RAN3, and RAN2 can therefore follow same principles for the path switch procedure as agreed in RAN3.

- Aligned with RAN3, RAN2 to agree that source gNB or serving gNB of the Remote UE triggers path switching and also decides the path switching type (i.e direct or indirect path) for the L2 U2N Remote UE.

## Inter-gNB indirect-to-direct path switching

For service continuity of L2 UE-to-Network relay, the baseline procedure in the case of remote UE switching from an indirect L2 U2N relay path to direct Uu cell is included in the Annex-A. This is referred to as Scenario A in the Release 18 WID, shown in Figure 1 below.

Figure 1: Scenario A - Inter-gNB indirect-to-direct path switching

As agreed in the last RAN2 meeting, intra-gNB indirect-to-direct path switching can be used as baseline. To support this scenario, which is similar to traditional handover, we think that no further impact to RAN2 signalling is needed.

No specific difference with respect to service continuity on RAN2 Uu interface signalling is observed between inter-gNB and intra-gNB path switching for indirect to direct communication link.

### Lossless Path Switch

Based on RAN3’s email discussion on service continuity during RAN3#117 [R3-225084], several companies agree that there is no further stage-3 signalling needed to support scenario A, however it was raised that if PC5 RLF occurs during the path switching process, there can be some DL data loss towards the Remote UE (i.e. the source gNB may not forward packets that are not acknowledged by the Remote UE towards the target gNB). It was left for RAN2 to discuss about this case to determine a resolution and way forward.

While reestablishment and recovery is already supported in PDCP layer, we do however realize that there can be some corner cases where data loss may occur if the R17 mechanism for U2N Relaying is used. In one example for DL scenario, relay UE receives and acknowledges delivery of PDUs on RLC AM bearer from the source gNB. The source gNB decides to perform path switching to direct path following measurement event X1 and sends RRCReconfiguration message to Remote UE, upon which Remote UE stops UP and CP transmission via the L2 U2N Relay UE. During this time, if PC5 RLF occurs, data loss could occur. This is because the RLC is terminated at each hop. The source gNB may have received acknowledgement from lower layer for certain packets successfully received by the relay UE, however, upon PDCP re-establishment the Remote UE is not aware of these packets and they are consequently lost.

Even though the PDCP is terminated between the Remote UE and the gNB, and does not have per-hop termination as in the case of RLC layer, for PDCP data recovery (if performed by the remote UE per gNB configuration as in the case of intra-gNB path switching), the PDCP entity performs retransmission for packets for which the successful delivery has not been confirmed by lower (RLC) layers.

In the last RAN2 meeting, companies could not reach a consensus to rely only on R17 mechanismsfor the case of inter-gNB path switching which is supported in R18. Considering the example of indirect to direct path switching, if we were to support lossless delivery over indirect path when PC5 RLF may occur during i2d path switching, some impacts that can be foreseen include (i) informing relay UE of remote UE’s path switching, (ii) data/information buffering at the relay UE, or (iii) suspending PC5 link release until PDCP recovery process is complete. We believe it should be possible to reach a compromise by employing low impact enhancements to R17 mechanisms to support lossless delivery in R18 e.g remote UE could send a PDCP status report to the source gNB, before the source gNB performs SN status transfer to the target gNB.

- RAN2 can discuss two options for providing lossless delivery in DL and UL in the inter-gNB service continuity cases (i) scenarios where data loss could occur are considered as corner cases and no enhancements to Rel-17 mechanisms are needed, (ii) low impact measures for support of lossless delivery are adopted e.g Remote UE sends a PDCP status report to the source gNB, before the source gNB performs SN status transfer to the target gNB, or PC5 link is maintained until PDCP data recovery process is complete etc.

## Inter-gNB direct/indirect-to-indirect path switching

For service continuity of L2 U2N Remote UE, the baseline procedure in the case of L2 U2N Remote UE switching from direct Uu path to indirect path via a U2N Relay UE connected to another gNB is included in the Annex-B. This is referred to as Scenario B Release 18 WID, shown in Figure 2 below.

Figure 2: Scenario B - Inter-gNB direct-to-indirect path switching

The baseline procedure for indirect to indirect path switching, where the candidate relay UE may be selected from a different serving gNB as well, is included in Annex-D. This is referred to as Scenario D Release 18 WID, shown in Figure 3 below.

Figure 3: Scenario D – Inter-gNB indirect-to-indirect path switching

Since the Remote UE and target Relay UE are served by different gNB, handover request and acknowledgement messages need to be exchanged between source gNB and target gNB. RAN3 is planning to focus on the Xn and F1 impacts of these scenarios but currently they are still down-selecting between the source gNB and target gNB for the selection of direct/indirect path and then, which node selects the target Relay UE for indirect path.

RAN3 has agreed to update Handover Request/Acknowledgement messages over Xn for inter-gNB direct to indirect path switching and inter-gNB indirect to indirect path switching scenarios.

### Selection of Target Relay UE

In RAN2#120, the discussion on selection of target relay UE was not pursued in RAN2 with the understanding that RAN3 may reach a consensus in this regard. In the last RAN3 #118 meeting, however, this topic could not be concluded. RAN3 discussed three options in the email discussion in R3-226850, as listed below

- Option 1: source gNB selects one target Relay UE and sends the ID related information to the target gNB

- Option 2: source gNB sends a list of candidate target Relay UE information to the target gNB for selection

- Option 3: source gNB provides also the measurement information of Remote UE to the target gNB for selection of target Relay UE

The following agreements were made in RAN3 and some open aspects were identified

Agreements: Focus on the following two ways for the future discussion, - Way1: to go for Op1, and Op2 can be further discussed. - Way2: accept Op2, or at least as a compromise. No more discussion on Op3 in RAN3. Open issues: For Op2, continue discussion on following: - FFS on which node (source node or target node) decides the target cell in case of inter-gNB path switching Proponents of Option 2 should provide more details on the whole mechanism, e.g., - Whether source node can choose candidate relay UEs belonging to multiple target cells or can we restrict to candidate relays belonging to one target cell - Whether source node can choose candidate relay UEs belonging to multiple target gNBs or can we restrict to candidate relays belonging to one target gNB - Potential stage-3 impacts (e.g., number of candidate relays that needs to be signaled to target gNB)

A straightforward approach which has thus far been adopted in RAN2 with regards to this issue is to leave the decision on RAN3. However, the aspect of discussion regarding the RRC state of target Relay UE, lies more in RAN2 domain. It was agreed in RAN2 119e that “For inter-gNB d2i path switching and intra-/inter-gNB i2i path switching in Rel-18, the network can select a target U2N relay UE in any RRC state, i.e., RRC_CONNECTED/IDLE/INACTIVE.” [1]. The selection of Target Relay UE can have different implications when the Relay UE is in RRC_CONNECTED or if it is in RRC_IDLE/INACTIVE states. We therefore think that it is worthwhile to discuss this issue in parallel in RAN2. To this end, RAN2 can also consider only the two down-selected options in RAN3 for the discussion in RAN2. In the case of option 3, when the source gNB provides both the measurement results and the candidate relay UE IDs to the target gNB, this would imply that the bulk of measurement report is to be sent over Xn or using inter-node RRC signalling causing additional signalling overhead. Therefore if RAN3 has discounted option 3, it is prudent for RAN2 to also not consider option 3 for discussion.

Aspect of discussion regarding the RRC state of target Relay UE, lies more in RAN2 domain

- RAN2 discusses the open aspect of selection of target Relay UE by the source or target gNB in parallel to discussions in RAN3 on this topic.

  - If RAN2 discusses the topic of target Relay UE selection, only the two down-selected options by RAN3 are considered (i) Option 1 – source gNB makes decision and (ii) Option 2 – target gNB makes decision.

Below we discuss pros and cons of the two options i.e when source gNB makes selection versus when target gNB makes selection of target relay UE.

#### Source gNB makes selection (option 1)

It seems the main point of contention for Option 1 (source gNB to select target relay UE) is that the source gNB is not aware of the RRC state for the target relay UE, and that if the selected target Relay UE is in RRC IDLE or INACTIVE state, some companies claim that it may lead to higher HO latency. In our view, RAN2 has already agreed that the relay UE can be in any RRC state (as captured above), therefore whether the source gNB or the target gNB selects the target relay UE, there has been no agreement on prioritizing RRC_CONNECTED candidate relay UEs, or that relay UEs in IDLE or INACTIVE state would be precluded from the candidate relay UE list. And if such is the case, then the latency for state transition from RRC_IDLE/INACTIVE to RRC_CONNECTED state is incurred in both cases, whether the source gNB or the target gNB decides on the target relay UE.

RRC state cannot be a factor to decide target relay UE because it is already assumed that relay UE can be in any RRC state.

RAN3 in #117bis-e meeting made the following agreement: “For direct/indirect to indirect path switching, enhance Xn: HANDOVER REQUEST to include at least the Remote UE L2 ID and Relay UE L2 ID. FFS whether to include a single Target Relay L2 ID or a list of Target candidate Relay L2 IDs.”. Here the FFS is for the case when the target gNB selects the target relay UE. There was also a WA in RAN3 in the last meeting, that the serving cell of the relay UE can be signalled to the target gNB with no spec impact using the existing IE Target cell Global ID. If source gNB decides on the path switch type (direct/indirect), and then can also select the serving cell of the relay UE, the target gNB is only left to select the target relay UE (if at all).

Based on the discussion presented above, and similar to legacy where source gNB selects the target cell (and target gNB), we think it is appropriate for the source gNB to make the decision of target relay UE selection.

- For support of inter-gNB scenarios B/D of switching from a direct/indirect to an indirect link, the source gNB chooses the target Relay UE for the Remote UE. The Remote UE ID and the target Relay UE ID are forwarded to the target gNB e.g using an inter-node message or HANDOVER REQUEST message.

#### Target gNB selects target relay UE (Option 2)

For the case when the candidate relay UE is in RRC_IDLE state, since the UE is not registered, the target gNB will not have the AS context for that relay UE. If multiple selected candidate relay UEs in RRC_IDLE are shortlisted by the source gNB, and it is left up to the target gNB to select the target Relay UE. The target gNB can fetch AS context of multiple candidate relay UEs and make the selection based on Uu signal strength in the case of option 2 when the source gNB does not share the measurement information. This seems a lot of overhead and lag at the target gNB’s end. In this case, if the candidate Relay UEs are in RRC_IDLE, we don’t see much benefit for the target gNB to make this selection of target relay UE.

Considering the overhead/complexity of fetching AS context when candidate relay UE is in RRC IDLE, the target gNB would likely select one target relay before checking candidate Relay UE’s AS context. When the target relay UE is in RRC_IDLE state, the available information at target gNB is less than that at source gNB in Option 2, when the target relay UE is in RRC_IDLE state.

In Option 2/3, for the case when the candidate relay UE is in RRC_CONNECTED state, it is possible that the target gNB (which is the target Relay UE’s serving gNB) has a better idea of the Uu signal quality of the target Relay UE whereas if the source gNB were to make the decision, then it will be based on the Remote UE’s PC5 signal strength measurement report. However, this benefit does not seem to be significant enough because any relay UE identified as a candidate relay UE already meets certain Uu signal quality threshold even if source gNB is to make the selection. Additionally, if target relay UE is in RRC IDLE/INACTIVE state, the target gNB doesn’t initially know the Uu signal quality. Moreover, in the case of Option 2, target gNB doesn’t even know SL signal quality because source gNB doesn’t provide it. Furthermore, if target gNB is to select the target relay UE, then in the case where candidate relay UEs are served by different (more than one) target gNBs, this could lead to complicated decision of target relay UE and potential latency in the path switching process.

Using Uu signal quality at target gNB may not provide significant gain and it may cause more signaling overhead if target relay UE is in RRC_IDLE or RRC_INACTIVE mode.

## Intra-gNB indirect-to-indirect path switching

This is similar to the case of relay reselection but with gNB involvement, and the baseline procedure is included in Annex-C. This is referred to as Scenario C in the Release 18 WID, shown in Figure 4 below.

Figure 4: Scenario C - Intra-gNB indirect-to-indirect path switching

From RAN3 perspective, it was agreed in the RAN3 meeting #117e that “Current signaling can support Scenario C, i.e., intra-gNB indirect to indirect path switch.” since both Remote UE and Relay UE are served by the same gNB.

In RAN2 meeting #119bis-e, it was agreed to introduce a measurement event Z1:

- Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2

It was FFS whether another measurement event needs to be defined, as event Z2:

- Z2:  Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE

One of open issues of Z2 is if/how to compare SL-RSRP of serving U2N relay UE and SD-RSRP of candidate U2N relay UE because it was also decided to re-use SL-RSRP or SD-RSRP measurement quantities for path switching. For the case of event Z2, if the measurement result for the source and target relay UE is based on different reference signal, i.e SL-RSRP or SD-RSRP, it is not decided whether the offset term can be used when comparing these link quality measurements directly. Furthermore, a concern was raised that the L2IDs may be different between the serving relay UE’s communication message (based on which SL-RSRP is measured) and discovery message (based on which SD-RSRP is measured) and the remote UE can potentially not know that the relay UE is physically the same. To this end, RAN2 sent an LS to SA2 on L2IDs for U2U/U2N and we can wait for SA2 response in this aspect before deciding whether or not to introduce measurement event Z2.

- RAN2 to wait for SA2 response to LS on L2IDs for U2U/U2N before deciding whether or not to introduce measurement event Z2.

# Conclusion

In this contribution we discuss aspects related to service continuity for inter-gNB path switching scenarios and for indirect-indirect intra-gNB path switching scenario, and make the following observations and proposals:

Observation 1.	No specific difference with respect to service continuity on RAN2 Uu interface signalling is observed between inter-gNB and intra-gNB path switching for indirect to direct communication link.

Observation 2.	RAN3 has agreed to update Handover Request/Acknowledgement messages over Xn for inter-gNB direct to indirect path switching and inter-gNB indirect to indirect path switching scenarios.

Observation 3.	Aspect of discussion regarding the RRC state of target Relay UE, lies more in RAN2 domain

Observation 4.	RRC state cannot be a factor to decide target relay UE because it is already assumed that relay UE can be in any RRC state.

Observation 5.	Considering the overhead/complexity of fetching AS context when candidate relay UE is in RRC IDLE, the target gNB would likely select one target relay before checking candidate Relay UE’s AS context. When the target relay UE is in RRC_IDLE state, the available information at target gNB is less than that at source gNB in Option 2, when the target relay UE is in RRC_IDLE state.

Observation 6.	Using Uu signal quality at target gNB may not provide significant gain and it may cause more signaling overhead if target relay UE is in RRC_IDLE or RRC_INACTIVE mode.

Proposal 1.	Aligned with RAN3, RAN2 to agree that source gNB or serving gNB of the Remote UE triggers path switching and also decides the path switching type (i.e direct or indirect path) for the L2 U2N Remote UE.

Proposal 2.	RAN2 can discuss two options for providing lossless delivery in DL and UL in the inter-gNB service continuity cases (i) scenarios where data loss could occur are considered as corner cases and no enhancements to Rel-17 mechanisms are needed, (ii) low impact measures for support of lossless delivery are adopted e.g Remote UE sends a PDCP status report to the source gNB, before the source gNB performs SN status transfer to the target gNB, or PC5 link is maintained until PDCP data recovery process is complete etc.

Proposal 3.	RAN2 discusses the open aspect of selection of target Relay UE by the source or target gNB in parallel to discussions in RAN3 on this topic.

Proposal 3.1.	If RAN2 discusses the topic of target Relay UE selection, only the two down-selected options by RAN3 are considered (i) Option 1 – source gNB makes decision and (ii) Option 2 – target gNB makes decision.

Proposal 4.	For support of inter-gNB scenarios B/D of switching from a direct/indirect to an indirect link, the source gNB chooses the target Relay UE for the Remote UE. The Remote UE ID and the target Relay UE ID are forwarded to the target gNB e.g using an inter-node message or HANDOVER REQUEST message.

# References

[1] R2-2209301, Report of 3GPP TSG RAN WG2 meeting #119-e, Online, August 2022.

[2] TS 38.300 NR and NG-RAN Overall description; Stage-2, V 15.11.0

# Annex

## Annex-A – Indirect to Direct Path Switching

Figure 5: Procedure for remote UE switching from indirect relay path to direct Uu cell for inter-gNB scenario

Step 1: Measurement configuration and reporting to evaluate relay and Uu link measurement. The measurement results from U2N Remote UE are reported when configured measurement reporting criteria are met. The sidelink relay measurement report shall include at least U2N Relay UE's source L2 ID, serving cell ID (i.e., NCGI), and sidelink measurement quantity information. The sidelink measurement quantity can be SL-RSRP of the serving U2N Relay UE, and if SL-RSRP is not available, SD-RSRP is used.

Step 2: The source gNB decides to switch the U2N Remote UE onto direct Uu path with target gNB.

Step 3/4: Handover request and acknowledgement between source gNB and target gNB. May require RAN3 involvement in case any changes are required since the source is the U2N relay UE.

Step 5: The source gNB sends RRCReconfiguration message to U2N remote UE. The U2N Remote UE stops UP and CP transmission via U2N Relay UE after reception of RRCReconfiguration message from the source gNB.

Step 6: Transfer the uplink/downlink PDCP SN and HFN status

Step 7: Remote UE performs Random Access to target gNB

Step 8/9: Deliver buffered data

Step 10: Remote UE feedback the RRCReconfigurationComplete to the target gNB via direct path, using the configuration provided in the RRCReconfiguration message. From this step, the UE (i.e U2N Remote UE in previous steps) uses the RRC connection via the direct path to the gNB.

Step 11: The source gNB sends RRCReconfiguration message to U2N relay UE to reconfigure the connection between the U2N relay UE and the gNB. The RRCReconfiguration message to the U2N Relay UE can be sent any time after step 5 based on gNB implementation.

Step 12: The PC5 link is released between remote UE and the relay UE. The timing to execute link release is up to UE implementation. The U2N Relay UE can execute PC5 connection reconfiguration to release PC5 Relay RLC channel after reception of RRCReconfiguration by source gNB or the U2N Remote UE can execute PC5 connection reconfiguration to release PC5 Relay RLC channel upon reception of RRCReconfiguration message from source gNB in Step 5.

Step 13: The data path is switched from the indirect path to the direct path.

NOTE:	Step 13 can be executed any time after step 7. Step 13 is independent of step 11 and step 12.

## Annex-B – Direct to Indirect Path Switching

Figure 6: Procedure for remote UE switching from direct Uu cell to indirect relay path for inter-gNB scenario

Step 1: Remote UE reports one or multiple candidate relay UE(s) and Uu measurements, after it measures/discovers the candidate U2N relay UE(s).

-	Remote UE may filter the appropriate relay UE(s) meeting higher layer criteria when reporting, in step 1.

-	The reporting can include at least the U2N Relay UE’s ID, U2N Relay UE’s serving cell ID, and sidelink measurement quantity information. The sidelink measurement quantity can be SL RSRP of the candidate U2N Relay UE and if SL-RSRP is not available, SD-RSRP is used..

Step 2: The source gNB decides to switch to a target relay UE associated with target gNB. The actual selection of the target relay UE in Step 2b can be done by the source gNB based on the measurement report from the Remote UE, or by the target gNB in which case the source gNB forwards the measurement results from the Remote UE to the target gNB as part of the Handover Request message in Step 3.

Step 3/4: Handover request and acknowledgement between source gNB and target gNB. May require RAN3 involvement in case any changes are required since the target is the relay UE.

Step 5: Transfer the uplink/downlink PDCP SN and HFN status

Step 6/7: Deliver buffered data

Step 8: Target gNB i.e the U2N Relay UE’s serving gNB sends RRCReconfiguration message to the target U2N Relay UE (like preparation), which can include at least Remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration. In case the selected U2N Relay UE is in RRC_IDLE or RRC_INACTIVE, after receiving the path switch command, the U2N Remote UE establishes a PC5 link with the U2N Relay UE and sends the RRCReconfigurationComplete message via the U2N Relay UE, which will trigger the U2N Relay UE to enter RRC_CONNECTED state. For the case when U2N Relay UE is in RRC_IDLE or RRC_INACTIVE, then Step 10 is performed before step 8.

Step 9: The source gNB sends RRCReconfiguration message to U2N Remote UE. The contents in the RRCReconfiguration message can include at least U2N Relay UE ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end radio bearer(s). This contains the handover information for the remote UE. While contention free RACH resources are generally allocated in the legacy Rel-15 HO procedure [2], however, for this case dedicated RACH resources are not required since the Remote UE is connected to the target gNB via the Relay UE only. The U2N Remote UE stops UP and CP transmission over Uu after reception of RRCReconfiguration message from the gNB.

Step 10: U2N Remote UE establishes PC5 connection with target U2N Relay UE, if the connection has not been setup yet.

Step 11: The U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete to the target gNB via the U2N Relay UE, using the target configuration provided in RRCReconfiguration in Step 8.

Step 12: The data path is switched from direct path to indirect path between the U2N Remote UE and the target gNB.

## Annex-C – Intra-gNB Indirect to Indirect Path Switching

Figure 7: Procedure for indirect-to-indirect relay path switching for intra-gNB scenario

Step 1:  Measurement configuration and reporting procedures are performed to evaluate relay link measurement. The measurement results from U2N Remote UE are reported when configured measurement reporting criteria are met. The sidelink relay measurement report shall include at least U2N Relay UE's source L2 ID, serving cell ID, and sidelink measurement quantity information. The sidelink measurement quantity can be SL-RSRP of the serving U2N Relay UE, and if SL-RSRP is not available, SD-RSRP is used.

Step 2/3: The gNB decides to switch the U2N Remote UE to a target U2N Relay UE B. Then the gNB sends an RRCReconfiguration message to the target U2N Relay UE B, which can include at least Remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

Step 4: The source gNB sends RRCReconfiguration message to U2N Remote UE. The contents in the RRCReconfiguration message can include at least U2N Relay UE ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end radio bearer(s). This contains the handover information for the remote UE.

Step 5: The U2N Remote UE establishes PC5 connection with target U2N Relay UE (if it does not already exist).

Step 6: It is up to U2N Remote UE implementation whether to release or keep the unicast PC5 link. The PC5 link is released between remote UE and the relay UE. The timing to execute link release is up to UE implementation. The source U2N Relay UE A can execute PC5 connection reconfiguration to release PC5 Relay RLC channel after reception of RRCReconfiguration from gNB. or the U2N Remote UE can execute PC5 connection reconfiguration to release PC5 Relay RLC channel upon reception of RRCReconfiguration message from gNB in Step 4.

Step 7: The U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the Relay UE B.

Step 8: The data path is switched from indirect path via Relay UE A to indirect path via Relay UE B.

## Annex D – Inter-gNB indirect to indirect path switching

Figure 8: Procedure for indirect-to-indirect relay path switching for inter-gNB scenario

Step 1: Measurement configuration and reporting procedures are performed to evaluate relay link measurement. The measurement results from U2N Remote UE are reported when configured measurement reporting criteria are met. The sidelink relay measurement report shall include at least U2N Relay UE's source L2 ID, serving cell ID, and sidelink measurement quantity information. The sidelink measurement quantity can be SL-RSRP of the serving U2N Relay UE, and if SL-RSRP is not available, SD-RSRP is used.

Step 2: The source gNB decides to switch to a target relay UE. The actual selection of the target relay UE in Step 2b can be done by the source gNB based on the measurement report from the Remote UE, or by the target gNB in which case the source gNB forwards the measurement results from the Remote UE to the target gNB as part of the Handover Request message in Step 3.

Step 3/4: Handover request and acknowledgement between source gNB and target gNB. May require RAN3 involvement in case any changes are required since the target is the relay UE.

Step 5: The target gNB (serving gNB for U2N Relay UE B) sends an RRCReconfiguration message to the target U2N Relay UE B (like preparation), which can include at least Remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

Step 6: The target gNB Y sends RRCReconfiguration message to U2N Remote UE via source Relay UE A. The contents in the RRCReconfiguration message can include at least U2N Relay UE ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end radio bearer(s). This contains the handover information for the remote UE.

Step 7: The U2N Remote UE establishes PC5 connection with target U2N Relay UE (if it does not already exist).

Step 8: It is up to U2N Remote UE implementation whether to release or keep the unicast PC5 link. If U2N Remote UE decides to release the unicast PC5 link, it triggers the L2 release procedure.

Step 9: The U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the target gNB Y via the Relay UE B.

Step 10: The data path is switched from indirect path via Relay UE A to indirect path via Relay UE B.