3GPP TSG-RAN WG2 Meeting #121	R2-2301694

# Athens, Greece, 27th Feb – 3rd Mar, 2023

Agenda Item:	6.8.2

Source:	Ericsson

Title:	Initial BWP for RedCap UEs configured with RA and CG-SDT

Document for:	Discussion, Decision

# 1	Introduction

In the previous meeting, RAN2 discussed a scenario where a RedCap UE is to perform CG-SDT in a RedCap-specific BWP that is not associated with any SSB comes with unclarity on what to expect.

Without clarification, it may be so that the RedCap UE must switch to initial BWP associated with CD-SSB for evaluation before CG transmission, and then switch back to the RedCap-specific initial BWP for PDCCH monitoring as search space is configured in the RedCap-specific initial DL BWP. This may occur as often as for every new transmission using CG-SDT. The basis for this understanding, i.e., SSB evaluation after confirmation of an initial transmission of CCCH, can be found in 38.321, procedure in section 5.8.2:

---

For an uplink grant configured for configured grant Type 1 for CG-SDT on the selected uplink carrier as in clause 5.27, when CG-SDT is triggered and not terminated, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

…

1>	else if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

2>	if at least one SSB corresponding to the configured uplink grant with SS-RSRP above the cg-SDT-RSRP-ThresholdSSB is available:

…

3>	else if PDCCH addressed to C-RNTI has been received after the initial transmission of CG-SDT with CCCH message (i.e., subsequent new transmission for CG-SDT):

4>	if SS-RSRP of the SSB selected for the previous transmission for CG-SDT is above cg-SDT-RSRP-ThresholdSSB and this SSB is associated with this configured uplink grant:

5>	select this SSB.

4>	else if SS-RSRP of the SSB selected for the previous transmission for CG-SDT is not above cg-SDT-RSRP-ThresholdSSB:

5>	select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant.

3>	if SSB is selected above:

4>	indicate the SSB index to the lower layer;

4>	consider this configured uplink grant as valid.

…

In this contribution we discuss this scenario along with the case for RA-SDT, which is similar, and the options to address the problems that have been discussed in the previous meeting.

# 2	Discussion

## 2.1	Initial Downlink BWP

A RedCap UE uses the initial DL BWP at least during initial access. The first initial downlink (DL) BWP configured in MIB is CORESET #0 (which is known as MIB-configured initial DL BWP). UE can receive the configuration of initial DL BWP in SIB after receiving CORESET #0. Specifically, the BWP configuration, i.e., initialDownlinkBWP IE, provides information about the bandwidth and location of the initial DL BWP, subcarrier spacing, and cell-specific PDCCH and PDSCH parameters of the BWP. Using a separate SIB-configured initial DL BWP for RedCap can be beneficial for flexibility and offloading purposes.

## 2.2	Synchronization Signal Block (SSB)

During cell search a UE aims at acquiring time and frequency synchronization with a cell and to detect physical layer cell ID (PCI) of the cell. In NR, the synchronization signal block (SS block or SSB) consists of primary and secondary synchronization signals (PSS and SSS) and physical broadcast channel (PBCH). During initial cell search, a UE first aims at detecting PSS followed by SSS. Time and frequency synchronization as well as cell ID detection are done using PSS and SSS.

An SSB associated with essential system information, i.e., SIB1, that a UE is required to acquire for initial access in a cell is called cell-defining SSB (CD-SSB). For RedCap UEs in connected mode it is also possible to configure non cell defining SSBs (NCD-SSBs).

In connected mode, gNB configures “measurement objects” to inform the UE at which frequency position to search for SSBs. When an SSB is associated with an RMSI, the SSB is referred to as a Cell-Defining SSB (CD-SSB). A PCell is always associated to a CD-SSB located on the synchronization raster. If a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by a RedCap UE. Non-Cell-Defining SSB (NCD-SSB), which is not associated with CORESET#0, is mainly used for some RRM procedures e.g., performing measurements.

When in connected mode if a RedCap UE is configured, via RRC signaling, with a DL BWP that is not associated with CD-SSB (no CORESET#0), it is expected that the BWP either contains NCD-SSB for the serving cell or no SSB. For the latter case, measurement gaps are configured for the UE to retune to the BWP that contains an SSB.

A UE configured with CG-SDT needs to perform measurements using SSB for TA validation, SDT resource validity, synchronization, and RRM (serving and neighbor cell measurements). If the active BWP is not associated with SSB, UE needs to periodically retune to the BWP associated with CD-SSB and retune back after performing the measurements. Since PDCCH that schedules the paging message and provides CMAS/ ETWS indication in a short message is transmitted within the BWP associated with CD-SSB, UE would also need to periodically retune to that BWP. Note that operating in a BWP not associated with CD-SSB also means that CORESET#0 is not provided in this BWP.

NCD-SSSB is not supported for RedCap UEs in idle/inactive.

The following agreements were made in RAN2 with the intention to clarify the SDT framework:

- 1st agreement, SDT session: “For CG-SDT purpose, RAN2 has basic assumption that SSB will be configured in initial BWP with CG-SDT.   Notify RAN1”

- 2nd agreement, RedCap session: “RAN2 Assumption: For CG-SDT purpose, RAN2 has basic assumption that SSB will be configured in initial BWP with CG-SDT. For RedCap FFS if SSB refers to CD-SSB or any SSB”

Meanwhile, the corresponding discussion in RAN1 has concluded:

Conclusion:

- No issue is identified for RedCap UEs supporting RA-SDT to support initial (non-subsequent) RA-SDT transmission in a RedCap-specific separate initial BWP without CD-SSB.

Conclusion:

The following cases can be revisited in RAN1#112:

- Subsequent RA-SDT transmission in a RedCap-specific separate initial BWP without CD-SSB

- CG-SDT in a RedCap-specific separate initial BWP without any SSB

- CG-SDT in a RedCap-specific separate initial BWP without CD-SSB but with NCD-SSB

With the assumption below:

- If the UE is a RedCap UE and if RedCap-specific BWP in which UE performs CG-SDT doesn’t include any SSB it may be so that the RedCap UE must retune to default initial BWP for SSB evaluation before each subsequent CG transmission, and then switch back to the RedCap-specific initial BWP for PDCCH monitoring as search space is configured in the RedCap-specific initial DL BWP.

RAN2 discussed the following options in the last meeting to address the issue:

- Option 1: For CG-SDT purpose, RAN2 has basic assumption that CD-SSB will be configured in initial BWP with CG-SDT.

- Option 2: For CG-SDT purpose, RAN2 has basic assumption that NCD-SSB will be configured in initial BWP with CG-SDT.

- Option 3: Allow CG-SDT on an initial BWP without SSB and it is up to UE implementation how to handle this scenario (potentially with separate UE capability)

The following assumptions can be made based on the option above::

Option 1: Not applicable to this specific discussion for RedCap initial BWP not associated with CD-SSB w.r.t CG-SDT. It simply removes the possibility to configure a RedCap initial BWP not associated with CD-SSB, i.e., associated with NCD-SSB, or no SSB,with CG-SDT. This limits the options in supporting SDT for RedCap UEs.

Option 2: Allows the combination of RedCap initial BWP associated with either CD-SSB or NCD-SSB with CG-SDT. Support for this combination may need a separate UE capability as this would introduce support for UEs performing SDT through introducing support for NCD-SSB in inactive state, configured in for example IE NonCellDefiningSSB in the RRCRelease message, or alternatively, relying on that UE retains the connected mode configuration of the active BWP associated with NCD-SSB.

Option 3: Allows the combination of RedCap initial BWP associated with either CD-SSB, NCD-SSB, or no SSB with a CG-SDT configuration. This is based on the assumption that a UE implementation may decide on which premises channel (TA) measurement on the CD-SSB or NCD-SSB is valid prior to each CG-SDT transmission when evaluating SSB RSRP against a RSRP change threshold.

For Option 3, the basis for this being a realistic solution, especially for the case with no SSB, is the following:

In 38.133, Table 5.5.3-1 and Table 5.5.3-2 provide conditions for when RSRP measurements are to be considered valid. As a result, one can assume that for achieving the goal of the SDT threshold change, the validity norm stipulated here indicate that for SSB evaluation, frequent measurements are not always required, i.e., that for the particular scenario and for when the time between SDT transmissions are short, frequent retuning to measure on the CD-SSB is not required. For example, it is stated in 38.133: "Additionally, the UE shall not transmit in an CG-SDT occasion that occurs more than 640ms after T2." meaning that UE can consider an RSRP measurement valid and does not in all instances need to perform new measurements for the change threshold during that period (RSRP2 in 38.133 being e.g., the latest SS-RSRP measurement from the initial measurement).

To capture this in 38.321, a change in the procedure in section 5.8.2 may not be required as the determination of an SSB evaluation itself can still apply according to procedure, while a new measurement with retuning is not always required, i.e., this could be clarified with a “Note” pertaining to the particular scenario, perhaps even related to the RedCap specific configuration on an initial BWP wo CD-SSB:

Here is an example for the note:

NOTE x:	In SS-RSRP evaluation for cg-SDT-RSRP-ThresholdSSB validation associated with a configured uplink grant in a BWP not associated with SSB, it is up to UE implementation to determine the validity of a SS-RSRP measurement on CD-SSB given by conditions in Table 5.5.3-1 and Table 5.5.3-2 in 38.133[11]

For RedCap UEs with CG-SDT, it should be possible.to configure the initial BWP associated with any/no SSB.

It up to UE implementation to determine the validity of RSRP change conditions based on existing criteria on an initial BWP not associated with CD-SSB.

A BWP not associated with CD-SSB does not include CORESET#0. PDCCH transmission that indicates CMAS and ETWS via short messages and schedules the paging message is performed using the BWP associated with CD-SSB (and includes CORESET#0). For any of the options discussed above, UE needs to periodically retune to BWP that includes CORESET#0 to monitor paging. However, it should be discussed what applies for RedCap specifically considering also the above discussion.

In 38.300, section 9.2.5, the following text is captured:

“While in RRC_CONNECTED and while in RRC_INACTIVE with ongoing SDT procedure, the UE monitors the paging channels in any PO signaled in system information for SI change indication and PWS notification.”

A UE with ongoing SDT procedure can intermittently retune to a BWP associated with CD-SSB (and thus includes CORESET#0) to monitor for paging.

The maximum value of timer T319a associated with the time that UE is required to monitor PDCCH is 4 seconds. The maximum possible value for the modification cycle is 40.96 seconds. If the modification period in the serving cell is configured with a value larger than the value T319a timer is configured with, it should be possible for the UE to check whether there exists a short message indicated by the network during that particular modification period, i.e., the UE should not simply rely on receiving such information via dedicated signaling. On the other hand, one needs to consider also the case for ETWS/CMAS since a RedCap UE needs to know and switch to a BWP associated with CD-SSB to acquire the updated SIB 6/7/8 if its active BWP is not associated with CD-SSB. In that case the network may need to provide such information to the UE via dedicated signaling.

It is up to network and UE implementation to ensure that a RedCap UE can receive/acquire system information update notifications and SI updates during an ongoing SDT procedure.

Even though only CG-SDT has been mentioned for the discussion above, it applies to the case for RA-SDT (after the initial transmission) which should be considered equally with respect to having no/any SSB (CD-SSB or NCD-SSB).

The scenario discussed for a RedCap UE that performs CG-SDT in a RedCap-specific BWP that is associated with any/no SSB should also be considered for a RedCap UE that performs RA-SDT.

# 3	Conclusions

In this contribution we have made the following observations:

Observation 1	A UE with ongoing SDT procedure can intermittently retune to a BWP associated with CD-SSB (and thus includes CORESET#0) to monitor for paging.

Observation 2	The scenario discussed for a RedCap UE that performs CG-SDT in a RedCap-specific BWP that is associated with any/no SSB should also be considered for a RedCap UE that performs RA-SDT.

Based on the discussion in the previous section we propose the following:

Proposal 1	For RedCap UEs with CG-SDT, it should be possible.to configure the initial BWP associated with any/no SSB.

Proposal 2	It up to UE implementation to determine the validity of RSRP change conditions based on existing criteria on an initial BWP not associated with CD-SSB.

Proposal 3	It is up to network and UE implementation to ensure that a RedCap UE can receive/acquire system information update notifications and SI updates during an ongoing SDT procedure.

# References

RP-221806, WID on IoT NTN enhancements, MediaTek, RAN#96, June 6-9, 2022

RP-220529, Additional enhancements for NB-IoT and LTE-MTC, Huawei, HiSilicon, RAN#95e, March 2022.

TS 36.331, Radio Resource Control (RRC) protocol specification, V17.1.0, June 2022.

TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), V16.1.0, June 2021.