3GPP TSG-RAN WG2 #121	R2-2302192

Athens, Greece, 27th February – 3rd March 2023

Agenda item:	6.7

Source:	Ericsson

Title:	[AT121][403][POS] Network control for MG activation/deactivation UL MAC CE

Document for:	Discussion and Agreement

# 1	Introduction

This document is to kick off the following email discussion:

[AT121][403][POS] Network control for MG activation/deactivation UL MAC CE (Ericsson)

Scope: Discuss the proposals in R2-2301303, R2-2301829, and R2-2301828 and conclude on the expected behaviour.

Intended outcome: Report and agreeable CR if necessary

Deadline: Wednesday 2023-03-01 1900 EET

# 2	Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |
| CATT | Jianxiang Li(lijianxiang@catt.cn) |
| Samsung | June Hwang (june77.hwang@samsung.com) |
| vivo | Xiang Pan (panxiang@vivo.com) |
| Lenovo | Hyung-Nam Choi (hchoi5@lenovo.com) |
| Xiaomi | Xiaolong Li (lixiaolong1@xiaomi.com) |
| LG | Jonggil Nam (jonggil.nam@lge.com) |
| Intel | Yi Guo (yi.guo@intel.com) |

# 3	Discussions

## 3.1	Motivation of CRs

- The feature UL MAC CE is optional for gNB

- UL MAC CE is implemented fully but not partially (i.e NW if implements would implement both the decoding and necessary action to act on the MAC CE)

- The MAC subheader associated with the content generated by MAC layer does not have an L field as opposed to data generated by upper layers which are indicated with the L field. Hence, there is a risk of packet discard when UE sends content which NW does not understand.

Do company agree to the RRC CR as provided in the RRC CR in R2-2301304 to provide the configuration for UL MAC CE?

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT | Yes |  |
| ZTE | No | As clearly stated at the session, there are 3 options on the table: - assume the network always supports it, (2) assume the network can decode the MAC PDU anyway, (3) the network may be unable to decode the MAC PDU and an RRC indication is needed. We suggest to go with (1) to avoid any misunderstanding and NBC change |
| Samsung | Yes | We think this is rather curing the broken principle not NBC case. |
| vivo | No | NBC concern is not addressed. If the UE is implemented according to the CR while the network is not, the UE can never send the UL MAC CE to request the pre-MG. If the gNB already spends a lot of effort to support the RRC configuration of pre-MG, it should be able to decode the UL MAC CE with less effort. |
| Lenovo | Yes with comment | In general, we don’t mandate the NW to support a certain functionality. So, if UL MAC CE is optional for the NW then it’s good for the UE to know whether it can send UL MAC CE or not. This would also avoid any interoperability issues. Some minor comments to the CR: - Cover page: “Clauses affected” is empty. - We prefer to change the value of posMG-Request-r17 from “allowed” to “enabled”. |
| Xiaomi | No | If UE supports UL MAC CE for pre-configured MG activation request but gNB doesn’t provide the configuration for UL MAC CE, LMF may not trigger the gNB to activate the preconfigured MG since the LMF assume UE will trigger it, thus the pre-configured MG doesn’t work. |
| LG | No | Agree with vivo. We think NBC issue is not resolved on CR. |
| ZTE2 | Yes | According to the further offline discussion with Ericsson, we tend to agree with the intention of adding this indication to make the function more clear. |
| Intel | Yes | We agree the issue raised by Erisson, i.e. the network may not identify UL MAC CE if not support the feature. |

Do company agree to the MAC CR as provided in R2-2301828?

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT | Yes with comments | #1. TS 38.331 CRxx is missed TS 37.355 CR #2. If the UE is configured with pre-configured measurement gap and the parameter posMG-Request in TS 38.331[5] is indicated as” allowed” |
| ZTE | No | Same as Q1 |
| Samsung | Yes with comments | As indicated by CATT, the sentence should be modified. |
| vivo | No |  |
| Lenovo | Yes | CATT’s suggestion looks ok but instead of “allowed” we prefer saying “enabled”. |
| Xiaomi | No |  |
| LG | No |  |
| ZTE2 | Yes |  |
| Intel | Yes |  |

# 4 Conclusion

- The NRPPa trigger and UE UL MAC CE Trigger for Pre-Configured MG feature are decoupled

- RRC Configuration includes configuration to allow UE to send UL MAC CE. Corresponding change is also reflected in MAC specification

- The LPP capability is split into DL capability and UE capability. A new UL capability is introduced.

RAN2 to agree to introduce NW configuration to enable transmission of UL MAC CE. The corresponding CRs for RRC R2-2302226 and MAC R2-2302231 are agreed.

The current one bit LPP capability to indicate support for both UL MAC CE and DL MAC CE are split so that a new capability is introduced to align with RRC Capability which has two separate capabilities.