3GPP TSG-RAN WG2 Meeting #121bis-e	R2-2302541

April 17-26, 2023

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3947 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC correction on BFD/RLM relaxation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2023-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN4 LS (R4-2220731/R2-2300055) , RAN4 has informed RAN2 that: - The release 17 relaxed RLM/BFD requirements do not apply for SCG deactivated cells. - RAN4 agrees to not define new relaxed RLM/BFD requirements for PSCell/SCell(s) in deactivated SCG. The existing RRC spec has however specified the case for relaxed RLM/BFD for PSCell in deactivated SCG for which RAN4 does not support. In RAN2#121, following agreements are made. Go option 2: Keep the current situation for RLM/BFD relaxation for SCG deactivation, i.e. no change in RAN2, no change in RAN4. RAN2 clarify that in the case of SCG deactivation and bfd-and-RLM is set to true, UE will perform the RLM/BFD according to the requirements for SCG deactivation of measCyclePSCell as specified in TS 38.133 no matter whether goodServingCellEvaluationBFD-r17 and goodServingCellEvaluationRLM-r17 is configured for SCG. This needs to be reflected in the RRC spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the clarification in RRC spec that “In the case of SCG deactivation and bfd-and-RLM is set to true, UE shall perform the RLM/BFD according to the requirements for SCG deactivation of measCyclePSCell as specified in TS 38.133 [14] no matter whether relaxed measurement criterion for good serving cell quality is configured for SCG.”. Impact analysis Impacted 5G architecture options: NR-DC, EN-DC Impacted functionality: RLM/BFD relaxation Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no relaxed RLM/BFD requirements for PSCell/SCell(s) in deactivated SCG. - If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC spec is not aligned with RAN4 spec as there is no relaxed RLM/BFD requirements for PSCell/SCell(s) in deactivated SCG. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.7.13	RLM/BFD relaxation

In case both low mobility criterion and good serving cell criterion are configured for RLM/BFD relaxation, the UE is allowed to perform RLM and/or BFD relaxation according to requirements specified in TS 38.133 [14] when both relaxed measurement criterion for low mobility and relaxed measurement criterion for good serving cell quality are met.

In case only the good serving cell quality criterion is configured for RLM/BFD relaxation, the UE is allowed to perform RLM and/or BFD relaxation according to requirements specified in TS 38.133 [14] when the relaxed measurement criterion for good serving cell quality is met.

In case SCG is deactived and bfd-and-RLM is set to true, UE shall perform the RLM/BFD according to the requirements for SCG deactivation of measCyclePSCell as specified in TS 38.133 [14] no matter whether relaxed measurement criterion for good serving cell quality is configured for SCG.