3GPP TSG-RAN WG2 #121bis	R2-2302575

E-meeting, April 2023

Agenda Item:	6.1.3.2

Source:	OPPO

Title:	Discussion on ue-PowerClassPerBandPerBC-r17

Document for:	Discussion, Decision

Introduction

This is to discuss the ue-PowerClassPerBandPerBC-r17.

Discussion

In the LS, there is one point

Response to 2): ue-PowerClassPerBandPerBC-r17 indicates the power class that a UE supports for each individual band within a given band combination, while powerClass or powerClass-v1610 indicates the power class for this band combination, in other words, the maximum total output power . If indicated, ue-PowerClassPerBandPerBC-r17 shall supersede other power class capabilities such as ue-PowerClass/powerClass and its extensions in determining the power class of the individual bands within a band combination.

It is not super clear how to interpret the word ‘supersede’. If it means that as long as ue-PowerClassPerBandPerBC-r17 is present, the other field (as mentioned above, including the per-BC value powerClass) can be ignored, it seems not correct.

Our understanding is that, e.g., for a BC including band-A and band-B

1/ the old per-BC powerClass restricts the ‘power-on-A + power-on-B’

2/ the new per-FS ue-PowerClassPerBandPerBC-r17 restricts the the ‘power-on-A’ and ‘power-on-B’ respectively

So both retrictions have to be satisfied.

- RAN2 confim the new ue-PowerClassPerBandPerBC and the legacy powerClass fields can be both present, and both restrictions should be satisfied. If RAN2 cannot converge on this, send an LS to R4 to confirm.

Conclusion

We have the following proposals:

Proposal 1	RAN2 confim the new ue-PowerClassPerBandPerBC and the legacy powerClass fields can be both present, and both restrictions should be satisfied. If RAN2 cannot converge on this, send an LS to R4 to confirm.

Reference

- R4-2303630, Reply LS on clarification for ue-PowerClassPerBandPerBC-r17 (R4 16-8).