3GPP TSG-RAN WG2 #121bis-e	R2-2302616

Electronic meeting, 17th – 26th April 2023

Source:	NTT DOCOMO, INC., Samsung, Huawei, HiSilicon

Title:	RAN2 work plan for MIMO evolution

Document for:	Discussion and decision

Agenda Item:	7.20.1

- Introduction

From this meeting, RAN2 is allocated with TUs for discussion on NR18 MIMO evolution [1].

This contribution summarizes open RAN2 issues both raised in the previous RAN2 meeting and described in the latest LS from RAN1 [2] and provides a tentative work plan to finish RAN2 works just for purpose of information.

- Open issues

  - Introduction of two-TA

In the previous RAN2 meeting, we generally discussed potential issues on introduction of two TAs, upon receiving LS from RAN1 [3]. For information, company proposals in their contributions for the previous meeting are listed below.

| TDoc # | Company | Proposals |
| --- | --- | --- |
| R2-2300338 | Sharp | Proposal 1: RAN2 discuss how the MAC entity consider a Serving Cell associated with two TAG to be uplink time aligned. Proposal 2: RAN2 discuss the impact to RLF detection when a Spcell is configured with multi-TRP. Proposal 3: RAN2 discuss the impact to existing procedure if CBRA triggered by PDCCH order is supported for Multi-TRP operation. |
| R2-2300907 | Ericsson | Proposal 1 RAN2 to discuss the feasibility of reusing existing TAGs to enable two TAs within one serving cell. Proposal 2 RAN2 to discuss whether to send LS to RAN1 from RAN2#121 to ask further clarification questions |
| R2-2301035 | Fujitsu | Two TAGs per serving cell Proposal 1: RAN2 discusses whether to support at most one or two Primary TAGs per MAC entity. Proposal 2: For multi-DCI multi-TRP operation with two TAs, RAN2 should discuss how to handle TA timer, considering: - Option 1: TA timer per TAG (i.e. existing mechanism) with some enhancements - Option 2: TA timer per serving cell Per TRP UE-initiated RACH procedure Proposal 3: For multi-DCI based Multi-TRP operation with two TAs, RAN2 is asked to consider the following enhancements to support per TRP UE-initiated RACH procedure: - possible TRP specific configuration, e.g. TRP specific DL RS group, TRP specific PRACH configuration, TRP specific thresholds for RA procedure initialization. |
| R2-2301291 | Nokia | Proposal 1: Discuss whether the existing TAG space (i.e., up to 4 TAGs) is sufficient for multi-DCI multi-TRP purpose. Proposal 2: Initial TA acquisition for the 2nd TAG is initiated by the network and performed through PDCCH order with CFRA procedure. Proposal 3: From RAN2 point of view, PDCCH order triggered CFRA for the reason of TA acquisition for the second TAG has to be known to the UE before/upon the RA procedure initiation. Indicate this to RAN1. Proposal 4: Discuss how the RACH configuration to be used for TA acquisition for the 2nd TAG is pre-provisioned to the UE in inter-cell scenario. Proposal 5: No new UE based trigger is defined to initiate RA procedure for acquiring timing for a TAG when at least one TAT at the UE is running (ie., same as with CA sTAG). |
| R2-2301791 | ZTE | Proposal 1. RAN2 Confirms there is no need to enhance CBRA procedure to support per TRP UE initiated RACH procedure for obtaining TA value of TRP. And send a reply LS with RAN2 understanding to RAN1. |

Issue on the multiple TA for mTRP has many RAN2 impacts and RAN2 has a decision role for the details on this issue, so RAN2 first consider this issue during the initial phase of this WI.

  - RRC issues

Please find Annex to see all RAN1 agreements with potential RAN2 impact (abstracted from RAN1 LS [2]).

Note that RAN1 plans to send an excel sheet of the parameter list to RAN2 after May meeting, according to RAN1 status report [4].

- To meet the functional freeze target date of 2023.Q3 for Rel-18, RAN1 will - Start review of draft CRs after RAN1#112bis-e (April) starting with select WIs (eDSS, NCR, MC-Enh) - And expand to all other Rel-18 WIs after RAN1#113 (May) - Send out first LS to RAN2 and RAN3 on Rel-18 higher layer signaling parameter after RAN1#113 (May) - Further refined parameter list to RAN2/RAN3 after RAN1#114 (August) - Send out first LS to RAN2 on Rel-18 UE features RAN1#113 (May) - Further refined UE feature list to RAN2 after RAN1#114 (August)

For MIMO Rel-16/17, RAN2 has progressed the MIMO WI with two main issues, i.e. RRC and MAC impacts. For RRC impacts, RAN1 provided the RAN1 parameters in excel sheet and RAN2 implemented them to the RRC specification. It was reasonable because RAN1 needed more time to finalize the RAN1 features to create the RAN1 parameters (i.e. this work conducted in the very late phase of WI).

In Rel-17 feMIMO, RAN2 managed to nicely solve a number of issues with the excel sheet for RRC parameters provided by RAN1 (e.g. the TCI state lists) but some other things were largely missed, e.g. the "TCI referencing" feature has not been thoroughly analysed. RAN2 had a lot of discussion with RAN1 using LSes for better understanding on each feature and had to write extensive text in field descriptions after completion of the WI.

At least for Rel-18 MIMO now, we think the RRC impacts should be more discussed by RAN1 and we believe RAN1 will request the RAN2 action if needed, thus we expect not a few of RRC changes suggested in RAN1 LS [2] can be discussed after receiving the parameter list. On the other hand, to avoid issues like in Rel-17 feMIMO, early start of RRC issues can be focused on those attracting companies’ interests.

  - MAC issues

For MAC impacts, RAN2 identified the MAC issues (e.g. BFD/BFR, MAC CEs for TCI state framework, etc.) based on the WID and RAN1 progress, and started to solve the issue from the beginning of the WI. In Rel-18 MIMO, we think situation is similar with the previous release, RAN2 identify some issues related to the MAC impacts and those issues should be solved by RAN2. Based on RAN1 agreements and request in LS [2], we think at least details for the multiple TA for mTRP, and the unified TCI state enhancements for mTRP should be discussed by RAN2.

Followings are potential MAC issues according to RAN1 LS [2].

| Sub-agenda | RAN1 agreement | MAC impact |
| --- | --- | --- |
| eUTCI | Agreement (RAN1#110bis-e, 2022.10) On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value Agreement (RAN1#110bis-e, 2022.10) On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) | How to orchestrate unified TCI-states MAC CE and coresetPoolIndex to indicate unified TCI states for each of TRPs. |
| 2TA | Agreement (RAN1#110, 2022.08) For multi-DCI based multi-TRP operation with two TAs, support configuring two TAGs belonging to a serving cell. | (Referred in 2.1.) How to handle two TAGs associated with one serving cell, e.g., current TAC MAC CE can be reused or not. |
|  | Conclusion (RAN1#110, 2022.08) For multi-DCI based Multi-TRP operation with two TA enhancement, it is up to RAN2 to decide whether there is a need to enhance CBRA procedure to support per TRP UE-initiated RACH procedure. Agreement: Send LS to RAN2 regarding the above conclusion. Final LS in R1-2213004. | Whether any enhancement on CBRA procedure is needed for two-TA mTRP operation. |
| 8Tx UL | Agreement (RAN1#112, 2023.02) To support dual CW PUSCH transmission for rank>4 by an 8TX UE, reuse DL PDSCH scrambling mechanism to initialize the scrambling sequence generator for codeword q{0,1}, where , and are defined similar to the legacy single CW PUSCH transmission. Working Assumption (RAN1#110bis-e, 2022.10) For uplink transmission with rank>4, support dual CW transmission. Agreement (RAN1#110, 2022.08) Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability - For uplink transmission with rank<=4, single CW is supported - For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e The above applies only with regards to the work scope of this agenda item. | [Deprioritized. RAN1 could provide clearer MAC impacts later] Dual CW is supported for rank>4 8Tx UL transmission. For example, current MAC procedures (e.g., UL HARQ procedure) may have to reflect consideration of up to two TBs (Transport Blocks). |

- Work plan

| Meeting | TU | Work plan |
| --- | --- | --- |
| RAN2#121bis-e | 0.5 | Two-TA Continue discussion on high-level views on RAN2 (MAC, RRC, UE capability) impacts for introducing two TAGs for one serving cell. MAC Initial discussion on high-level views on MAC issues based on RAN1 LS (R1-2302226). Also, can discuss other issues based on companies’ interests. RRC Wait for RAN1 parameter list, but it is not precluded that RAN2 start discussion on high-level views on RRC impacts early based on companies’ interests. |
| RAN2#122 | 0.5 | Two-TA Continue discussion on high-level views on RAN2 (MAC, RRC, UE capability) impacts of introducing two TAGs for one serving cell. MAC Continue discussion on high-level views on MAC issues based on RAN1 LS (R1-2302226). Also, can discuss other issues based on companies’ interests. RRC Wait for RAN1 parameter list, but it is not precluded that RAN2 start discussion on RRC impacts early based on companies’ interests. |
| RAN2#123 | 0.5 | [RAN2 is expected to receive the parameter list from RAN1] Discuss and decide some detailed design (MAC, RRC, UE capability) for realization of RAN1 agreements and solutions for issues found in RAN2. Start drafting CRs (RRC, MAC, etc). |
| RAN2#123bis | 0.5 | Discuss and decide some detailed design (MAC, RRC, UE capability) for realization of RAN1 agreements and solutions for issues found in RAN2. |
| RAN2#124 | 0.5 | Finalize remaining details on design (MAC, RRC, UE capability) for realization of RAN1 agreements and solutions for issues found in RAN2. |

- References

[1] RP-223276, “WID Update: MIMO Evolution for Downlink and Uplink,” Samsung (Moderator), RAN#98-e.

[2] R1-2302226, “LS to RAN2/4 on Agreements for Rel-18 MIMO,” RAN1#112.

[3] R1-2213004, “LS on Multi-DCI Multi-TRP with two TAs,” RAN1#111.

[4] RP-230002, “RAN1 status report to RAN#99,” RAN#99.

Annex: All RAN1 agreements with potential RAN2 impact

eUTCI (Unified TCI framework extension)

RAN1#112

Agreement On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported “RAN2 impact: RRC configuration/parameter per PUCCH resource/resource group” “RAN4 impact: none”

RAN1#111

Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. “RAN2 impact: Presence of the new DCI field is configurable by RRC” “RAN4 impact: none” Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design “RAN2 impact: RRC configuration/parameter per PUCCH resource/resource group” “RAN4 impact: none”

RAN1#110b-e

Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value “RAN2 impact: MAC-CE for TCI state activation” “RAN4 impact: none” Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) “RAN2 impact: RRC configuration/parameter per CORESET” “RAN4 impact: none” Agreement On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) “RAN2 impact: MAC-CE for TCI state activation” “RAN4 impact: none”

RAN1#110

RAN1#109e

2TA (Two TAs for multi-DCI)

RAN1#112 (2023-02)

Agreement

For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following:

Associate TAG to TCI-state

- Associate TAG ID with UL/joint TCI state

- For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized

- A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG

- Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs

FFS: on how to handle association when Rel-15/16 spatial relation framework is used for

- PUCCH

- DG/CG Type 1/Type 2 PUSCH

- AP/SP/P SRS

“RAN2 impact: RRC, multi-TA”

“RAN4 impact: none”

Agreement

Confirm the following working assumption:

For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI

- the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

“RAN2 impact: RRC, multi-TA”

“RAN4 impact: none”

RAN1#111 (2022-11)

Conclusion

For multi-DCI based Multi-TRP operation with two TA enhancement, it is up to RAN2 to decide whether there is a need to enhance CBRA procedure to support per TRP UE-initiated RACH procedure.

Agreement: Send LS to RAN2 regarding the above conclusion. Final LS in R1-2213004.

“RAN2 impact: MAC, multi-TA”

“RAN4 impact: none”

RAN1#110 (2022-08)

Agreement

For multi-DCI based multi-TRP operation with two TAs, support configuring two TAGs belonging to a serving cell.

“RAN2 impact: RRC, multi-TA”

“RAN4 impact: none”

Agreement

For multi-DCI multi-TRP operation with two TAs, up to two n-TimingAdvanceOffset value per serving cell is supported

“RAN2 impact: RRC, multi-TA”

“RAN4 impact: none”

CSI (CSI enhancements)

Issue 1 (Type-II CJT)

“RAN2 impact: RRC, Rel-18 CJT codebook configuration”

“RAN4 impact: none”

[109-e] Agreement

On the Type-II codebook refinement for CJT mTRP, the resulting codebook(s) are associated with at least the following parameters:

  - Parameters for basis reporting, including

    - The number of basis vectors: gNB-configured via higher-layer signaling

      - FFS: Whether it is layer-common or layer-specific, whether it is per TRP/TRP-group or common for all TRPs

    - Basis selection indicator(s): a part of CSI report

      - FFS: Whether it is layer-common or layer-specific, whether it is per TRP/TRP-group or common for all TRPs

  - Quantized combining coefficients (W2): a part of CSI report

    - FFS: details of quantization scheme

  - Number of non-zero coefficients and bitmap to indicate non-zero coefficients, including whether it is per TRP/TRP-group (separate) or across all TRPs/TRP-groups (joint): a part of CSI report

  - Strongest coefficient indicator(s) (SCI(s)): a part of CSI report

    - FFS: One per TRP/TRP-group or common for all TRPs

    - FFS: Additional need for strongest TRP indicator

[110] Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP with NTRP>1 TRP/TRP-groups, the following is supported:

- The CMR comprises K>1 NZP CSI-RS resources, where one resource corresponds to one TRP/TRP-group (i.e. K=NTRP)

  - Each of the CSI-RS resources has a same number of CSI-RS ports

- Note: The terms TRP and TRP-group are used for discussion purposes only (no spec impact is implied).

[110] Agreement

For the Rel-18 Type-II codebook for CJT mTRP, support the following two modes:

- Mode 1: Per-TRP/TRP-group SD/FD basis selection which allows independent FD basis selection across N TRPs / TRP groups. Example formulation (N = number of TRPs or TRP groups):

W1,1W2,1Wf,1H⋮W1,NW2,NWf,NH

- Mode 2: Per-TRP/TRP group (port-group or resource) SD basis selection and joint/common (across N TRPs) FD basis selection. Example formulation (N = number of TRPs or TRP groups):

W1,1W2,1WfH⋮W1,NW2,NWfH

- Striving for the two modes to share commonality in detailed designs such as parameter combinations, basis selection, TRP (group) selection, reference amplitude, W2 quantization schemes.

- FFS: Depending on the decision on SCI design, whether additional per-TRP/TRP-group amplitude scaling and/or co-phase is needed or not, and whether they are a part of W2s

[110bis-e] Agreement

For the Rel-18 Type-II codebook for CJT mTRP, the switching between mode-1 and mode-2 is gNB-initiated via RRC signalling

[110bis-e] Agreement

On the Type-II codebook refinement for CJT mTRP, following legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II), for a given CSI-RS resource:

- SD basis selection is layer-common and polarization-common, with N1, N2, O1, O2 defined per Rel-16 specification for refinement based on Rel-16 regular eType-II, and per Rel-17 specification for refinement based on Rel-17 PS FeType-II

- FD basis selection is

  - For refinement based on Rel-16 regular eType-II: per-layer with Mv, pv, N3, and R defined per Rel-16 specification

  - For refinement based on Rel-17 PS FeType-II: layer-common with M, N3, and R defined per Rel-17 specification

  - FFS: Details on FD basis selection window

Note: The supported value(s) for each of the defined parameters are to be discussed separately (e.g. possibilities of adding new or removing existing value(s) in addition to those supported by legacy specification).

[110bis-e] Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP,

- Only CSI reporting over PUSCH is supported

  - FFS: Whether AP only, or both AP and SP (following legacy), is supported

- An associated Resource Setting includes a CMR comprising K≥1 NZP CSI-RS resources from one CSI-RS resource set

  - Periodic, semi-persistent, and aperiodic NZP CSI-RS are supported

  - The supported CSI-RS resource parameter settings follow the legacy specification (without additional enhancement)

  - FFS: Whether or not the K NZP CSI-RS resources are constrained to be in the same slot

[110bis-e] Agreement

On the SD basis selection for Type-II codebook refinement for CJT mTRP, support the following on the L parameter:

- Per-CSI-RS-resource Ln parameter

  - TBD: Whether {Ln, n=1, ..., N} are higher-layer configured by gNB, or the total n=1NLn is higher-layer configured by gNB while {Ln, n=1, ..., N} are reported by the UE, one L configured and {Ln} determined from configured L

  - FFS: The value of Ln is taken from a pre-defined set

[110bis-e] Conclusion: On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group and Strongest Coefficient Indicator (SCI) design, there is no consensus on supporting “strongest” CSI-RS resource indicator in addition to the agreed SCI.

- Note: This doesn’t preclude any (future) proposal on reference CSI-RS resource(s) for other purpose(s)

[110bis-e] Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP, following legacy, support both aperiodic and semi-persistent CSI reporting on PUSCH.

[110bis-e] Agreement

On the Type-II codebook refinement for CJT mTRP, the selection of N CSI-RS resources is performed by UE and reported as a part of CSI report where N∈{1,..., NTRP}

- N is the number of cooperating CSI-RS resources, while NTRP is the maximum number of cooperating CSI-RS resources configured by gNB via higher-layer signaling

- The selection of N out of NTRP CSI-RS resources is reported via NTRP-bit bitmap in CSI part 1

  - Note: The value of N is inferred from the selection

- A restricted configuration (gNB-configured via higher-layer signaling) where N=NTRP is supported

  - NTRP-bit bitmap is not reported when the restriction is configured

  - FFS: Whether other RRC-configured TRP selection restriction including configuring the value of N is supported

- This feature is UE optional

Note: This agreement does not impact the decision on Ln being configured by gNB or selected by UE

Note: per WID and previous agreement, the candidate values for NTRP of are 1, 2, 3, and 4.

Note: only one transmission hypothesis is reported. UE is not mandated to calculate CSI for multiple transmission hypotheses.

[110bis-e] Agreement

On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer:

- Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2)

  - FFS: Amplitude quantization table enhancement

  - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported

- Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111)

  - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N)

    - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported

- If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

[110bis-e] Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP, also support a constraint on the total number of non-zero coefficients (NZCs) summed across all layers:

- Following the legacy specification, the maximum total number is 2K0

[110bis-e] Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameters, for a given CSI-RS resource, the supported value(s) of the following parameters follow the legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II) specification:

- N1, N2, N3, O1, O2

- M (only for design based on Rel-17 PS FeType-II)

For the following parameters, decide in RAN1#111 whether the supported value(s) follow the legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II) specification or further refinement is needed:

- R: including, e.g. supporting only R=1, or supporting larger R values

- Mv/pv (Rel-16 regular eType-II): including, e.g. supporting smaller pv values such as {1/8, 1/4, 1/2} for v=1,2 and/or removing larger legacy value(s)

- : including, e.g. supporting smaller values such as {1/16, 1/8, 3/8}

Note: The outcome of Parameter Combination discussion will further restrict the supported combinations of parameter value(s)

FFS: For N>1, whether the maximum 2N1N2 (identical to the number of CSI-RS ports used for CMR) is limited to 32 just as in legacy specification

[110bis-e] Agreement

For the Rel-18 Type-II codebook for CJT mTRP, for mode-1, the number of FD basis vectors (Mv related to pv for Rel-16, M for Rel-17) is common across all N CSI-RS resources

[111] Agreement

For the Type-II codebook refinement for CJT mTRP, the values of the following codebook parameters are gNB-configured via higher-layer (RRC) signaling and common across all N CSI-RS resources:

- N1, N2

  - Note: Since (O1, O2) are fixed for a given (N1, N2), (O1, O2) are also common across all N CSI-RS resources

- R, 

  - Note: Since N3 is inferred from the number of CQI subbands and R, N3 is also common across all N CSI-RS resources

  - Note: Since per-layer K0 is defined jointly across all N CSI-RS resources, so is .

[111] Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter R, the supported value(s) from the legacy specification are reused.

- FFS: whether additional value 4 can also be added

[111] Agreement

On the Type-II codebook refinement for CJT mTRP:

- The maximum value of 2NN1N2 is 128

  - The support of 2NN1N2 >32 is UE optional for UEs supporting Rel-18 CJT CSI enhancement

- Note: Following the legacy specification on the maximum number of NZP CSI-RS ports per CSI-RS resource, the maximum value of 2N1N2 is 32.

[111] Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the SD basis selection, for a configured value of NTRP, a set of NL combinations of values for {L1, ..., LNTRP} is gNB-configured via higher-layer (RRC) signaling

- When NL>1, the selected combination of values for {L1, ..., LNTRP} is reported in CSI part 1 using an indicator, selected from the NL configured combinations

  - NL =1 is one of the supported candidate values

  - FFS: Other supported value(s) of NL, and its respective UE capability

  - FFS: The supported combinations of values for {L1, ..., LNTRP}

- Following the legacy design, the SD basis selection for the n-th (n=1,...,N) selected CSI-RS resource is indicated in CSI part 2 using a combinatorial indicator selected from a set of  PCSI-RS/2Ln codepoints where, for Rel-16-based refinement PCSI-RS = 2*N1N2.

- The supported candidate values for each of the Ln parameters include the legacy candidate values, i.e. {2,4,6} for Rel-16-based refinement, and

  - for Rel-17-based refinement, the gNB configures a set of N_L combinations for {alpha1, ..., alphaNTRP}  Ln=αnPCSI-RS/2 where αn=12,34, 1

FFS: Whether the set of NL combinations of values for {L1, ..., LNTRP} can be implicitly derived

Following the legacy design, for all the selected N CSI-RS resources, the SD basis oversampling group for each CSI-RS resource is indicated in CSI part 2 using an indicator selected from a set of O1O2 codepoints.

[112] Agreement

On the Type-II codebook refinement for CJT mTRP, only support NL ={2,4} as additional candidate values to NL=1.

- FFS: Additional restriction(s) depending on the configured value for NTRP

[112] Agreement

On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {Ln} for the higher-layer-configured value of NTRP (FFS by RAN1#112: whether the bracketed permutations are also supported):

- FFS by RAN1#112: whether other combinations can be supported

FFS (by RAN1#112bis-e): Whether/how the supported combinations of {n} for Rel-17-based refinement are derived from the supported combinations of {Ln} for Rel-16-based refinement

FFS: Whether the total number of Ln is a UE capability

| NTRP | {Ln} combination |
| --- | --- |
| 1 | {2} |
|  | {4} |
|  | {6} (analogous to legacy, only for total # ports =32, rank 1-2, R=1 |
| 2 | {2,2} |
|  | {2,4}, [{4,2}] |
|  | {4,4} |
| 3 | {2,2,2} |
|  | {2,2,4} [and its other permutations] |
|  | {4,4,4} |
| 4 | {2,2,2,2} |
|  | {2,2,2,4} [and its other permutations] |
|  | {2,2,4,4} [and its other permutations] |
|  | {4,4,4,4} |

[112] Agreement

On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {pv,} from where the value of {pv,} is gNB-configured via higher-layer (RRC) signaling:

- FFS by RAN1#112: whether other combinations can be supported

FFS (by RAN1#112bis-e): Whether/how the supported combinations of {M} for Rel-17-based refinement are derived from the supported combinations of {pv ,} for Rel-16-based refinement

| pv for layers 1-4 |  | Condition(s) |
| --- | --- | --- |
| {1/8, 1/8, 1/16, 1/16} | ¼ | -- |
|  | ½ | -- |
| {1/4, 1/4, 1/8, 1/8} | ¼ (*) | -- |
|  | ½ (*) | -- |
| {1/4, 1/4, 1/4, 1/4} | ¾ (*) | -- |
| {1/2, 1/2, 1/2, 1/2} | ½ | - Only applicable when NTRP≤3 and NL=1 - Optional |

(*) Supported by legacy Rel-16

[112] Agreement

On the Type-II codebook refinement for CJT mTRP, regarding CBSR, at least for restricting SD basis selection, the legacy CBSR scheme is fully reused for each of the RRC-configured NTRP CSI-RS resources (resulting in CSI-RS-resource-specific SD beam group restriction)

- FFS: Whether amplitude restriction is CSI-RS-resource-common or specific, and soft vs hard restriction

- FFS: Whether CBSR can be configured to be off for a CSI-RS resource

The same rank restriction is applied across NTRP CSI-RS resources

[112] Agreement

On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, regarding the list of supported combinations of {Ln}, only support the following additional combinations:

| NTRP | {Ln} combination |
| --- | --- |
| 2 | {4,2} |
| 3 | {2,4,2}, {4,2,2} |

No other permutations are supported.

FFS: For NTRP>1, in addition to the supported combinations/permutations, whether to support at least one additional combination where at least one of the Ln values (n=1, …, NTRP) is 6

[112] Agreement

On the Parameter Combination of Type-II codebook refinement for CJT mTRP, support linkage between the list of supported {Ln} combinations and list of supported {pv,} combinations via pairing each combination for {pv,} with at least one combination for {Ln}, for each NTRP value.

- FFS (by RAN1#112bis-e): The exact list of supported pairs/linkage, or restriction of {Ln} when paired to each of {pv,}

- FFS (by RAN1#112bis-e): Whether/How to support configuration signalling for indicating the linkage

- Note: While no additional codebook parameter will be introduced, the total number of SD basis vectors across CSI-RS resources can still be used as a criterion for choosing the supported pairs/linkage

Issue 2 (Type-II Doppler)

“RAN2 impact: RRC, Rel-18 Doppler codebook configuration”

“RAN4 impact: none”

[110bis-e] Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, when UE-side prediction is assumed, support UE “predicting” channel/CSI after slot l where the location of slot l is configured (from multiple candidate values) by gNB via higher-layer signalling

- Candidates of slot l location include the legacy CSI reference resource location (n – nCSI,ref ) and slot (n+δ) where δ ≥ 0

- FFS: Possible value(s) of δ and possible value(s) of WCSI

Note: Per legacy behavior, the legacy CSI reference resource, i.e., (n – nCSI,ref ), is reused for locating the last CSI-RS occasion used for a CSI report

For a UE that supports UE-side prediction, the support of l = (n – nCSI,ref ) is UE optional.

[110bis-e] Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, support the following codebook structure where N4 is gNB-configured via higher-layer signaling:

- For N4=1, Doppler-domain basis is the identity (no Doppler-domain compression) reusing the legacy W1, W2, and Wf, e.g. W1W2(Wf)H

- For N4>1, Doppler-domain orthogonal DFT basis commonly selected for all SD/FD bases reusing the legacy W1 and Wf, e.g. W1W2Wf⨂WdH

  - Only Q (denoting the number of selected DD basis vectors) >1 is allowed

  - TBD (by RAN1#110bis): whether rotation is used or not

  - FFS: identical or different rotation factors for different SD components

  - FFS: Whether Q is RRC-configured or reported by the UE

Note: Detailed designs for SD/FD bases including the associated UCI parameters follow the legacy specification

FFS: Whether one CSI reporting instance includes multiple W2 and a single W1 and Wf report.

[110bis-e] Agreement

For the Type-II codebook refinement for high/medium velocities, only CSI reporting over PUSCH is supported

- Following legacy, support both aperiodic and semi-persistent CSI reporting on PUSCH.

[110bis-e] Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, when N4>1, if multiple candidates of Q value are supported, the value of Q is gNB-configured via higher-layer (RRC) signalling

[110bis-e] Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the following CSI-RS resource types/structures for CMR:

- Time-domain behaviour for NZP CSI-RS resource: periodic (P), semi-persistent (SP), aperiodic (AP)

  - FFS: Whether to introduce constraints on allowed configuration

- Down select from the following:

  - Alt1. Support K>1 NZP CSI-RS resources, received via a single triggering instance, for aperiodic (AP) -CSI-RS-based channel measurement in a same CSI-RS resource set where the separation between 2 consecutive AP-CSI-RS resources is m slot(s):

  - Alt2. Support one NZP CSI-RS resource in a CSI-RS resource set, where K>1 occasions are received via a single triggering instance, for aperiodic (AP)-CSI-RS-based channel measurement where the separation between 2 consecutive AP-CSI-RS resources is m slot(s).

  - For any of the alternatives:

    - No CRI is reported

    - FFS: Details, e.g., supported value(s) of K, m, other use cases for the AP-CSI-RS resources (e.g., for training filter coefficients, prediction or performance monitoring)

- Support only one NZP CSI-RS resource for P or SP-CSI-RS-based channel measurement

[110bis-e] Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the following CSI-RS resource types/structures for CMR, support the following:

- (Alt1) Support K>1 NZP CSI-RS resources, received via a single triggering instance, for aperiodic (AP) CSI-RS-based channel measurement in a same CSI-RS resource set where the separation between 2 consecutive AP-CSI-RS resources is m slot(s)

[111] Conclusion

For the Rel-18 Type-II codebook refinement for high/medium velocities, there is no consensus on applying DD unit for CQI. Therefore, DD unit (of size d slots) is applied only to PMI.

- Note: This conclusion has no impact on the number of CQIs included in one CSI reporting instance (a separate issue to be decided separately)

[111] Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities,

- For PMI, DD unit duration of d (in slots) is the duration associated with each of the N4 W2 matrices (combining coefficients before DD compression at the UE, or after DD de-compression at the gNB).

- TBD (by RAN1#111): The time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), and the number of CQI(s) X included in a CSI report

[111] Agreement

For the Type-II codebook refinement for high/medium velocities, the parameter N4 (length of DFT vector, unit-less) is gNB-configured via higher-layer (RRC) signalling at least from the following set of candidate values: {1, 2, 4}

- FFS: If additional candidate value(s) of N4 are supported, e.g. 3, 5, 6, 8, 10, 16, 32, as well as the supported Parameter Combination(s)

[111] Agreement

For the Type-II codebook refinement for high/medium velocities, the parameter K (the number of AP-CSI-RS resources for the CMR) is gNB-configured via higher-layer (RRC) signalling at least from the following set of candidate values: {4, 8}

- FFS: If additional candidate value(s) of K are supported, e.g. 5, 12, 16, also taking into account other use cases (e.g. for training filter coefficients, prediction or performance monitoring) and TDD

[111] Agreement

For the Type-II codebook refinement for high/medium velocities, the parameter m (offset between two AP-CSI-RS resources for the CMR, in slots) is gNB-configured via higher-layer (RRC) signalling from the following set of candidate values: {1, 2}

- FFS: Whether 4, 5, 8, 12, and/or 16 are also supported as other candidate value(s)

[111] Agreement

For the Type-II codebook refinement for high/medium velocities, the parameter δ (in slots) is gNB-configured via higher-layer (RRC) signalling from a set of the following candidate values:

- First candidate value: δ=0,

- 2 additional non-zero values of parameter δ are supported

  - FFS: the non-zero value(s), to be selected from 1, 2, 3, 4, 5, 6, and 8

[111] Agreement

For the Type-II codebook refinement for high/medium velocities, regarding the parameter δ (in slots), support the additional value of 2

    - FFS (by RAN1#112): For the last supported additional value, down select between 1, 3, 4, and 5

[111] Agreement

For the Type-II codebook refinement for high/medium velocities, regarding the parameter N4 (length of DFT vector, unit-less), support 8 as an additional candidate value

- FFS (by RAN1#112): Whether any of the following additional candidate values are supported: 3, 5, 16, 32

- The candidate values supported by UE are reported via UE capability (details can be discussed in UE feature).

[111] Agreement

For the Type-II codebook refinement for high/medium velocities, regarding the parameter d (in slots),

- Support at least the following candidate value:

  - If the configured CMR is AP-CSI-RS, this candidate value is the configured value of m parameter

  - FFS: If the configured CMR is P/SP-CSI-RS

  - FFS: Whether in the above two cases, the number of slots between the last CSI-RS occasion no later than the legacy reference resource and the starting of WCSI window shall be integer multiples of d slots.

- FFS: Whether additional candidate value(s) of d are supported, e.g. d<m, d>m, d=1

If more than one candidate values of d are supported, the value of d is gNB-configured via higher-layer (RRC) signalling

[112] Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following:

- Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices

- Optional features:

  - X=1 and the CQI is associated with:

    - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and

    - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrix

  - X=2 and

    - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and

    - The 2nd CQI is associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrix

    - FFS: Whether/how to include CQI overhead reduction for X=2

[112] Agreement

For the Type-II codebook refinement for high/medium velocities, regarding the parameter K (the number of AP-CSI-RS resources for the CMR), optionally support only K=12 as an additional candidate value

[112] Agreement

For the Type-II codebook refinement for high/medium velocities, regarding the parameter δ (in slots), in addition to 0 and 2, δ=1 is additionally supported

[112] Agreement

For the Type-II codebook refinement for high/medium velocities, regarding the parameter d (in slots),

- for P/SP-CSI-RS, support d equal to the periodicity of the CSI-RS resource

- for AP-CSI-RS, also support d =1

[112] Agreement

For the Type-II codebook refinement for high/medium velocities,

- The constraint on the maximum number of non-zero coefficients (NZCs) per-layer (K0) is defined jointly across all Q DD basis vectors.

  - FFS: How K0 is calculated

- Also support a constraint on the total number of non-zero coefficients (NZCs) summed across all Q DD basis vectors and across all layers:

  - Following the legacy specification, the maximum total number is 2K0

[112] Agreement

On the Type-II codebook refinement for high/medium velocities based on Rel-16 regular eType-II codebook (if supported), for the purpose of choosing the supported Parameter Combinations

- Regarding the codebook parameter pv, in addition to the supported values from the legacy specification, introduce as additional candidate values

  - pv =1/8 for v=1,2 (hence 1/16 for v=3,4)

  - pv =1/2 for v=1,2,3,4

- Regarding the codebook parameter , in addition to the supported values from the legacy specification, introduce as an additional candidate value = 1/8

- Regarding the codebook parameter L, the supported values from the legacy specification apply

Issue 3 (TDCP)

“RAN2 impact: RRC, TDCP reporting configuration”

“RAN4 impact: none”

[112] Agreement

For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows:

- Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported

  - FFS: Candidate values for delay

- Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported

  - For Y>1, the phase can be configured to be absent for all the Y delays

  - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource

  - TBD: Candidate value(s) for Y>1

- FFS: Value of Dbasic

[112] Agreement

For the Rel-18 TRS-based TDCP reporting, regarding the value of parameter Y for Y>1, down-select from the following alternatives:

- Alt1. The value of Y is gNB-configured via higher-layer (RRC) signalling

- Alt2. The value of Y follows the delays from the configured TRS resource

- Alt3. The value of Y is UE-selected and reported

The value of Y is a UE capability

DMRS (Increased number of orthogonal DMRS ports)

RAN1#112

Conclusion (RAN2) Dynamic switching between R15 DMRS port and R18 DMRS port by a scheduling DCI is not supported in Rel-18

RAN1#111

RAN1#110b-e

Agreement (RAN2 RRC) For more than 4 layers SU-MIMO PUSCH, support - Both Rel.15 Type 1/Type 2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports. - For UE supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with either of Rel.15 Type 1/Type 2 DMRS ports or Rel.18 eType 1/eType 2 DMRS ports. - RRC based indication is supported as the baseline. FFS whether DCI based indication is further needed. - For UE not supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with Rel.15 Type 1/Type 2 DMRS ports only.

RAN1#110

RAN1#109e

SRS (SRS enhancement targeting TDD CJT and 8 TX)

RAN1#112

Agreement (RAN2 RRC) For SRS interference randomization, support: Opt. 3: Both cyclic shift hopping and comb offset hopping. - At least the two features can be separately configured - FFS: Combined cyclic shift hopping and comb offset hopping for a UE - FFS: Separate or combined with SRS sequence group hopping / sequence hopping - FFS: Associated UE capability “RAN2 impact: RRC, SRS configuration / UE capability for comb offset hopping / cyclic shift hopping” “RAN4 impact: none” Agreement (RAN2 RRC) For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs. Option 1: Reuse the SRS sequence identity nIDSRS. Option 2: Introduce new ID(s). - FFS: the value range, one new ID or two separate new IDs, default ID(s) “RAN2 impact: RRC, SRS configuration of sequence ID for comb offset hopping / cyclic shift hopping” “RAN4 impact: none” Agreement (RAN2 RRC) For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports. At least s = 2. - FFS: s = 4, s = 8. m = 2,4,8, 10,12,14, and m is a multiple of s. Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol. - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports. - FFS: down selection from existing resource mapping designs FFS: which subset of 8/s ports are mapped onto each OFDM symbol. FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters. “RAN2 impact: RRC, SRS configuration of TDM parameters for 8 Tx SRS” “RAN4 impact: none” Agreement (RAN2 RRC) For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof), - Option 2: For comb 4, do not support 4 comb offsets. “RAN2 impact: RRC, SRS configuration of non-TDM parameters for 8 Tx SRS” “RAN4 impact: none” Agreement (RAN2 RRC) For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options: Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions. Option 2: Within a slot, hopping based on only the symbol index l'. Option 3: No intra-slot hopping. FFS: Time domain hopping behaviour further depends on system frame number (SFN) nf. - FFS: reinitialization periodicity of N radio frames or reinitialization based on system frame number. FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately) FFS: At least support reinitialization at the beginning of each radio frame. “RAN2 impact: RRC, SRS configuration for comb offset hopping / cyclic shift hopping” “RAN4 impact: none” Agreement (RAN2 RRC) For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern: Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols. Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols. “RAN2 impact: RRC, SRS configuration of TDM parameters for 8 Tx SRS” “RAN4 impact: none”

RAN1#111

Agreement (RAN2 RRC) For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, FFS: Hopping pattern Support at least hopping based on slot index, OFDM symbol index - FFS: Use of symbol group based on repetition factor - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource - FFS: Re-initialization periodicity Applicable to at least periodic/semi-persistent SRS with usage antennaSwitching FFS: Other types of SRS FFS: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively FFS: Combined comb offset hopping and cyclic shift hopping, supporting both, or down selecting one “RAN2 impact: RRC, SRS configuration for comb offset hopping / cyclic shift hopping” “RAN4 impact: none” Agreement (RAN2 RRC) For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on: Option 1: The hopping pattern is based on the pseudo-random sequence c(i), initialized with a network-configured ID. FFS: The ID could be cell ID nIDcell, SRS sequence identity nIDSRS, C-RNTI, or a new ID FFS: The relation between the legacy group / sequence hopping and the new hopping “RAN2 impact: RRC, SRS configuration for sequence ID for comb offset hopping / cyclic shift hopping” “RAN4 impact: none” Agreement (RAN2 RRC) For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), at least support: - For comb 2, support 1 and 2 comb offsets - For comb 4, support 2 and [4] comb offset - For comb 8, support 4 comb offsets “RAN2 impact: RRC, SRS configuration of non-TDM parameters for 8 Tx SRS” “RAN4 impact: none” Agreement (RAN2 RRC) For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols - Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM) - FFS: m can be legacy values, i.e., 2,4,[8,10,12,14]. “RAN2 impact: RRC, SRS configuration of TDM parameters for 8 Tx SRS” “RAN4 impact: none”

RAN1#110b-e

Agreement (RAN2 RRC) Support at least one of the following for SRS interference randomization - Randomized code-domain resource mapping for SRS transmission by introducing cyclic shift hopping / randomization to SRS resource - Comb offset hopping for SRS - The comb offset is determined pseudo-randomly as a function of time (e.g., slot index, symbol index) and/or NW configured ID with a certain UE-specific initialization. - FFS: Other details, e.g., how the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion. “RAN2 impact: RRC, SRS configuration for comb offset hopping / cyclic shift hopping” “RAN4 impact: none” Agreement (RAN2 RRC) For comb offset hopping for SRS and for randomized code-domain resource mapping for SRS transmission via cyclic shift hopping / randomization, further study the following: - The hopping pattern (e.g., the pseudo-random sequence, time-domain granularity for hopping) - The time-domain parameter and/or behavior (e.g., slot index, symbol index, re-initialization behavior) - Network-configured ID for UE-specific initialization - How the comb offset / cyclic shift value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion - Potential issue on multiplexing with legacy UEs if CS hopping and/or comb offset hopping are enabled - Applicability to periodic/semi-persistent/aperiodic SRS Other details are not excluded “RAN2 impact: RRC, SRS configuration for comb offset hopping / cyclic shift hopping” “RAN4 impact: none” Agreement (RAN2 RRC) For an 8-port SRS resource in a SRS resource set ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with m OFDM symbols (m >= 1), at least support the 8 ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof). - m takes the legacy values, i.e., 1,2,4,8,10,12,14. “RAN2 impact: RRC, SRS configuration of non-TDM parameters for 8 Tx SRS with AS” “RAN4 impact: none” Agreement (RAN2 RRC) For one single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH, when the SRS resource is configured with n ports (n <= 8) and m OFDM symbols (m >= 1), at least support the n ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof). - n can be 8 - m takes the legacy values, i.e., 1,2,4,8,10,12,14. “RAN2 impact: RRC, SRS configuration of non-TDM parameters for 8 Tx SRS with CB” “RAN4 impact: none”

RAN1#110

Agreement (RAN2 RRC) For 8 Tx SRS, at least support - 8 ports in 1 SRS resource for ‘antennaSwitching’; - FFS 8 ports in one or multiple SRS resources for ‘codebook’ Above does not imply support for 8 ports in one or multiple OFDM symbols “RAN2 impact: RRC, SRS configuration of parameters for 8 Tx SRS” “RAN4 impact: none” Agreement (RAN2 RRC) For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR) “RAN2 impact: RRC, SRS configuration of parameters for 8 Tx SRS” “RAN4 impact: none” Agreement (RAN2 RRC) For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol. FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols. “RAN2 impact: RRC, SRS configuration of parameters for 8 Tx SRS” “RAN4 impact: none” Agreement (RAN2 RRC) For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols. - Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling. “RAN2 impact: RRC, SRS configuration of parameters for 8 Tx SRS for NCB” “RAN4 impact: none”

RAN1#109e

STxMP (UL precoding indication for multi-panel transmission)

RAN1#112

Agreement(RAN2 RRC) On dynamic switching between STxMP SFN scheme and sTRP transmission: - The legacy maxRank/Lmax is applied to sTRP transmission - For configuration of SFN, - Alt2: Configure a separate parameter for the maximal number of layers for STxMP SFN Working Assumption(RAN2 RRC) For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following: - For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1) - For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1) - FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same. - Note: This corresponds to the case that digital ports are shared between the panels - Note: RAN1 supports both implementations that digital ports are shared or separate among panels Agreement (RAN2 RRC) Among the two SRS resource sets configured for multi-DCI based STxMP PUSCH+PUSCH, the SRS resource set with lower set ID is the first SRS resource set.

RAN1#111

Agreement(RAN2 RRC) Support the SFN scheme for single-DCI based STxMP PUCCH transmission Agreement(RAN2 RRC) - For multi-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission, - Alt1: The first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1 - The PUSCH is associated with SRS resource set with the same value of coresetPoolIndex - FFS: Which is the first SRS resource set, e.g., the set with lower set ID. - Regarding how to interpret the SRI/TPMI field in DCI: - For DG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where scheduling DCI format 0_1 or 0_2 is received - For Type 2 CG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where activation DCI is received. - For Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig and the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set Agreement(RAN2 RRC) For the SFN scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets for CB or NCB. - FFS: Number of SRS resources of SRS resource set, and number of SRS ports of SRS resource - The DCI indicates two SRI fields and TPMI fields for SFN transmission, - On the indication of number of layers for CB and NCB PUSCH: - Alt1: Similar to rel-17 mTRP TDM scheme, the number of layers is indicated by the first SRI field (for NCB PUSCH) or the first TPMI field (for CB PUSCH) Agreement(RAN2 RRC) For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission: - Use the 2-bit “SRS resource set indicator” DCI field to dynamically indicate the sTRP or SDM transmission. - FFS: how to interpret each codepoint of “SRS resource set indicator”. - For the maximal number of layers for sTRP transmission, down-select: - Option1: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec - Option2: Configuring one additional maximal numbers of layers for sTRP transmission, in addition to maxRank in current spec. - Down-select one from the following for maximal number of layers of SDM transmission: - Alt1: Configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately. - Alt1a: The maxRank (or Lmax) in current spec is also applied to the first SRS resource set and the second SRS resource set, separately. - Alt2: Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set - Alt3: no dedicated configuration for SDM. The maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission. - Alt4: The maximal number(s) of layers in above Option1/2 also applied to the first SRS resource set and the second SRS resource set, separately. - FFS: To ensure the same size of DCI for sTRP and SDM cases, how to use/interpret the TPMI/SRI field(s) and whether to do reserved bit or zero padding.

RAN1#110b-e

Agreement(RAN2 RRC) For SDM scheme of single-DCI based STxMP PUSCH - Configure two SRS resource sets for CB or NCB . - FFS : These two SRS resource sets can have different number of SRS resources for codebook -based or non-codebook based. - For codebook -based PUSCH , DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set. - For non-codebook based PUSCH and codebook -based PUSCH , DCI indicates two SRI fields and each field indicates SRS resource(s) for each SRS resource set separately. - FFS : For codebook -based PUSCH , the two SRS resources indicated by the two SRI fields can have different number of SRS ports Agreement(RAN2 RRC) Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH: - Configure two SRS resource sets for CB or NCB. - FFS: Whether/how to associate coresetPoolIndex with SRS resource set implicitly or explicitly. - FFS: the maximal number of configured/indicated SRS resources in each set for NCB/CB - FFS: the maximal number of SRS ports in each set for CB. - FFS: Separate codebooks and separate maxRanks are configured for different SRS resource sets. - For type 1 CG-PUSCH (if supported), FFS how to associate the PUSCH with one TRP - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH Agreement(RAN2 RRC) Support SFN-based transmission scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18 Agreement(RAN2 RRC) For the switching between SDM scheme of single-DCI based STxMP PUSCH and Rel-17 mTRP PUSCH TDM scheme, Alt2 is supported. FFS: Whether Alt1 is supported in addition to Alt2. - Alt1: Support dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and Rel-17 mTRP PUSCH TDM scheme - FFS: how to support dynamic switching, e.g., using the indicated PUSCH repetition number - Note: It is up to gNB implementation to configure SDM scheme of single-DCI based STxMP PUSCH or Rel-17 mTRP PUSCH TDM scheme or both of them in RRC. Dynamic switching between them is only when both schemes are configured in RRC. - Alt2: Support RRC-based switching between SDM scheme of single-DCI based STxMP PUSCH and Rel-17 mTRP PUSCH TDM scheme Agreement(RAN2 RRC) Support SFN-based transmission scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18

RAN1#110

RAN1#109e

8Tx UL (SRI/TPMI enhancement for enabling 8 TX UL)

RAN1 Meeting #112

Agreement

For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook, the following pairs of (N1, N2) values are supported,

- (N1, N2) = (4, 1)

- (N1, N2) = (2, 2)

A pair of (N1, N2) can be configured with subject to UE capability.

“RAN2 impact: RRC Codebook configuration based on (N1,N2)”

“RAN4 impact: none”

Agreement

Fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook

- Precoding matrices generated according to (O1, O2) = (1, 1) is supported

- Further study additional support of precoding matrices generated according to (O1, O2) where O1>1 or O2>1

  - Subject to UE capability

- FFS: Different O1, O2 values for different ranks

“RAN2 impact: RRC Codebook configuration based on (O1,O2)”

“RAN4 impact: none”

Agreement

To support dual CW PUSCH transmission for rank>4 by an 8TX UE, for MCS indication, support

- Alt.2: A second MCS field (5 bits) is indicated for the second codeword

“RAN2 impact: DCI presence configuration by RRC”

“RAN4 impact: none”

Agreement

To support dual CW PUSCH transmission for rank>4 by an 8TX UE, a second set of NDI (1 bit) and RV (2 bits) fields are indicated.

- FFS: Details on how to signal

“RAN2 impact: DCI presence configuration by RRC”

“RAN4 impact: none”

Agreement

To support dual CW PUSCH transmission for rank>4 by an 8TX UE, reuse DL PDSCH scrambling mechanism to initialize the scrambling sequence generator for codeword q{0,1},

where , and  are defined similar to the legacy single CW PUSCH transmission.

“RAN2 impact: MAC/RRC handling of q, , and ”

“RAN4 impact: none”

Agreement

For CB-based 8TX PUSCH transmission, where Mode 2 uplink full power transmission (if supported) is not used, re-use legacy Rel-15 mechanism, that is

- when only one SRS resource in a resource set is configured, the SRI field in DCI is absent,

- when two SRS resources are configured in a resource set, 1 bit of SRI field in DCI is used to indicate the selected SRS resource in the set.

“RAN2 impact: DCI presence configuration by RRC”

“RAN4 impact: none”

Agreement

For partially coherent uplink precoding by an 8TX UE codebook,

- When Ng=2

  - Precoding design is based on Rel-15 UL 4TX codebook,

    - Full-coherent precoders are used

      - FFS whether partial-coherent precoders are needed

- When Ng=4, down-select from,

  - Alt1:

    - Precoding design is based on Rel-15 UL 2TX codebook,

      - Full-coherent precoders are used

  - Alt2:

    - Precoding design is based on Rel-15 UL 4TX codebook,

      - Partial-coherent precoders are used

“RAN2 impact: RRC Codebook configuration based on Ng”

“RAN4 impact: none”

RAN1 Meeting #111

Agreement

For a fully coherent uplink precoding by an 8TX UE,

- Support NR Rel-15 single panel DL Type I codebook as the starting point for design of the codebook

  - FFS: For a constructed codebook with size M based on above method, unless M=2N; otherwise, round up the codebook size to the smallest integer 2N>M by adding 2N-M precoders generated via Alt 2a.

- No LS to RAN4 will be needed

“RAN2 impact: RRC Codebook and DCI configuration”

“RAN4 impact: none”

RAN1 Meeting #110bis-e

Agreement

Support the following cases for codebook design for 8TX precoders

- Full coherent precoders with Ng=1

  - FFS: Full coherent precoders with Ng=2, Ng=4

- Partial coherent precoders with Ng=2 and Ng=4

  - This does not imply any relation with the number of TPMI indications for 8TX precoder

- Non-coherent precoders

“RAN2 impact: RRC Codebook configuration based on Ng”

“RAN4 impact: none”

Agreement

For 8TX UE codebook-based uplink transmission,

- For partially/non-coherent precoding, support NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook

“RAN2 impact: RRC Codebook configuration based on UE coherency”

“RAN4 impact: none”

Agreement

For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, Alt1 is supported, that is

- Alt1: A single SRS resource set configured with up to 8 single-port SRS resources

- FFS: Configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively.

“RAN2 impact: SRS configuration by RRC”

“RAN4 impact: none”

Agreement

For SRS configuration supporting codebook -based UL transmission for an 8TX UE ,

- Support configuration of 1 SRS resource set containing up to X 8-port SRS resource(s), where X = 2

  - FFS : Other values for X, if needed

“RAN2 impact: SRS configuration by RRC”

“RAN4 impact: none”

Working Assumption

For uplink transmission with rank>4, support dual CW transmission.

“RAN2 impact: RRC/MAC configuration for the 2nd CW”

“RAN4 impact: none”

RAN1 Meeting #110

Agreement

For 8TX PUSCH, at least support

- Ng=1, 2, 4

Note: The above does not restrict the Ng for the non-coherent case

“RAN2 impact: RRC configuration based on Ng”

“RAN4 impact: none”

Agreement

Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability

- For uplink transmission with rank<=4, single CW is supported

- For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e

The above applies only with regards to the work scope of this agenda item.

“RAN2 impact: RRC/MAC configuration for the 2nd CW based on UE capability”

“RAN4 impact: none”