3GPP TSG-RAN WG2 Meeting #121bis-e                                                             R2-2302891

Electronic meeting, April 17 – 26, 2023

Agenda item: 		7.12.2.1

Source: 		Qualcomm Incorporated

Title: 	Support of mobile-IAB indicator in Msg. 5

Document for:	Discussion

# 1	Introduction

In the last meeting, RAN2 discussed whether the mIAB-MT should include a mobile-IAB indicator in MSG5. RAN2 achieved the following agreements [1]:

mIAB indicator UE to Network Postponed, AMF selection in the base-station is a Ran3 function, Ran2 expect RAN3 to ask for it if support for this is needed

This contribution rediscusses this topic.

# 2	Discussion

## 2.1 	Support for mobile-IAB indication in Msg.5

SA2 has captured the following normative text in TS 23.501, vs. 18.0, section 5.35A.1:

For a MBSR node, it provides a mobile IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the mobile IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB-node with mobility and includes the mobile IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform mobile IAB authorization.

This specification implies that the mobile IAB-MT needs to indicate the support of mobile IAB in Msg.5.

Proposal: Based on normative text in Ts 23.501 vs. 18.0, section 5.35A.1, the mobile IAB-MT to include a mobile-IAB indication in Msg. 5.

# Conclusion

This contribution discussed the need for mobile IAB indicator in MSG.5. The following proposal has been made:

Proposal: Based on normative text in Ts 23.501 vs. 18.0, section 5.35A.1, the mobile IAB-MT to include a mobile-IAB indication in Msg. 5.

# References

- Chairman Notes; TSG RAN WG2 Meeting #121, Athens, Greece, February/March, 2023