3GPP TSG RAN WG2 #121bis-e		R2-23xxxxx

Electronic meeting, 17th – 26th April 2023

Title:	Draft LS on RRC configuration of Tx state in Rel-18 UL Tx switching

Response to:

Release:	Release 18

Work Item:	NR_MC_enh-Core

Source:	NTT DOCOMO, Inc. [to be RAN WG2]

To:	RAN WG1

Cc:	RAN WG4

Contact person:	Riki Okawa

riki.ookawa.rp@nttdocomo.com

Attachments:	none

1	Overall description

RAN2 has discussed how to introduce RRC configuration of an associated band based on the following RAN1 agreement.

Agreement: Following working assumption is confirmed with updates. Working Assumption At least for dual UL, reuse existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState to solve the issue on ambiguous switching state at least for following cases - Case#1 of the issue: two Tx chains are currently associated with band A, and next transmission is 1 port transmission on band B, but there are multiple possible switching cases where 1P on band B is supported - if twoT is indicated, both of two Tx chains are switched to band B - if oneT is indicated, one Tx chain is switched to band B while another Tx chain remains on band A - Case#2 of the issue: two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, but there are multiple possible switching cases where 1P on band C is supported - if twoT is indicated, both of two Tx chains are switched to band C - if oneT is indicated, one Tx chain is switched to band C while how to determine the associated band for another Tx chain is FFS - Alt.1: based on gNB’s configuration/indication e.g., new RRC parameter - Alt.2: based on predefined rule - Other alternative is not precluded FFS for other potential cases

RAN2 achieved the following agreements.

- RAN2#121 For RRC configuration to clarify ambiguous Tx state, RAN2 should introduce an RRC configuration that associates a band to another band which the unused Tx chain is switched to when the switch is from concurrent transmission on two bands to 1 Tx transmission on another band. - RAN2#121bis-e P2: RAN2 reuse uplinkTxSwitching-DualUL-TxState-r17 to indicate the state of Tx chains for dualUL mode.

Furthermore, RAN2 discussed expected behaviours when a UE is indicated to switch from two bands to one different band (e.g. A+B => C) and agreed the following understanding as a baseline in RAN2#121bis-e.

P3-2: Baseline R2 “understanding” (can be modified and clarified in offline) - When the UE is indicated to switch from two bands to one different band (e.g. A+B => C), follow below logic when determine the switched Tx: - - If network indicates 1port transmission on band C, - and uplinkTxSwitching-DualUL-TxState is set to oneT, and the associated band is configured to band C: - ---- Switch 1Tx chain to band C and switch another Tx chain to associated band; - - Else if network indicates 1port transmission on band C, but uplinkTxSwitching-DualUL-TxState is not configured or is set to twoT, or associated band is not configured for band C: - ---- Switching 2Tx chains to band C.

RAN2 would like to ask RAN1 to take the above understanding into account and inform RAN2 if any issue.

2	Actions

To RAN WG1

ACTION: 	RAN2 respectfully asks RAN1 to take the above understanding into account and inform RAN2 if any issue.

3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #122	22nd - 26th May 2023 	Incheon, KR

TSG RAN WG2 Meeting #123	21st – 25th August 2023	Toulouse, FR