3GPP TSG-RAN WG2 #121bis-e	R2-230xxxx

Online, 17th – 26th April, 2023

Title:		[AT121bis-e][417][POS] LS on GNSS integrity parameters (Huawei)

Source:	Huawei, HiSilicon

Agenda item:	8.2.4

Document for:	Discussion and Decision

- Introduction

During R2#120 meeting, LS R2-2213320 has been sent to SA2 for the parameters sent between LCS client/UE/AF and LMF

RAN2 would like to thank SA2 for the LS on GNSS integrity requirement provisioning. RAN2 would like to provide the following answer to SA2's question on the parameters that are needed: - LCS client/UE/AF sends TIR, AL, TTA to the LMF - LMF returns the system available/unavailable indication to the LCS client/UE/AF

While in this meeting, LS R2-2302404has been received from SA4, enquiring about the range of the paremters.

CT4 is implementing the GNSS integrity requirements as agreed SA2 CR S2-2300953 defined in TS 23.273 below: The LCS service request may include integrity requirements including Time-to-Alert (TTA), Target Integrity Risk (TIR) and Alert Limit(AL). Definitions of these parameters are specified in TS 38.305 [9]. However, there is no clear data structure definition of TTA, TIR and AL in TS 38.305, CT4 could not implement this feature based on current definition. CT4 would like to kindly ask RAN2 to define the data structure of TTA, TIR and AL, and provide the related reference to CT4 in order to implement this feature.

During R2#121bis, the following email discussion has been arranged for the discusison for the reply LS

[AT121bis-e][417][POS] LS on GNSS integrity parameters (Huawei)

Scope: Consider the LS in R2-2302404 and draft a reply.

Intended outcome: Report and approvable LS

Deadline: Friday 2023-04-21 1000 UTC

In this contribution, we discuss the data structure of the TTA, TIR, AL and propose a reply LS to CT4

- Discussions

In the current LPP spec, only the data structure for Target Integrity Risk has been defined, as follows:

CommonIEsRequestLocationInformation ::= SEQUENCE {

locationInformationType		LocationInformationType,

triggeredReporting			TriggeredReportingCriteria	OPTIONAL,	-- Cond ECID

periodicalReporting			PeriodicalReportingCriteria OPTIONAL,	-- Need ON

additionalInformation		AdditionalInformation		OPTIONAL,	-- Need ON

qos							QoS							OPTIONAL,	-- Need ON

environment					Environment					OPTIONAL,	-- Need ON

locationCoordinateTypes		LocationCoordinateTypes		OPTIONAL,	-- Need ON

velocityTypes				VelocityTypes				OPTIONAL,	-- Need ON

...,

[[

messageSizeLimitNB-r14	MessageSizeLimitNB-r14		OPTIONAL	-- Need ON

]],

[[

segmentationInfo-r14	SegmentationInfo-r14		OPTIONAL	-- Need ON

]],

[[

scheduledLocationTime-r17

ScheduledLocationTime-r17	OPTIONAL,	-- Need ON

targetIntegrityRisk-r17

TargetIntegrityRisk-r17		OPTIONAL	-- Need ON

]]

}

The reason is that during the previous R2 discussion, we have agreed that only Mode 1 for the integrity report is supported in R2#117

Agreements:

Proposal 3. Release 17 supports only Reporting Mode 1 (PL reporting). Reporting Mode 2 can be revisited in future releases.

Proposal 4. For reporting Mode 1, TTA is not needed.

Proposal 5 (modified). Provide achievable TIR as optional parameter in the Integrity Information Result

While during the discussion in R2#120, for a separate discussion on the assistance data for UE-based integrity, the following contribution has been proposed for adding the AD for alert limit and time to alert, with proposed TP

R2-2212892 Integrity measurements definition and missing integrity requirements Ericsson discussion Rel-17

We think the data structure of the contribution above can be taken as the baseline for the discussion on the reply LS to CT4

Question1: Do companies agree to adopt the values for Alert Limit and Time to Alert in R2-2212892 for the data structure of TIR and AL?

| Company | Yes/No | Comment |
| --- | --- | --- |

With the above, we have also drafted a reply LS to CT4 for this issue, we’d like also to collect companies’ view on this

Quesiton2: Do companies have any comment on the reply LS in draft LS in Annex A?

| Company | Comment |
| --- | --- |

- Conclusions

TBD

- Annex A: Draft LS

3GPP TSG RAN WG2#121bis-e                                                                                  R2-230

Online, 17th - 26th April, 2023

Title:	Reply LS on GNSS integrity requirement parameters definition

Response to:	C4-230655 “LS on GNSS integrity requirement parameters definition”

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	RAN2

To:	CT4

Cc:	SA2

Contact person:	Yinghao Guo

yinghaoguo@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NA

1	Overall description

RAN2 would like to thank CT4 for the LS on GNSS integrity requirement parameters definition, and would like to ask CT4 to take the following RAN2 feedback into consideration:

Question: CT4 would like to kindly ask RAN2 to define the data structure of TTA, TIR and AL, and provide the related reference to CT4 in order to implement this feature.

Answer: For TIR, the data structure is defined by IE targetIntegrityRisk, which is specified in TS 37.355. For TTA and AL, the following range of values can be adopted with field description

IntegrityRequirements-r17 ::= SEQUENCE {

horizontalAlertLimit-r17			INTEGER (0..50000),

verticalAlertLimit-r17				INTEGER (0..50000)				OPTIONAL, -- Need ON

timeToAlert-r17			        	INTEGER (1,2000)				OPTIONAL, -- Need ON

...

}

IntegrityRequirements field descriptions

horizontalAlertLimit This field indicates the horizontal alert limit for the integrity principle of operation by the device along the semi-major axis of the error ellipse. Scale factor 0.01 metre; range 0 – 500 meters.

verticalAlertLimit This field indicates the vertical alert limit for the integrity principle of operation by the device. Scale factor 0.01 metre; range 0 – 500 metres. To be compared to the horizontal protection level determined by the device.

timeToAlert The maximum allowable elapsed time from when the protection level (PL) exceeds the Alert Limit (AL) until the function providing positioning integrity annunciates a corresponding alert. Scale factor 0.1 second.

2	Actions

To CT4

ACTION: RAN2 kindly requests CT4 to take the above answers into consideration.

3	Dates of next RAN WG2 meetings

RAN2 #122                      22-26 May 2023                    Incheon

<meeting_identity>	<start_date> - <end_date>	<town>, <country>