3GPP TSG-RAN WG2 Meeting #122		R2-2305337

Incheon, Korea, May. 22th- May. 26th, 2023

Agenda Item:	7.3.4

Source:	vivo

Title:	Discussion on cell selection/re-selection

Document for:	Discussion and Decision

Introduction

In RAN2#121 meeting, cell selection/re-selection mechanism for network energy saving was further discussed with the below agreement [1]:

- RAN2 confirms that non-NES UEs can access to NES cells if NES solution is backwards compatible

In this contribution, we provide our consideration on the technique of cell selection/re-selection for network energy saving.

Discussion

According to the agreement made in RAN2#121, RAN2 confirms that non-NES UEs can access to NES cells if NES solution is backwards compatible. Therefore, RAN2 needs to evaluate whether the potential NES solutions are backwards compatible, as stated below:

SSB-less SCell operation for inter-band CA/ Techniques in spatial and power domains/ CHO procedure enhancement

In R18 NES WI, the techniques listed above only are designed for R18 NES capable UEs in RRC_CONNECTED state, so it has no effect on legacy UEs.

Inter-node beam activation and paging enhancements

Inter-node beam activation and paging enhancement are designed for interacting between between NG-RAN nodes.

Therefore, there is no impact for legacy UEs.

Enhancement on cell DTX/DRX mechanism

The note of enhancement on cell DTX/DRX mechanism in WID [2] is the following:

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

Based on the note, there is no impact on legacy idle/inactive UEs.

Based on RAN1 and RAN2 agreements, when the cell DTX/DRX is activated, Rel-18 UE supporting cell DTX/DRX is not expected to transmit and receive the following signals/channels to the gNB during non-active periods of cell DTX/DRX:

- Periodic/Semi-persistent CSI report

- Periodic/Semi-persistent SRS

- UE doesn’t monitor SPS occasions

- UE does not transmit on CG occasions

- UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period

There are two options for legacy UE camping R18 NES cell when the cell DTX/DRX is activated/deactivated.

Option1: without bar enhancement for R18 NES cell

- Case 1.1: bar all idle/inactive UE, i.e., R18 NES UE and legacy UE.

- Case 1.2: without bar, i.e., allow both R18 NES UE and legacy idle/inactive UE camping

Option2: R18 bar enhancement for cell DTX/DRX, i.e., legacy UE bar only for R18 NES cell

- Case 2.1: bar legacy UE only for R18 NES cell

| Cases | Pros. | Cons. |
| --- | --- | --- |
| Cell DTX/DRX is activated: |  |  |
| Case 1.1 | Legacy behaviour | Only service for RRC connection UE. |
| Case 1.2 | There is no performance loss of legacy idle/inactive UEs. | When legacy UEs enter the RRC_CONNECTED state during non-active periods of cell DTX/DRX: - Unreasonable network configuration: - It may not conducive to legacy UE power saving if legacy UE still transmits CSI report, SRS, and keep monitoring SPS or PDCCH for dynamic grants/assignments for new transmissions; - Data missed for legacy UE if legacy UE still keep transmitting on CG occasions; |
| Case 2.1 | It will avoid unreasonable configuration for network. | Legacy UE performance is degraded if the legacy UEs cannot find a suitable or good quality cell. |
| Cell DTX/DRX is deactivated: |  |  |
| Case 1.1 | Legacy behaviour | Only service for RRC connection UE. |
| Case 1.2 | Legacy behaviour. | If a large number of legacy UEs camp on the NES cell, it will further hinder the cell DTX/DRX is activated. |
| Case 2.1 | If the network only needs serve NES capable UEs, cell DTX/DRX can be activated easilier. | Legacy UE performance is degraded if the legacy UEs cannot find a suitable or good quality cell. |

Table 1: The pros and cons of the cases

Based on the above pros and cons analysis, we think when legacy UE enters the RRC_CONNECTED state during non-active periods of cell DTX/DRX, unreasonable network configuration can be avoided via network implementation. For example, CSI report, SRS, SPS and CG can be configured in cell DTX/DRX active period. Compared to the degradation of UE performance caused by barring legacy UE, UE camping on the cell adopting cell DTX/DRX techniques still can work normally. Similarly, when cell DTX/DRX is deactivated, in order to activate the cell DTX/DRX through barring legacy UE, this motivation seems to be non-existent and the performance of legacy UE cannot be guaranteed. At this time, the network should not activate the cell DTX/DRX. Thus, it is not necessary to prevent legacy UE camping on cells adopting the cell DTX/DRX techniques for the time being at least.

- The Rel-18 NES techniques are designed for RRC_CONNECTED UEs except for the techniques of inter-node beam activation and paging enhancement. But there is no impact for legacy idle/inactive UEs camping on cell adopting the Rel-18 NES techniques.

- For legacy UEs in RRC_CONNECTED state, it can work normally on the cell adopting cell DTX/DRX techniques for the time being at least, whether there is a need to prevent legacy UE camping on cell capable of the cell DTX/DRX techniques depends on the detailed discussion.

Based on the above observation, legacy UEs can camp on the cell adopting the Rel-18 NES techniques. Whether there is a need to prevent legacy UE camping on cell adopting the cell DTX/DRX techniques which depends on the further detailed discussion. Therefore, based on this and we propose the following:

- RAN2 to postpone the discussion of cell (re-)selection until the details for cell DTX/DRX techniques are further specified.

Conclusion

Based on the discussion, we have the following observation and proposals:

- The Rel-18 NES techniques are designed for RRC_CONNECTED UEs except for the techniques of inter-node beam activation and paging enhancement. But there is no impact for legacy idle/inactive UEs camping on cell adopting the Rel-18 NES techniques.

- For legacy UEs in RRC_CONNECTED state, it can work normally on the cell adopting cell DTX/DRX techniques for the time being at least, whether there is a need to prevent legacy UE camping on cell capable of the cell DTX/DRX techniques depends on the detailed discussion.

- RAN2 to postpone the discussion of cell (re-)selection until the details for cell DTX/DRX techniques are further specified.

References

[1] R2-2301903, Report from Session for on NES, UAV Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning.

[2] RP-223540, Network energy savings for NR, Huawei.

- Appendix

  - RAN1#121bis-e Agreements

Agreement From RAN1 point of view, Rel-18 UE supporting cell DRX is not expected to transmit the following signals/channels to the gNB during non-active periods of cell DRX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - FFS: SRS for positioning - FFS: - HARQ feedback for SPS PDSCH - FFS whether there will be exception case(s) for UE transmitting listed signals/channels during non-active periods of DRX - FFS Whether the listed signals/channels can be configurable by gNB - FFS: Whether the same or different UE behavior is applicable with or without C-DRX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period

  - RAN2#121 Agreements

Agreements 1. There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs 2. Rel-18 NES capable CONNECTED UE(s) can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX (i.e., same behavior for cell DTX and cell DRX). No further enhancements for CBRA and CFRA will be pursued. 3. Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell. FFS whether we have DTX UE specific inactivity timer . FFS on configuration signalling and stage 3. 4. Confirm study item agreement that we can have separate DTX and DRX configuration. We will focus on designing DTX/DRX for at least single configuration. FFS whether multiple configuration of cell DTX or DRX will be supported.

  - RAN2#121bis-e Agreements

Agreements - As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period - As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods - As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic - (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH. For the FFS case there may be some exceptions. - The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. FFS how to deal with any exceptions (e.g. SR if agreed and RACH). FFS how to deal with retransmissions