3GPP TSG-RAN WG2 Meeting #122		R2-2305430

Incheon, Korea, May. 22th- May. 26th 2023

Agenda Item:	7.8.3

Source: 	vivo

Title:         	Discussion on flight path reporting

Document for: 	Discussion and Decision

- Introduction

In RAN2#121bis meeting, RAN2 has reached the following agreements related to the flight path reporting for NR UAV [1]:

Agreements:

- Flightpath update indication in UAI is configurable by the network

- Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE

- Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307]

- As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s)

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication.  FFS if further differentiation is needed if we decide to have delta signaling

In this contribution, we will further discuss the remaining issues for NR UAV flight path reporting.

- Discussion

Triggering for updating flight path:

Regarding to the below FFS:

- As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s)

When the UAV flight path changes, the UE needs to notify the network. If the network is interested in the updated flight path information, the network can request the UE to report its latest flight path via UE information request/response message. However, there are concerns that the UE may trigger the flight path update indication frequently, which leads to signaling overhead while the network may not be interested in the updated information. Therefore, In RAN2#121bis, it was agreed to support a simple network-controlled mechanism for triggering the flight path update indication.

UAV may change their flight path due to a big obstacle appeared in the way or the change of destination/flight commission. Basically, the flight path change in these cases can be significant, and needs to be indicated to the network. If the UAV changes its flight path due to a small obstacle in the way, the flight change could be small and the change is meaningless at the network side, so this update indication should be avoided. To achieve this, a threshold for waypoint and timestamp can be considered. For example, when the waypoints have changed more than the threshold, the flight path update indication shall be triggered. And this threshold should be configurable by the network. Thus, we have the following proposal:

Proposal 1: if any waypoint/timestamp has changed more than the configured threshold, the UE triggers flightpath update indication.

flightpath update indication:

Regarding to the below FFS:

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication.  FFS if further differentiation is needed if we decide to have delta signaling

Basically, if the UAV have a new plan, the entire path will be re-generated. From this point, delta reporting has few benefits. In addition, currently, delta signaling is not supported for UAI, which requires the network to store this information. And there is no reason to introduce the complexity just for UAV. Therefore, as a simple solution, there is no need to introduce delta signaling for flight path update. Thus, a single indication for both initial and updated flightpath availability is sufficient. Thus, the below proposal is given:

Proposal 2: Delta signaling for flight path report updating is not supported.

Proposal 3: Confirm a single indication is used for both initial and updated flightpath availability.

- Conclusion

In this paper, the following proposal are given:

Proposal 1: if any waypoint/timestamp has changed more than the configured threshold, the UE triggers flightpath update indication.

Proposal 2: Delta signaling for flight path report updating is not supported.

Proposal 3: Confirm a single indication is used for both initial and updated flightpath availability.

- References

[1] R2-2304203 Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning.