3GPP TSG-RAN WG2 #122	R2-2305532
Incheon, Korea, May 2023

Agenda Item:	7.5.2

Source:	OPPO

Title:	Discussion on XR awareness

Document for:	Discussion, Decision

Introduction

This paper will discuss XR awareness.

Discussion

XR traffic assistance information

In Rel-16 URLLC, TSCAI is introduced to describe TSC traffic characteristics. The TSCAI is provided to the RAN from SMF per QoS flow, which contains the flow direction, periodicity and burst arrival time. Knowing that the TSC traffic characteristics are useful for RAN on more efficient scheduling for the QoS flows that have periodic and deterministic traffic characteristics.

Table 1: TSC Assistance Information (TSCAI) in TS 23.501

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two data bursts. |
| Burst Arrival Time (optional) | The latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow direction). |
| Survival Time (optional) | Survival Time, as defined in TS 22.261 [2], refers to the time period an application can survive without any data burst. |

- TSCAI is designed to provide the gNB with the TSC characteristics which contain direction, periodicity and burst arrival time.

Similar to TSC, XR is also periodic traffic with latency requirements. To guarantee efficient scheduling for the XR service, it would be good to let the gNB know XR characteristics.

According to SA2’s progress, NG-RAN may be provided with the UL and/or DL Periodicity by the SMF, which has been reflected in TS 23.501.

Although the UL periodicity is designed to provide for DRX, it can be easily extended for the CG configuration. We assume the gNB can obtain UL periodicity from the CN for capacity enhancement and power saving, therefore, the UL periodicity report by the UE to the gNB seems redundant.

- SA2 already specifies the UL and/or DL Periodicity obtained by the gNB from the SMF.

No need to report the UL periodicity from the UE to the gNB.

Taking TSCAI as a reference and targeting efficient uplink scheduling for the XR service, besides the UL periodicity, we think another important parameter is burst arrival time (i.e. BAT).

In our understanding, BAT is very useful for the RAN to configure a CG configuration that well matches the XR service characteristics/requirements. Otherwise, the transmission latency of XR packets would be introduced inevitably. Since the UL packets are generated by the UE application layer, it is possible for a UE AS layer to obtain BAT through its internal interaction. Since the gNB knows the XR periodicity, once the UE AS layer may know the arrival time of the first XR packet and then reports it to the gNB, the gNB can know the arrival time of subsequent XR packets in advance. As a result, the gNB can use such information to configure CG occasions for subsequent traffic transmission.

One may suggest using UL Jitter instead of UL BAT. To us, UL BAT has its own role to let the gNB know the reference point of the first packet arrival, while UL jitter is just a relative value range to UL BAT. Without this reference point, it is hard for the gNB to allocate the CG resources for subsequent traffic transmissions.

RAN2 assumes the UE to provide the gNB with the burst arrival time to configure proper CG resources for the XR service.

In RAN2#121bis, RAN2 concludes to report UL jitter and leaves some open issues.

- 3.	UE can report jitter information associated to UL XR traffic. How UE derives this jitter is left up to implementation (similarly as it is captured by SA2 for the jitter associated with the periodicity in DL. FFS what exactly is reported to the RAN (aim to have similar information as for DL). FFS on UL traffic data arrival reporting.

In our view, the UE can report the uplink jitter range which reflects long-term measurement results, due to the reason below. Also, UAI is the most straightforward way to report the UL jitter.

- For UL jitter, RAN2 aims to have similar information as for DL. As approved in CR S2-2306201, the CN provides the jitter range for DL.

At reception of  measured Traffic Parameter(s) from the UPF in the N4 Session Level Report, the SMF includes the N6 jitter range together with the associated DL periodicity in the TSCAI and forwards it to the NG-RAN in an NGAP message, see clause 5.27.2.

- RAN1 assumes the average jitter value is always zero. Thus, reporting the average jitter value seems meaningless.

- It is impractical for a UE to report real-time jitter considering the signalling overhead.

The UE uses UAI to report the UL jitter range associated with a QoS flow/DRB of the XR service.

PDU set information

In the previous RAN2 meetings, RAN2 concludes the below for PDU set handling in both UL and DL.

- 1: From RAN2 viewpoint, the following information would be useful for PDU set handling in UL and DL:

- Semi-static information (from CN to RAN): At least PSER and PSDB.

- Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed, including means to determine at least PDU set boundaries.

Also, SA2 agrees that the below DL PDU set information is identified by UPF and informed to RAN via the GTP-U header of the user plane packet, which has been reflected in TS 23.501.

In our understanding, the information highlighted in yellow (i.e. PDU set association information) is used to identify which PDU belongs to which PDU set, and the information highlighted in green is used to identify the PDU set type for individual PDU set handling. With such DL PDU set information, the gNB can easily identify each individual PDU set and its type as well as have a proper PDU set operation by its implementation.

- The DL PDU set information is provided to NG-RAN by SMF/UPF for the integrated/differentiated PDU set handling.

It depends on the gNB implementation/configuration on how to use DL PDU set information.

Similar to DL, UL PDU set operation is also based on identifying the association between PDUs as well as the importance of a PDU set. In our view, the most straightforward way is to reuse the parameters designed for DL, i.e. the UE AS obtains the PDU set information (e.g. PDU set sequence number, Indication of end PDU of the PDU set, PDU sequence number within a PDU set, PSI, etc) by the interaction from the upper layer and use such PDU set information for UL PDU set handling.

For the uplink, the UE AS layer knows the association between PDUs as well as PSI of a PDU set based on the PDU set information interacted from the UE’s higher layer. RAN2 reuse the parameters designed for DL PDU Set identification for UL PDU set identification, including e.g. PDU set sequence number, Indication of end PDU of the PDU set, PDU sequence number within a PDU set, PSI, etc.

LS on the N6 PDU set identification

In the SA4 LS(R2-2304493), SA4 requests the RAN2 feedback on the feasibility and value of having additional signalling bits related to the End of Burst and inter-burst time within Rel-18. In the SA4’s assumption, the indication of End of Burst is provided via UP packets as well as the inter-burst time is useful for power saving and is delivered to the 5G system dynamically.

On the indication of End of Burst, we understand that SA2 has studied and confirmed the necessity/value of providing such information as well as the details have already been specified in TS 23.501 and known by RAN2. Thus, RAN2 can simply confirm the feasibility/value of having additional signalling bits related to the End of Burst within the Rel-18 timeline.

On the indication of the inter-burst time, we prefer negative feedback on the feasibility and value due to the reasons below, 1) The definition and the application scenario of the inter-burst time are unclear. For example, we have no idea whether the inter-burst time is designed for periodic traffic as well as whether it can be represented by the combination of periodicity and N6 jitter. 2) SA2#157 is the last SA2 meeting for the normative work of R18 XRM but the inter-burst time hasn’t been discussed in SA2/RAN2 before. It seems weird if RAN2 supports the inter-burst time for UL but SA2 does not support the inter-burst time for DL. Note that video service exists in both UL and DL as well as the UE power to be saved depends on the data status information of both UL and DL.

A reply LS is sent to SA4 with the following 1) Confirm the feasibility and value of the End of Burst, 2) Indicate RAN2 cannot support the inter-burst time.

Conclusion

We have the following observations:

Observation 1	TSCAI is designed to provide the gNB with the TSC characteristics which contain direction, periodicity and burst arrival time.

Observation 2	SA2 already specifies the UL and/or DL Periodicity obtained by the gNB from the SMF.

Observation 3	The DL PDU set information is provided to NG-RAN by SMF/UPF for the integrated/differentiated PDU set handling.

We have the following proposals:

Proposal 1	No need to report the UL periodicity from the UE to the gNB.

Proposal 2	RAN2 assumes the UE to provide the gNB with the burst arrival time to configure proper CG resources for the XR service.

Proposal 3	The UE uses UAI to report the UL jitter range associated with a QoS flow/DRB of the XR service.

Proposal 4	It depends on the gNB implementation/configuration on how to use DL PDU set information.

Proposal 5	For the uplink, the UE AS layer knows the association between PDUs as well as PSI of a PDU set based on the PDU set information interacted from the UE’s higher layer. RAN2 reuse the parameters designed for DL PDU Set identification for UL PDU set identification, including e.g. PDU set sequence number, Indication of end PDU of the PDU set, PDU sequence number within a PDU set, PSI, etc.

Proposal 6	A reply LS is sent to SA4 with the following 1) Confirm the feasibility and value of the End of Burst, 2) Indicate RAN2 cannot support the inter-burst time.

Reference

[1] RP-230786 Updated WID on XR Enhancements for NR, Nokia, Qualcomm

[2] TR 23.700-60 Study on XR (Extended Reality) and media services

[3] R2-2302309 TR 38.835 v1.0.2, Nokia (Rapporteur)

[4] TS 23.501 v18.1.0