3GPP TSG-RAN WG2 Meeting #122                      	                                    R2-2305627

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.23.2

Source: 	CMCC

Title:  	Discussion on the network timing synchronization status monitoring

WID/SID:	         NR_TRS_URLLC

Document for:	Discussion and Decision

Introduction

In the RAN#121bis meeting, the network timing synchronization status and reporting issue have been discussed and the following items have been achieved:

Agreements:

- RAN2 to confirm that there is no need to support “group of cells across gNBs” for scope of the report ID.  The scenario, as per RAN3 will be supported - different cells within gNB that are served by different DUs and different clock quality is possible.

- Confirm the following SA2’s conclusions regarding NG-RAN procedures on 5G clock quality information:

a.	RRC_INACTIVE/IDLE UE to be informed of timing synchronization status via an event ID in SIB9 by comparison with the one maintained locally.

b.	RRC_CONNECTED UE to be informed of 5G clock quality information via dedicated RRC message

- Postpone the UAC until CT1’s feedback on UAC framework to randomize UE access to network in the time domain

- 5G Clock Quality information is carried in DLInformationTransfer message.

- FFS if update of event ID is informed to UE by SI modification procedure

- For Idle/Inactive mode RAN2 assumes that the NAS layer triggers the RRC connection procedure based on inputs from AS.

- The AS layer of the UE determines if there a change of event ID and it indicates a change in the RAN timing synchronization status to NAS layers.  FFS to double check for connected mode and inactive.

Hence, in this contribution, we will discuss the issues on the network timing synchronization status monitoring.

Discussion

In the TSN network, it is required in [1] that RAN as part of 5GS needs to detect the network timing synchronization status and report the network timing synchronization status to the UE. In SA2’s study as below, the gNB broadcasts the SIB9 with a reference report ID to inform the UE the change of the network timing synchronization status.

- Informing UEs in RRC_Inactive/Idle state about a change of the RAN clock quality information: - The gNB includes in SIB9 a reference report ID as a notification for the UEs reading the SIB9 that there is new clock quality information available. The UE compares the reference report ID with locally stored reference report ID to determine if it had retrieved the last available clock quality information already. - The reference report ID consists of the scope of the report ID and an Event ID (an integer). Scope may either identify a group of cells within a single gNB or a group of cells across gNBs. The latter would reduce the amount of signalling even further since then UEs that move to another gNB would not need to retrieve the clock quality details.

As described above, a reference report ID consisting of the scope of the report ID and the Event ID should be included in SIB9 to inform UE on the network timing synchronization status. Since only a group of cells within a single gNB is supported in this version, the gNB ID could be used as the scope of the report ID.  The UE determines the status of network timing synchronization by checking the change of the reference report ID. Therefore, either the change on the gNB ID or the Event ID, the UE think the change has happened in the network timing synchronization status.

Proposal 1: The UE determines the status of network timing synchronization by checking the change of the reference report ID. Therefore, either the change on the gNB ID and/or the Event ID, the UE determines the change has happened in the network timing synchronization status.

In the last meeting, it was concluded as below that the AS layer of the RRC_IDLE UE determines the change of the networking timing synchronization status and indicates a change to NAS layer.

1. The AS layer of the UE determines if there a change of event ID and it indicates a change in the RAN timing synchronization status to NAS layers. FFS to double check for connected mode and inactive.

There is still open issue for the procedure used in the RRC_CONNECTED or RRC_INACTIVE mode when UE finds the RAN clock quality information changes and tries to reconnect to the gNB. As described in [1] below, the UE triggers the RRC connection resume procedure in RRC_INACTIVE mode when UE determines that the RAN clock quality information changes.

- If the UE has determined that the RAN clock quality information has changed and the UE has been requested by the TSCTSF to connect to the network in the case that the RAN clock quality information changes, the UE performs a registration (if the UE is in RRC_IDLE) or the UE Triggered Connection Resume in RRC Inactive procedure (if the UE is in RRC_INACTIVE).

And in [2] as below, the RAN node directly sends the clock quality information, e.g. clock quality metrics or acceptable/not acceptable indication, to UE by using DLInformationTransfer message as agreed last meeting.

When determining the clock quality metrics for a UE and when determining whether clock quality is acceptable or not acceptable for a UE, NG-RAN considers whether propagation delay compensation is performed while UE capabilities and internal inaccuracies are assumed to be budgeted by the client network operator when agreeing the required clock accuracy with the 5G network operator. To provision clock quality information to the UEs, the NG-RAN uses unicast RRC signalling: - For UEs in RRC Connected state, the NG-RAN uses unicast RRC signalling. - The UE that is not in RRC_CONNECTED state may establish or resume the RRC connection to receive the clock quality information from the NG-RAN.

The AS could trigger the RRC connection resume procedure with specific cause to notify the gNB for trigger of the network timing synchronization states change procedure. Therefore the AS could trigger the process without notifying the NAS layer in RRC_INACTIVE state. A new resume cause could be introduced in RRCResumeRequest-IEs as a notification for the gNB to provide the clock quality information, e.g. clock quality metrics or acceptable/not acceptable indication.

Proposal 2: The AS could trigger the process without notifying the NAS layer in RRC_INACTIVE state. A new resume cause could be introduced in RRCResumeRequest-IEs as a notification for the gNB to provide the clock quality information, e.g. clock quality metrics or acceptable/not acceptable indication.

- Conclusion

In this contribution, the discussion on the network timing synchronization status monitoring is provided.

Proposal 1: The UE determines the status of network timing synchronization by checking the change of the reference report ID. Therefore, either the change on the gNB ID and/or the Event ID, the UE determines the change has happened in the network timing synchronization status.

Proposal 2: The AS could trigger the process without notifying the NAS layer in RRC_INACTIVE state. A new resume cause could be introduced in RRCResumeRequest-IEs as a notification for the gNB to provide the clock quality information, e.g. clock quality metrics or acceptable/not acceptable indication.

Reference

[1] TR 23.700-25 V18.1.0 Study on timing resiliency and TSC and URLLC enhancements (2023-03)

[2] TR 23.501 V18.1.0, System architecture for the 5G System (5GS); (Release 18), 2023-03