3GPP TSG-RAN WG2 Meeting #123	R2-2307102

Toulouse, France, August 21-25, 2023

Agenda Item:	7.7.4.1.2

Source: 	vivo

Title:  	Further Discussion on NTN-NTN Mobility for Earth-moving Cell

Document for:	Discussion and Decision

Introduction

Regarding cell reselection enhancement for earth-moving cell, agreements reached in RAN2#122 [1] are shown as follows:

- Re-use epochTime-r17 in ntn-Config IE to provide the time reference for an Earth moving cell reference location.

- Re-use t-Service-r17 format for the IE used to trigger UE neighbour cell measurements prior to cell replacement due to feeder link switch. FFS whether we reuse exactly the same IE name as in R17 (updating the field description) or a new one

Agreements:

- Location-based cell reselection criteria are not pursued in R18.

Furthermore, regarding the cell’s reference location acquisition for earth-moving cell for CHO, an agreement was reached in RAN2#121 [2] as follows, but the FFS in the agreements and further details were not discussed in subsequent meetings.

- For location-based CHO for earth-moving cells we follow the solution being investigated for cell reselection to allow the UE to derive the serving cell’s reference locations as the cells move. FFS whether the same mechanism can also be used for the candidate cell’s reference location

In this contribution, we will provide our analyses and proposals to resolve FFS highlighted in yellow above for earth-moving cell and discuss the details of estimating the reference location for location-based CHO.

Discussion

Time-based cell reselection for earth-moving cell

RAN2 has agreed an IE is used to trigger UE neighbour cell measurements prior to cell replacement due to feeder link switch in earth-moving system. One remaining issue for time-based cell reselection for earth-moving cell is to discuss whether we reuse exactly the same IE name (i.e., t-Service-r17) as in R17 (updating the field description) or a new one, the related agreement is shown as follows.

- Re-use t-Service-r17 format for the IE used to trigger UE neighbour cell measurements prior to cell replacement due to feeder link switch. FFS whether we reuse exactly the same IE name as in R17 (updating the field description) or a new one

From our perspective, these two candidate solutions are acceptable, but the latter (i.e., introduce a new parameter to trigger UE neighbour cell measurements prior to cell replacement due to feeder link switch in earth-moving system) is preferred. Specifically, if the t-Service-r17 is reused in moving cell, the R17 legacy UE which supports time-based RRM measurements of neighbour cells in NTN quasi-earth fixed system will perform time--based RRM measurements in a moving cell. Even though this will not result in bad UE experience, after all, it allows UE to do something beyond the UE capability. Hence, a new IE is preferred to make the spec more normative. Furthermore, in the case that a new IE is used, to complete the feature of time-based measurement initiation in earth-moving system, similar to time-based measurement initiation in quasi-earth fixed system, RAN2 needs to specify that UE should start measurements on neighbour cells before the time of feeder link switching in earth-moving system.

Proposal 1: A new IE is introduced to indicate the time of feeder link switching in earth-moving system.

Proposal 2: RAN2 to specify that UE shall start measurements on neighbour cells before the time of feeder link switching in earth-moving system. The exact time of measurement initiation is up to UE implementation.

Reference location acquisition for CHO

In the previous meeting, RAN2 agreed that for earth-moving cell reselection, the UE can derive the trajectory of serving cell based on ephemeris, epochTime, and reference location at epochTime for the serving cell. Besides, RAN2 agreed the same mechanism can be adopted for location-based CHO for earth-moving cells, i.e., if the serving cell is an earth-moving cell, UE can derive the serving cell’s reference location for location-based CHO. The related agreement is excerpted below.

- For location-based CHO for earth-moving cells we follow the solution being investigated for cell reselection to allow the UE to derive the serving cell’s reference locations as the cells move. FFS whether the same mechanism can also be used for the candidate cell’s reference location

Based on the agreement for location-based CHO above, whether the same mechanism can also be used for the candidate cell’s reference location should be discussed. For location-based CHO, UE determines whether CondEvent D1 is fulfilled based on the distance between UE location and the serving cell reference location and the distance between UE location and each candidate cell reference location. If the candidate cell is an earth-moving cell, the reference location of the cell is time-varying, the legacy mechanism (i,e., the NW provides a fixed reference location through CHO configuration) is not applicable. So for location-based CHO, the same mechanism should be used for the candidate cell’s reference location, i.e., UE should derive the candidate cell’s reference location if the cell is an earth-moving cell.

Proposal 3: For location-based CHO, UE derives the candidate cell’s reference location if the candidate cell is an earth-moving cell.

Take the mechanism of deriving serving cell reference location for cell reselection as a baseline, three parameters are needed to derive the reference location for the serving cell and candidate cells, that is ephemeris, epochTime, and reference location at epochTime for the corresponding cell. Next, in order to complete the feature of estimating the reference location for location-based CHO, how UE obtains these three parameters is needed to be discussed. Furthermore, the RRC signaling of existing condEventD1 is excerpted as follows.

condEventD1-r17                  SEQUENCE {

distanceThreshFromReference1-r17 INTEGER(0.. 65525),

distanceThreshFromReference2-r17 INTEGER(0.. 65525),

referenceLocation1-r17           ReferenceLocation-r17,

referenceLocation2-r17           ReferenceLocation-r17,

hysteresisLocation-r17           HysteresisLocation-r17,

timeToTrigger-r17                TimeToTrigger

},

In our understanding, there are three candidate solutions to solve the above issues listed as follows.

- Option 1: ephemeris and epochTime for each earth-moving cell (including the serving cell and candidate cell) are included in the CHO configuration; the existing referenceLocation1 is the reference location at epochTime for the serving cell and referenceLocation2 is the reference location at epochTime for the corresponding candidate cell, where the epochTime refers to the epochTime in the CHO configuration.

- Option 2: ephemeris, epochTime and reference location at epochTime for each earth-moving cell (including the serving cell and candidate cell) are included in the SIB19 of the serving cell; UE should ignore the reference location in CHO configuration if the corresponding cell is an earth-moving cell.

- Option 3: ephemeris, epochTime for each earth-moving cell (including the serving cell and candidate cell) are included in the SIB19 of the serving cell; the existing referenceLocation1 is the reference location at epochTime for the serving cell and referenceLocation2 is the reference location at epochTime for the corresponding candidate cell, where the epochTime refers to the epochTime in SIB19 of the serving cell.

Furthermore, according to the existing location-based CHO mechanism, UE directly uses the reference location in the CHO configuration (i.e., referenceLocation1 and referenceLocation2) to determine whether condEvent D1 is fulfilled. UE does not estimate the reference location of the serving cell and/or candidate cell without being indicated by the NW. For option 1, UE can implicitly know that it needs to derive the reference location for the cell if ephemeris and epochTime for the corresponding cell are included in the CHO configuration. For option 2 and option 3, since the signaling of CHO configuration is not affected, an explicit indication is needed to indicate that UE should derive the reference location for the corresponding cell.

In summary, to draw a full picture of reference location acquisition for location-based CHO, the following proposal is raised.

Proposal 4: For location-based CHO for earth-moving cells, RAN2 to discuss how UE obtains ephemeris, epochTime and reference location at epochTime for each earth-moving cell.

Conclusions

Based on the analyses given above, we have the following observations and proposals:

Proposal 1: A new IE is introduced to indicate the time of feeder link switching in earth-moving system.

Proposal 2: RAN2 to specify that UE shall start measurements on neighbour cells before the time of feeder link switching in earth-moving system. The exact time of measurement initiation is up to UE implementation.

Proposal 3: For location-based CHO, UE derives the candidate cell’s reference location if the candidate cell is an earth-moving cell.

Proposal 4: For location-based CHO for earth-moving cells, RAN2 to discuss how UE obtains ephemeris, epochTime and reference location at epochTime for each earth-moving cell.

Reference

- Chairman_notes of 3GPP TSG RAN WG2 meeting #122

- Chairman_notes of 3GPP TSG RAN WG2 meeting #121bis-e