3GPP TSG-RAN WG2 Meeting #123	R2-2307104

Toulouse, France, August 21-25, 2023

Agenda Item:	7.7.4.2

Source: 	vivo

Title:  	Further discussion on service link switching with unchanged PCI

Document for:	Discussion and Decision

Introduction

In RAN2#122 meeting, the following agreements on the topic of service link switch with unchanged PCI were achieved [1]:

Agreements:

- t-Service in SIB19 can also be interpreted by Rel-18 UE in Connected mode to know that a satellite change or feeder link change happens

- In hard switch unchanged PCI scenario (i.e. no handover), the UE needs to know the time the UE attempts to re-synchronize. (FFS whether a new “t-Start” / a t-gap is needed or whether t-Service can be reused (i.e. no other IE) if the gap is very short/zero).

This contribution will continue to discuss the remaining issues on this topic and give our proposals.

Discussion

In the last meeting, RAN2 agreed UE needs to know when to perform re-synchronize due to hard satellite switching without PCI change, but how to indicate the time of re-synchronizing is FFS. This issue is also discussed in email [Post122][114] [2]. There are three options to indicate the re-synchronize time listed as follows.

- Option 1: Introduce a new “t-Start”;

- Option 2: Introduce a new t-gap;

- Option 3: reuse t-Service (i.e. no other IE).

Based on the latest summary of the [Post122][114], the recommended proposal is as following:

Proposal 1: the unchanged PCI mechanism can be applied to both the continuous coverage where the coverage gap can be ignored and the coverage gap is not zero. FFS the indication is t-gap or t-start.

We share the same view that option 3 should first be excluded since it is more suitable for the scenario where the gap between the source satellite stopping time and upcoming satellite serving time is very short and negligible. Option 2 requires that NW broadcast both t-service and t-gap for UE to determine the exact time of performing UL re-synchronization. However, in R17 NTN, t-service is introduced to assist UE in performing cell reselection in quasi-earth-fixed cell. This parameter also may be used to assist UE in performing cell reselection in earth-moving cell in R18. We do not think it is a good idea to couple several features together. From this perspective, option 1 is preferable, which make the different feature work independently.

Proposal 1: A new “t-Start” should be introduced to indicate the re-synchronize time.

UE will encounter DL and UL synchronization loss when service link switching occurs since the propagation delay of the source satellite and the upcoming satellite will be different. From UE perspective, after the procedure is triggered, UE shall obtain DL synchronization on the beam of the target satellite. For the UE in RRC_CONNECTED, UE additionally needs to get UL synchronized with the target satellite on the beam of the new service link. This means that if it reaches re-synchronize time (i.e., t-Start), the UE in RRC_CONNECTED shall initiate random access procedure and TAR procedure.

Proposal 2: At the re-synchronize time (i.e, “t-Start” in Proposal 1), the UE in RRC_CONNECTED initiates random access procedure and TAR procedure to get UL synchronized with the target satellite on the new service link.

Regarding how the benefits come with this unchanged PCI mechanism, the main motivation to keep the PCI unchanged is to avoid the L3 mobility and thus avoid unnecessary reconfiguration of those unchanged UE-specific parameters, because now it is the same gNB on the ground that is connected with both the old satellite and the new satellite. Then, those UE-specific configurations during L3 mobility can be saved, and the UE simply reuses the configurations prior to the service link switching.

Proposal 3: After switching to the new satellite with the unchanged PCI, the UE continues using the configurations prior to the service link switching.

Next, to complete the feature of hard service link switching with unchanged PCI, how UE determines whether this procedure fails should be discussed. We think the solution to determining the failure of synchronization due to hard service link switching with unchanged PCI can be considered in two directions. On the one hand, since the UE performs uplink synchronization through the RACH, the failure of synchronization due to hard service link switching with unchanged PCI can be determined by the existing way of determining RACH failure, i.e., MAC layer indicates layer random access problem to RRC layer in the case of the maximum number of preamble transmissions is reached, then RRC layer consider radio link failure to be detected. On the other hand, in service link switching without feeder link switching scenario, the motivation of unchanged PCI is to replace the handover procedure to save the signaling overhead, so a similar way of determining handover failure can be considered, i.e. a timer can be used for determining the failure of synchronization due to hard service link switching with unchanged PCI. To make it more intuitive, the two ways for determining the failure of synchronization due to hard service link switching with unchanged PCI are listed as follows.

- Option 1: Reuse the existing way of determining RACH failure;

- Option 2: Timer-based mechanism similar to the way of determining handover failures.

Upon service link switching occurs, all the UE in the cell needs to re-synchronize to the new satellite. Due to the limited resources on the network side, there is a high probability that one RACH process will fail for UEs at the cell edge or UEs with poor coverage. If option 1 is adopted, these UEs have a high probability of performing RRC connection re-establishment in the cell where the failure occurred. In order to save signaling overhead to a greater extent, UE should be allowed to perform random access procedures more times to get synchronization with the target satellite on the new service link, so option 2 is more suitable to be adopted as the way of determining the failure of synchronization due to hard service link switching with unchanged PCI. More specifically, UE should start a timer at “t-Start”, if the timer expires, UE considers synchronization procedure due to hard service link switching with unchanged PCI fails.

Proposal 4: Timer-based mechanism can be used to determine the failure of synchronization due to hard service link switching with unchanged PCI.

In the RAN2#121bis meeting, RAN2 left an FFS below to discuss if the combination of RACH-less Handover and the unchanged PCI approach can be supported [3].

FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported

Basically, service link switching without PCI changing is introduced to avoid L3 mobility while RACH-less HO is introduced to reduce signalling overhead during the L3 mobility procedure. Since the motivation of the two schemes is not fully compatible, we do not think there is any motivation to combine these two mechanisms. Technically speaking, for the “unchanged PCI” scenario, UE in RRC_CONNECTED only needs to perform UL synchronization when service link switching between two satellites occurs. There is no need to involve any L3 signaling, which is different from RACH-less mechanism (i.e., UE still transmits the RRCReconfigurationComplete message to the target gNB).

Proposal 5: Do not support the combination of RACH-Less HO and unchanged PCI mechanism.

Conclusions

This contribution will discuss in Rel-18 NTN and give the following observations and proposals.

Proposal 1: A new “t-Start” should be introduced to indicate the re-synchronize time.

Proposal 2: At the re-synchronize time (i.e, “t-Start” in Proposal 1), the UE in RRC_CONNECTED initiates random access procedure and TAR procedure to get UL synchronized with the target satellite on the new service link.

Proposal 3: After switching to the new satellite with the unchanged PCI, the UE continues using the configurations prior to the service link switching.

Proposal 4: Timer-based mechanism can be used to determine the failure of synchronization due to hard service link switching with unchanged PCI.

Proposal 5: Do not support the combination of RACH-Less HO and unchanged PCI mechanism.

Reference

- Chairman_notes of 3GPP TSG RAN WG2 meeting #122

- Report of [Post122][114][NR NTN Enh] Unchanged PCI (CMCC)

- Chairman_notes of 3GPP TSG RAN WG2 meeting #121bis-e