3GPP TSG-RAN WG2 Meeting #123	 R2-2307120

Toulouse, France, 21st – 25th August, 2023

Agenda Item:		7.5.4.3

Source:			Huawei, HiSilicon

Title:			Discussion on RAN2 impacts of Multi-PUSCH CG

Document for:	Discussion and Decision

Introduction

RAN1 has rather extensively discussed multi-PUSCH CG design in their previous meetings and their agreements thus far are summarized in the Annex.

In this paper, we discuss potential impacts of Multi-PUSCH CG from RAN2's perspective, including how the MAC entity determines unused CG occasions.

Discussion

  - HARQ process ID

For the design of multi-PUSCH CG, it has been agreed in RAN1 that, for the multiple UL grants within a single CG period, the HARQ process of the first UL grant is determined by the legacy formula and those for the remaining CG grants are obtained by incrementing HARQ PID of the preceding CG PUSCH[1].

Agreement From RAN1 perspective, for determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH: - The HARQ process ID for the first configured PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable - HARQ Process ID = [X*floor( (CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes - HARQ Process ID = [X*floor((CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2 - X= the number of configured PUSCHs in the CG period - The HARQ process ID of the remaining configured and valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable.

Based on the above agreement, since the HARQ PID determination is specified in RAN2 MAC spec, RAN2 should confirm on the agreement above and make proper changes in the MAC spec.

Observation: RAN1 has reached agreement on the HARQ PID calculation for multi-PUSCH CG:

- HARQ process ID for the first configured PUSCH in a period is determined based on the legacy CG procedure

- The HARQ process ID of the remaining configured and valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2)

However, we notice that, when harq-ProcID-Offset2 is configured, the HARQ PID of the remaining configured and valid CG PUSCHs in the period should be moduled nrofHARQ-Processes rather than (nrofHARQ-Processes + haq-ProcID-Offset2), after incrementing the HARQ PID of the preceding PUSCH in the period by one.

Proposal1: Regarding the HARQ process ID for multi-PUSCH CG, RAN2 to confirm that

- the HARQ process ID of the first UL grant in the CG period is determined by the module operation with nrofHARQ-Processes and then add harq-ProcID-Offset2, if configured

- the HARQ process ID of the remaining configured and valid CG PUSCH is determined by incrementing the HARQ process ID of the preceding PUSCH in the period

Due to characteristic of non-integer periodicity, more than one CG configurations are required to match the XR traffic pattern. In the unlicensed band, the HARQ PID cannot be shared between multiple CG configurations, as per legacy specification described in TS38.321:

NOTE 5: If cg-RetransmissionTimer is not configured, a HARQ process is not shared between different configured grant configurations in the same BWP.

To support XR video transmission usually requires multi-PUSCH and hence multiple HARQ PIDs for a CG configuration. While this might be problematic if there are multiple CG configurations. When the number of CG configurations required is not large, e.g. 3 CG configurations for 60Hz XR video, the number of HARQ PID may be sufficient. However, with the growing number of CG configurations, e.g. 9 configurations for 90Hz XR video, there is an inevitable scarcity in the number of HARQ PIDs. For example, assuming 4 CG occasions within a CG period, at least 36 HARQ PIDs are needed to support 90Hz video transmission, which is beyond the maximum number of 16 HARQ PIDs (32 in NTN) supported.

Observation:  There is a scarcity of HARQ PIDs, when multiple CG configurations with multi-PUSCH are configured for XR video traffic.

To enable the multi-PUSCH CG configurations to support XR uplink video transmission with more frame rates, it is suggested to cope with the issue of scarce maximum number of HARQ PIDs.

Proposal2: RAN2 to address the issue of scarcity of HARQ PIDs when multi-PUSCH CGs are configured.

  - UTO-UCI

Since the size of the data burst is Gaussian distributed, it is possible that the size of the data burst is larger or smaller than the size of UL grant provided by the multi-PUSCH CG. When the size of the burst is larger, the legacy BSR procedure will be reused. However, when the size of the burst is smaller than the size of multi-PUSCH CG, it is worth discussing how to efficiently utilize the allocated CG resources. Thus, the UTO-UCI is motivated as agreed by RAN1.

MAC layer modeling

Although RAN1 has reached agreements on designing UTO-UCI, there remains issue about how to determine the content of the UTO-UCI or how to determine whether the CG occasions are unused. As per legacy specification, if there is no data to be transmitted, MAC entity is allowed not to generate MAC PDU and the CG occasion can be skipped by the UE, no transmission power consumption imposed. Hence, if UE determines the usage of CG occasions up to implementation, UE can reserve the CG occasions even though there is no pending data at no power cost.

Observation: UE does not need to indicate the CG occasion as unused if determination of CG occasion is up to UE implementation.

Therefore, the introduction of UTO-UCI will becomes meaningless if the determination of UTO-UCI is up to UE implementation. Since the MAC entity has the knowledge of the volume of data within the LCH(s) which can be mapped to a specific CG configuration and of the MAC PDU size for each CG occasion, it is reasonable that the determination of which CG occasions are used/unused is made by the MAC entity.

Proposal3: Whether UTO-UCI indicates CG occasion(s) as used/unused should be specified in MAC specification.

For multi-PUSCH CG, the retransmission can be performed by DG. Hence, for the UTO-UCI, it only needs to be carried by CG. Hence, the UTO-UCI is only transmitted when new transmission is performed on multi-PUSCH.

Proposal4: UTO-UCI is transmitted for each new transmission on CG occasions within a CG period when the MAC PDU is obtained.

As RAN1 has agreed to send UTO-UCI at each available CG occasion within UTO_period, we think that the MAC entity should determine the used/unused indication in the UTO-UCI based on the data from the LCH(s) that can be mapped to a CG configuration only.  The data from the LCH(s) not allowed to be mapped to the CG configuration should not be considered when designing the content of the associated UTO-UCI.

Proposal5: For each PUSCH transmission where UTO-UCI is sent, MAC entity determines the used/unused CG occasions based on the data from the LCH(s) which can be mapped to the corresponding CG configuration.

UTO period

For the sake of efficient usage of the CG resources, RAN1 has agreed to introduce UTO-UCI bitmap to indicate unused CG occasions such that the network can re-allocate the CG occasions to the other UEs. RAN1 is still discussing about different alternatives of specifying the mapping between UTO-UCI and PUSCH resources and other details. The currently considered options can be seen in their agreements summarized in the Annex of this document.

As per RAN1#113 meeting[1], the UTO-UCI is associated with a certain CG configuration:

Agreement Indication of UTO-UCI by CG PUSCHs associated to a CG configuration, is enabled by configuration of an RRC parameter. - FFS on whether/how to extend to multiple CG configurations

In any case, the concept of UTO_period was introduced to confine the applicability of UTO-UCI and the UE needs to determine the usage of future CG occasions within the UTO_period, e.g. how many CG occasions will be unused, prior to sending the associated UTO-UCI. When the UTO_period is across more than one CG periods, e.g.  the UTO_period larger than the CG period, the MAC entity will determine the usage of CG occasions in CG period #n+1 based on the data arrived in CG period #n. However, CG periodicity will be typically configured to match the periodicity of the XR traffic. As a consequence, it is highly likely that the CG occasions in CG period #n+1 are indicated as unused at the time of CG period #n, and there will be insufficient resources for the data arriving in CG period #n+1. Therefore, we believe that the UTO_period should be confined within a single CG period. To be more specific, the UTO-UCI should be only applicable to the CG occasions within the CG period where UTO-UCI is sent.

Proposal6: The UTO-UCI should be only applicable to the CG occasions within the CG period where UTO-UCI is sent.

Conclusions

HARQ process ID

Proposal1: Regarding the HARQ process ID for multi-PUSCH CG, RAN2 to confirm that

- the HARQ process ID of the first UL grant in the CG period is determined by the module operation with nrofHARQ-Processes and then add harq-ProcID-Offset2, if configured

- the HARQ process ID of the remaining configured and valid CG PUSCH is determined by incrementing the HARQ process ID of the preceding PUSCH in the period

Proposal2: RAN2 to address the issue of scarcity of HARQ PIDs when multi-PUSCH CGs are configured.

UTO-UCI

Proposal3: Whether UTO-UCI indicates CG occasion(s) as used/unused should be specified in MAC specification.

Proposal4: UTO-UCI is transmitted for each new transmission on CG occasions within a CG period when the MAC PDU is obtained.

Proposal5: For each PUSCH transmission where UTO-UCI is sent, MAC entity determines the used/unused CG occasions based on the data from the LCH(s) which can be mapped to the corresponding CG configuration.

Proposal6: The UTO-UCI should be only applicable to the CG occasions within the CG period where UTO-UCI is sent.

- References

Chair note of RAN1#113,

- https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_113/Inbox/Chair_notes/Chair's%20notes%20RAN1%23113%20v20.zip

Chair note of RAN1#112bis-e,

- https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_112b-e/Inbox/Chair_notes/Chair's%20notes%20RAN1%23112bis-e%20v15.zip

- Annex – RAN1 agreements on multiple CG PUSCHs

- RAN1#113 meeting – TDRA[1]

Agreement For time domain resource allocation for multi-PUSCH CGs, support - For TDRA determination (based on NR-U framework) - For Type-1, follow the rules for DCI format 0_0 on UE specific search space, as defined in Clause 6.1.2.1.1 of TS 38.214. - Note: To determine the configuration of TDRA, PUSCH repetition type A is assumed according to description in 6.1.2.3 in 38.214 for Type-1. - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one. - For Type-2, the TDRA table is determined by the TDRA table associated with activation DCI, as defined in Clause 6.1.2.1 of TS 38.214. - Note: The DCI format for activation DCI with pusch-RepTypeA is applicable. - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one. - N is configured by higher layers - A single SLIV is determined from TDRA. - The SLIV used for 1st PUSCH per CG period. - PUSCH is used in each of N consecutive slots per CG period - Note: N is configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. N configuration is independent from cgRetransmissionTimer configuration. - To determine corresponding slots for CG PUSCHs in a period of a multi-PUSCH CG configuration: - For the first PUSCH in the period, follow the legacy procedures. - For remaining PUSCHs in the period - ForType-1 and Type-2, reuse the corresponding procedures for NR-U by applying the RRC parameters N, instead of cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. Conclusion For time domain resource allocation for multi-PUSCH CGs, there is no consensus for further enhancement for operation on TDD

- RAN1#113 meeting – HARQ process IDs[1]

Agreement From RAN1 perspective, for determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH: - The HARQ process ID for the first configured PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable - HARQ Process ID = [X*floor( (CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes - HARQ Process ID = [X*floor((CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2 - X= the number of configured PUSCHs in the CG period - The HARQ process ID of the remaining configured and valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable. - Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB. Send an LS to RAN2 to convey the above RAN1 agreement. Final LS is in R1-2306233.

- RAN1#112bis-e meeting – UTO-UCI design[2]

Agreement The existing CG-UCI encoding and multiplexing procedures are reused for encoding the “UTO-UCI” in a configured grant PUSCH in absence or presence of other UCIs being multiplexed in the PUSCH, by applying the following adjustments: - The “UTO-UCI” is used instead of CG-UCI in the corresponding procedures for encoding of CG-UCI and/or HARQ-ACK, whichever is present. - For determining the beta-offset, - Beta offset is configured for the “UTO-UCI” - If UTO-UCI and HARQ-ACK is not jointly encoded, the beta offset for the “UTO-UCI” is used in the procedures instead of CG-UCI beta offset - If UTO-UCI and HARQ-ACK is jointly encoded, HARQ-ACK beta offset is used in the procedures instead of CG-UCI beta offset - FFS on sequence generation order between UTO-UCI and HARQ-ACK - FFS on dropping rule between UTO-UCI and HARQ-ACK when joint encoding is not configured - Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

- RAN1#113 meeting – UTO-UCI details[1]

Agreement For a CG configuration with UTO-UCI indication enabled, to determine the indicated CG PUSCH by a UTO-UCI indication, consider the following options for further down-selection: Option A-1a: - Configure the RRC parameter UTO_period. - FFS range value of UTO_period - Alt-1: values in time unit (e.g., XR traffic periodicity) - Alt-2: one or multiple of CG periodicity given by integer values (n=1, 2, ..) - The starting time of the first period of UTO periodicity starts at the same as starting time of the first period of the CG configuration and ends after UTO_period. The next UTO period(s) are followed after the first UTO period. - A transmitted CG PUSCH that is confined within a UTO period, carries UTO-UCI that is applicable to the CG PUSCH TOs within the UTO period. Option A-2a: - Configure the RRC parameter UTO_period. - FFS range value of UTO_period - Alt-1: values in time unit (e.g., XR traffic periodicity) - Alt -2: one or multiple of CG periodicity given by integer values (n=1, 2, ..) - Configure the RRC parameter UTO_offset. - FFS range value of UTO_offset - The starting time of the first period of UTO periodicity starts at the same as starting time of the first period of the CG configuration and ends after UTO_period. The next UTO period(s) are followed after the first UTO period. - A transmitted CG PUSCH that is confined within a UTO period, carries UTO-UCI that is applicable to the CG PUSCH TOs within the UTO period and after UTO_offset from the end of the transmitted CG PUSCH. Option B-a: - Configure the RRC parameter UTO_period. - FFS range value of UTO_period - Alt-1: values in time unit (e.g., XR traffic periodicity) - Alt -2: one or multiple of CG periodicity given by integer value (n=1, 2, ..) - UTO_offset is the offset value. - Alt-1: UTO_Offset is provided by configuration. - FFS range value of UTO_offset - Alt-2: UTO_Offset = 0 - A transmitted CG PUSCH carries UTO-UCI that is applicable to the valid CG PUSCH TOs that are confined within UTO_period starting with UTO_offset from the end of the transmitted CG PUSCH. Option B-b2: - Configure the RRC parameter Nu (Nu is the size of bit-map) - FFS range value of Nu - UTO_offset is the offset value. - Alt-1: UTO_Offset is provided by configuration. - FFS range value of UTO_offset - Alt-2: UTO_Offset = 0 - A transmitted CG PUSCH, carries UTO-UCI that is applicable to the Nu consecutive and valid CG PUSCH TOs, starting with UTO_offset from the end of the transmitted CG PUSCH. FFS on whether/how to extend to multiple CG configurations