3GPP TSG-RAN WG2 Meeting #123	R2-2307216

21 Aug – 25 Aug, 2023, Toulouse

Agenda Item	: 7.15.4 (NR_SL_enh2)

Source	: LG Electronics Inc.

Title	: Discussion on SL-U others

Document for	: Discussion and Decision

1.	Introduction

This contribution presents our views on DTX issues in SL-U.

2.	Discussion

2.1 DTX & LBT failure of PSFCH transmission

In SL-U, RX UE may not be able to transmit PSFCH due to LBT failure. However, the TX UE cannot distinguish PSFCH transmission failure due to LBT failure. That is, in SL-U, a problem in which the TX UE frequently increases the DTX count may occur.

For this issue, RAN2 can discuss three options.

Option 1. RAN2 consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE.

Option 2. RAN2 can consider an assistance information of RX UE to distinguish between DTX and LBT failure of PSFCH transmission. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure did not occur.

Option 3. RAN2 does not consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE. In other words, if the TX UE does not receive the PSFCH, the DTX count is increased by 1.

Option 1 can prevent a frequent DTX count increasing behaviour of the TX UE, but it is difficult to correctly distinguish PSFCH transmission failure due to LBT failure of the RX UE. For example, the TX UE needs to know the SL-CAPC value related to the RX UE’s PSFCH transmission to perform LBT, but the TX UE cannot know this information.

In the case of Option 3, the TX UE can frequently increase the DTX count. However, the probability of LBT failure can be reduced by supporting multiple PSFCH occasions under consideration in RAN1. Multiple PSFCH opportunities provide more opportunities for PSFCH transmission, but does not solve the problem of not distinguishing between DTX and LBT failure of the PSFCH.

RAN2 determined the granularity of LBT indication as RB set. That is, when LBT occurs, the UE can continue transmission by using the resources of another RB set. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure dit not occur to help TX UE to distinguish the DTX and LBT failure of PSFCH. From this point of view, we prefer option 2 to option 1 and option 3.

Considering these points, RAN2 can discuss whether or how to support the TX UE procedure to distinguish PSFCH transmission failure due to the LBT failure of the RX UE.

Observation 1. RAN2 consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE.

Observation 2. RAN2 can consider an assistance information of RX UE to distinguish between DTX and LBT failure of PSFCH transmission. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure did not occur. RAN2 determined the granularity of LBT indication as RB set. That is, when LBT occurs, the UE can continue transmission by using the resources of another RB set. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure dit not occur to help TX UE to distinguish the DTX and LBT failure of PSFCH.

Observation 3. RAN2 does not consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE. In other words, if the TX UE does not receive the PSFCH, the DTX count is increased by 1. In the case of this option, the TX UE can frequently increase the DTX count. However, the probability of LBT failure can be reduced by supporting multiple PSFCH occasions under consideration in RAN1. Multiple PSFCH opportunities provide more opportunities for PSFCH transmission, but does not solve the problem of not distinguishing between DTX and LBT failure of the PSFCH.

Proposal 1. RAN2 can discuss whether to support the TX UE procedure to distinguish PSFCH transmission failure due to the LBT failure of the RX UE.

- Option 1. RAN2 consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE.

- Option 2. RAN2 can consider an assistance information of RX UE to distinguish between DTX and LBT failure of PSFCH transmission. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure did not occur.

- Option 3. RAN2 does not consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE. In other words, if the TX UE does not receive the PSFCH, the DTX count is increased by 1.

3.	Conclusion

This contribution presented remaining issues on SL-U, which can be summarized as follows:

Observation 1. RAN2 consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE.

Observation 2. RAN2 can consider an assistance information of RX UE to distinguish between DTX and LBT failure of PSFCH transmission. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure did not occur. RAN2 determined the granularity of LBT indication as RB set. That is, when LBT occurs, the UE can continue transmission by using the resources of another RB set. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure dit not occur to help TX UE to distinguish the DTX and LBT failure of PSFCH.

Observation 3. RAN2 does not consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE. In other words, if the TX UE does not receive the PSFCH, the DTX count is increased by 1. In the case of this option, the TX UE can frequently increase the DTX count. However, the probability of LBT failure can be reduced by supporting multiple PSFCH occasions under consideration in RAN1. Multiple PSFCH opportunities provide more opportunities for PSFCH transmission, but does not solve the problem of not distinguishing between DTX and LBT failure of the PSFCH.

Proposal 1. RAN2 can discuss whether to support the TX UE procedure to distinguish PSFCH transmission failure due to the LBT failure of the RX UE.

- Option 1. RAN2 consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE.

- Option 2. RAN2 can consider an assistance information of RX UE to distinguish between DTX and LBT failure of PSFCH transmission. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure did not occur.

- Option 3. RAN2 does not consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE. In other words, if the TX UE does not receive the PSFCH, the DTX count is increased by 1.

4.	Reference

[1] #122 report from session on NR SL, 	Samsung