3GPP TSG-RAN WG2 Meeting #123	 R2- 2307230

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	7.16.2.1

Source:	Xiaomi

Title:	Discussion on Architecture aspects

Document for:	Discussion

Introduction

In [1], Artificial Intelligence and machine learning for air interface is proposed. In this contribution, we give our understanding on the general architecture.

Discussion

Meta info

R2 assumes that from Management or Control point of view mainly some meta info about a model may need to be known. The detail of meta info is FFS.

AI model used for each use case should be different, since the input and output may be different. In each use case, there can be different scenarios. The scenario could be categorized by NW deployment topology, channel quality, UE speed. It’s possible to use one AI model in all scenarios. But the performance may not be as good as using different specific AI model in each scenario. Considering the trad-off between generalization and inference accuracy, it may be more efficient to deploy multiple AI models and activate/deactivate AI model according to the scenario. Therefore, each AI model may only achieve best performance in certain scenarios. It’s essential to include applicable info of each AI model in the meta info. So LCM can be performed accordingly, e.g. model switch/activation/deactivation.

Proposal 1: Meta info of AI model includes following applicable info,

- Applicable use case;

- Applicable scenario, e.g. NW deployment topology, channel quality, UE speed;

Note the AI model may require a minimum AI capability, e.g. computing capability or storage. However, the RAN node, especially UE, may not always have enough capability regarding computing or storage. It’s essential to know AI model required AI capability. So that gNB/UE can decide which AI model can be used or delivered to UE.

Proposal 2: Meta info of AI model includes required AI capability, e.g. computing capability and storage.

Vendor info is proposed by some companies as meta info. However, we understand such info is not useful. The argument of including vendor info is this information may be used for appropriately providing meta info to NG-RANs. However, we understand the meta info should be common to all vendors. Otherwise, vendors may provide different meta info, which makes gNB/UE implementation more complex. Also, such info may expose UE privacy. Therefore, we don’t see vendor info as essential meta info to enable AI/ML.

Proposal 3: Vendor info is not included in Meta info from RAN2 perspective.

If the LCM is performed at UE, UE can evaluate the applicable condition of each AI model. If the applicable condition of activated AI model is not fulfilled, UE can deactivate the AI model autonomously. If the LCM is performed at NW, NW can configure UE to report the applicable condition fulfilment or not. Then, NW can choose to activate or deactivate AI models.

Proposal 4: If LCM is performed by UE, UE evaluates the applicable condition of each AI model and activate/deactivate/switch AI model accordingly.

Proposal 5: If LCM is performed by NW, NW can configure UE to report the applicable condition fulfilment or not.

Capability reporting

RAN1 agreed that the UE capability can indicate support of AI/ML enabled Feature/FG as following highlighted in yellow.

- For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. - For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda. - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s). - FFS: relationship between functionality-based LCM and model-ID-based LCM - Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

We understand such capability of AI/ML enabled Feature/FG is similar as legacy capability. It describes whether UE supports the AI/ML enabled Feature/FG from general point of view. Which is static and doesn’t change. Therefore, legacy UE capability mechanisms as defined for RRC reported and LPP reported capabilities can be reused.

Proposal 6: Legacy UE capability mechanisms as defined in RRC and LPP can be reused to indicate the capability of AI/ML enabled Feature/FG.

RAN1 also agreed the UE can indicate supported AI/ML model ID/functionality as following,

- Once models are identified, UE can indicate supported AI/ML model IDs for a given AI/ML-enabled Feature/FG in a UE capability report as starting point. - FFS: applicability to model identification, Type A, type B1 and type B2 - FFS: Using a procedure other than UE capability report - Note: model identification using capability report is not precluded for type B1 and type B2 - Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities. - Study necessity, mechanisms, after model identification, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models.

The supported model/functionality may change dynamically due to UE’s available storage/computation change. Legacy capability mechanisms may not be applicable, since it doesn’t support UE initiated capability update. Also, the signalling may be a problem if the update is frequent. New capability mechanism should be introduced to indicate UE supported AI/ML model/functionality. UE shall be able to update the supported AI/ML model/functionality. Details can be further discussed.

Proposal 7: RAN2 to study new capability mechanism to allow UE updates supported AI/ML model/functionality. Details can be further discussed.

Functionality mapping

In the post 122 email discussion #060 Mapping of functions to physical entities, some companies proposed to consider the case that UE performs AI model training. However, RAN2 have agreed to deprioritize the online/real-time training. Offline training may require large volume of training data and powerful computation. We understand the UE may have computation, storage and power limitation. It may not be possible for UE to perform offline training. Therefore, we propose to deprioritize the model training at UE. This doesn’t exclude the case that the model is trained at UE vendor’s OTT server.

Proposal 8: Deprioritize the model training at UE.

Another open issue is whether to consider CN entity for model training for CSI/beam use case. Current observed data collection options terminate at gNB/OAM. There is no way for CN entity to obtain the training data. Additional spec impact is foreseen to enable CN entity to be aware of the training data of CSI/beam. The benefit is not clear compared with training at gNB/OAM.

Proposal 9: Deprioritize the model training at CN entity.

Architecture

In last meeting, following functional architecture was agreed,

Fig. 1 Functional architecture

The management needs to acquire performance monitoring inputs. In the figure, there are two arrows from data collection and inference. However, we understand the data collection shall support both legacy metrics and AI related metrics. Therefore, it seems the arrow from model inference to management is unnecessary and can be removed. It is already covered by the arrow between data collection and management since the collected monitoring data could also include the output of the inference. An arrow from model inference to data collection should be added, which represents the monitoring output data collection.

Proposal 10: Remove the arrow representing monitoring output from model inference to model management.

Proposal 11: Add an arrow representing monitoring output from model inference to data collection.

The functionality/model identification process is one important aspect in the whole LCM. Inference and management can reach common understanding on the applicable model/functionality. This procedure shall be reflected in the diagram. Management is responsible to manage the model or functionality identification. After the model is trained, then related functionality identification or model identification can be performed if needed. Hence, an arrow representing the model/functionality identification can be added between the model training and management.

Proposal 12: Add an arrow representing the model/functionality identification between model training and management

According to proposal 10-12, the functional architecture can be modified as following,

Fig 2 Modified Functional architecture

Proposal 13: Modify the functional architecture as fig 2.

Conclusion

Based on the discussion in section 2, we have following proposals:

Proposal 1: Meta info of AI model includes following applicable info,

- Applicable use case;

- Applicable scenario, e.g. NW deployment topology, channel quality, UE speed;

Proposal 2: Meta info of AI model includes required AI capability, e.g. computing capability and storage.

Proposal 3: Vendor info is not included in Meta info.

Proposal 4: If LCM is performed by UE, UE evaluate the applicable condition of each AI model and activate/deactivate/switch AI model accordingly.

Proposal 5: If LCM is performed by NW, NW can configure UE to report the applicable condition fulfilment or not.

Proposal 6: Legacy UE capability mechanisms as defined in RRC and LPP can be reused to indicate the capability of AI/ML enabled Feature/FG.

Proposal 7: RAN2 to study new capability mechanism to allow UE update supported AI/ML model/functionality. Details can be further discussed.

Proposal 8: Deprioritize the model training at UE.

Proposal 9: Deprioritize the model training at CN entity.

Proposal 10: Remove the arrow representing monitoring output from model inference to model management in functional architecture figure.

Proposal 11: Add an arrow representing monitoring output from model inference to data collection.

Proposal 12: Add an arrow representing the model/functionality identification between model training and management in functional architecture figure.

Proposal 13: Modify the functional architecture as fig 2.

Reference

[1]