3GPP TSG-RAN2 Meeting # 123	R2-2307240

# Toulouse, France, August 21 - August 25

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1636 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections on TS 38.321 for SL enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2023-06-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For sl-DRX-GC-BC-Cycle/sl-DRX-GC-BC-OndurationTimer/sl-DRX-GC-InactivityTimer determination, the case where default SL DRX configuration being adopted is missed, i.e., current specification only captured the case where the determination is based on the values associated with SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List while the values configured in sl-DefaultDRX-GC-BC-r17 are all missed and should be thus added. - For the sl-DRX-GC-InactivityTimer triggered by new groupcast transmission, it is now captured in the clause for “Behaviour of UE receiving SL-SCH Data”, but it is a UE behavior triggered by UE transmitting SL-SCH data, which is misleading. - In 5.28.2, in the current timer value determination for broadcast and groupcast, different wording is used for unknow reason, i.e., “configured” is used for single value case while “associated” is used for multiple value case; 1> if a single sl-DRX-GC-BC-Cycle that is mapped with one or multiple SL-QoS-Profile is configured to a Destination and interested cast type is associated to groupcast or broadcast: … 1> else if multiple sl-DRX-GC-BC-Cycle that are mapped with multiple SL-QoS-Profile are associated to a Destination Layer-2 ID and interested cast type is associated to groupcast or broadcast:. - The “in this SL DRX” in 5.28.2 is misleading since the monitoring of SL SCI behaviour is not restricted by a particular “SL DRX”; 1> if an SL DRX is in Active Time: 2> monitor the SCI (i.e., 1st stage SCI and 2nd stage SCI) in this SL DRX. 2> if the SCI indicates a new SL transmission:. - In 5.28.3, currently in the example of Tx UE deciding the DRX active time of the Rx UE(s), only includes sl-drx-onDurationTimer, sl-drx-InactivityTimer, sl-drx-RetransmissionTimer which are limited to unicast case while the groupcast/broadcast related DRX timer is missing; Furthermore, the UE transmitting SL-SCH Data determines the SL DRX Active time based on SL DRX timers that are running (e.g., sl-drx-onDurationTimer, sl-drx-InactivityTimer, sl-drx-RetransmissionTimer) or will be running in the future (e.g., sl-drx-onDurationTimer, sl-drx-InactivityTimer, sl-drx-RetransmissionTimer) at the UE(s) receiving SL-SCH data. The UE may select resource for the initial transmission of groupcast within the time when sl-drx-onDurationTimer or sl-drx-InactivityTimer of the destination is running.. - In 5.28.2/5.28.3, the groupcast inactivity timer name (currently is “sl-drx-InactivityTimer” and should be “sl-DRX-GC-InactivityTimer”), and in 5.28.3, the groupcast onduration timer name (currently is “sl-drx-onDurationTimer” and should be “sl-DRX-GC-BC-OndurationTimer”) are wrong and should be corrected. In 5.28.2 3> if Destination Layer-1 ID of the SCI (i.e., 2nd stage SCI) is equal to the 16 LSB of the intended Destination Layer-2 ID and the cast type indicator in the SCI is set to groupcast: 4> start or restart sl-drx-InactivityTimer for the corresponding Destination Layer-2 ID in the first slot after SCI reception.. In 5.28.3 Furthermore, the UE transmitting SL-SCH Data determines the SL DRX Active time based on SL DRX timers that are running (e.g., sl-drx-onDurationTimer, sl-drx-InactivityTimer, sl-drx-RetransmissionTimer) or will be running in the future (e.g., sl-drx-onDurationTimer, sl-drx-InactivityTimer, sl-drx-RetransmissionTimer) at the UE(s) receiving SL-SCH data. The UE may select resource for the initial transmission of groupcast within the time when sl-drx-onDurationTimer or sl-drx-InactivityTimer of the destination is running.. |
| Summary of change: |  | - Adding default SL DRX configuration in sl-DRX-GC-BC-Cycle/sl-DRX-GC-BC-OndurationTimer/sl-DRX-GC-InactivityTimer determination procedure in 5.28.2. - Relocate the sl-DRX-GC-InactivityTimer triggered by new groupcast transmission from 5.28.2 to 5.28.3. - Change “configured” to “associated” in 5.28.2; - Remove “in this SL DRX” in 5.28.2; - Add the groupcast/broadcast related DRX timer in the example of Tx UE deciding the DRX active time of the Rx UE(s) in 5.28.3; - Correct the timer name in 5.28.2/5.28.3 Impact analysis Impacted 5G architecture options: NR SA Impacted functionality SL DRX Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue because this CR is to add a missing case/relocate text/editorial. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue because this CR is to add a missing case/relocate text/editorial. If one UE is implemented according to this CR while the other UE is not, there is no inter-operability issue because this CR is to add a missing case/relocate text/editorial. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The determination of sl-DRX-GC-BC-Cycle/sl-DRX-GC-BC-OndurationTimer/sl-DRX-GC-InactivityTimer is not correct since missing of consideration of default SL DRX configuration - The missleading of sl-DRX-GC-InactivityTimer triggered by new groupcast transmission captured in the section of SL-SCH data reception remains in the sprcification. - The description wording of single value and multiple value cases are not aligned; - Misleading remains in the specification that the UE behavior on monitoring SL SCI is restricted by “this SL DRX”; - Misleading remains in the specification that the UE doesn’t need to consider the groupcast/broadcast related timer when determining the SL DRX active time of the Rx UE(s); - Wrong timer name remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.2, 5.28.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

5.28.2	Behaviour of UE receiving SL-SCH Data

When SL DRX is configured, the Active Time includes the time while:

-	sl-drx-onDurationTimer/sl-DRX-GC-BC-OndurationTimer or sl-drx-InactivityTimer/sl-DRX-GC-InactivityTimer is running; or

-	sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer is running; or

-	period of sl-LatencyBoundCSI-Report configured by RRC in case SL-CSI reporting MAC CE is not received; or

-	the time between the transmission of the request of SL-CSI reporting and the reception of the SL-CSI reporting MAC CE in case SL-CSI reporting MAC CE is received; or

-	Slot(s) associated with the announced periodic transmission(s) by the UE transmitting SL-SCH Data; or

-	the time between transmission/reception of Direct Link Establishment Request message (TS 24.587 [28]) or ProSe Direct Link Establishment Request message (TS 24.554 [29]) and reception of RRCReconfigurationSidelink message including initial DRX configuration or the link establishment procedure being aborted by upper layer; or

-	the time between transmission of RRCReconfigurationSidelink message including initial DRX configuration and reception of corresponding RRCReconfigurationCompleteSidelink or RRCReconfigurationFailureSidelink message.

When one or multiple SL DRX is configured, the MAC entity shall:

1>	if a single sl-DRX-GC-BC-Cycle that is mapped with one or multiple SL-QoS-Profile is associated to a Destination and interested cast type is associated to groupcast or broadcast:

2>	select the sl-DRX-GC-BC-Cycle that is mapped with one or multiple SL-QoS-Profile associated with the Destination.

1>	else if a single sl-DRX-GC-BC-Cycle that is configured in sl-DRX-GC-generic and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List is associated to a Destination and interested cast type is associated to groupcast or broadcast:

2>	select the sl-DRX-GC-BC-Cycle that is configured in sl-DRX-GC-generic associated with the Destination.

1>	else if multiple sl-DRX-GC-BC-Cycle that are mapped with multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List are associated to a Destination Layer-2 ID and interested cast type is associated to groupcast or broadcast:

2>	select the sl-DRX-GC-BC-Cycle whose length is the shortest one among multiple sl-DRX-GC-BC-Cycle that are mapped with multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List associated with the Destination Layer-2 ID.

1>	else if one or multiple sl-DRX-GC-BC-Cycle that are mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List, and one sl-DRX-GC-BC-Cycle that is configured in sl-DRX-GC-generic and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List are associated to a Destination Layer-2 ID and interested cast type is associated to groupcast or broadcast:

2>	select the sl-DRX-GC-BC-Cycle whose length is the shortest one among one or multiple sl-DRX-GC-BC-Cycle that are mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List, and the sl-DRX-GC-BC-Cycle that is configured in sl-DRX-GC-generic associated with the Destination Layer-2 ID.

1>	if a single sl-DRX-GC-BC-OndurationTimer that is mapped with one or multiple SL-QoS-Profile is associated to a Destination and interested cast type is associated to groupcast or broadcast:

2>	select the sl-DRX-GC-BC-OndurationTimer that is mapped with one or multiple SL-QoS-Profile associated with the Destination.

1>	else if a single sl-DRX-GC-BC-OndurationTimer that is configured in sl-DRX-GC-generic and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List is associated to a Destination and interested cast type is associated to groupcast or broadcast:

2>	select the sl-DRX-GC-BC-OndurationTimer that is configured in sl-DRX-GC-generic associated with the Destination.

1>	else if multiple sl-DRX-GC-BC-OndurationTimer that are mapped with multiple SL-QoS-Profile are associated to a Destination Layer-2 ID and interested cast type is associated to groupcast or broadcast:

2>	select the sl-DRX-GC-BC-OndurationTimer whose length is the longest one among multiple sl-DRX-GC-BC-OndurationTimer that are mapped with multiple SL-QoS-Profile associated with the Destination Layer-2 ID.

1>	else if one or multiple sl-DRX-GC-BC-OndurationTimer that are mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List, and one sl-DRX-GC-BC-OndurationTimer that is configured in sl-DRX-GC-generic and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List are associated to a Destination Layer-2 ID and interested cast type is associated to groupcast or broadcast:

2>	select the sl-DRX-GC-BC-OndurationTimer whose length is the shortest one among one or multiple sl-DRX-GC-BC-OndurationTimer that are mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List, and the sl-DRX-GC-BC-OndurationTimer that is configured in sl-DRX-GC-generic associated with the Destination Layer-2 ID.

1>	if a single sl-DRX-GC-InactivityTimer that is mapped with one or multiple SL-QoS-Profile is associated to a Destination and interested cast type is associated to groupcast:

2>	select the sl-DRX-GC-InactivityTimer that is mapped with one or multiple SL-QoS-Profile associated with the Destination.

1>	else if a single sl-DRX-GC-BC-InactivityTimer that is configured in sl-DRX-GC-generic and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List is associated to a Destination and interested cast type is associated to groupcast or broadcast:

2>	select the sl-DRX-GC-BC- InactivityTimer that is configured in sl-DRX-GC-generic associated with the Destination.

1>	else if multiple sl-DRX-GC-InactivityTimer that are mapped with multiple SL-QoS-Profile of a Destination Layer-2 ID and interested cast type is associated to groupcast:

2>	select sl-DRX-GC-InactivityTimer whose length is the longest one among multiple sl-DRX-GC-InactivityTimer that are mapped with multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List or cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List associated with the Destination Layer-2 ID.

1>	else if one or multiple sl-DRX-GC-BC-InactivityTimer that are mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List, and one sl-DRX-GC-BC-InactivityTimer that is configured in sl-DRX-GC-generic and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List are associated to a Destination Layer-2 ID and interested cast type is associated to groupcast or broadcast:

2>	select the sl-DRX-GC-BC-InactivityTimer whose length is the shortest one among one or multiple sl-DRX-GC-BC-InactivityTimer that are mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List, and the sl-DRX-GC-BC-InactivityTimer that is configured in sl-DRX-GC-generic associated with the Destination Layer-2 ID.

1>	if an sl-drx-HARQ-RTT-Timer expires:

2>	if the data of the corresponding Sidelink process was not successfully decoded or if the HARQ feedback (i.e., negative acknowledgement) is not transmitted for unicast due to UL/SL prioritization:

3>	start the sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer for the corresponding Sidelink process in the first slot after the expiry of sl-drx-HARQ-RTT-Timer.

When the cast type is groupcast or broadcast as indicated by upper layer, or the cast type is unicast for the reception of Direct Link Establishment Request message [28] or ProSe Direct Link Establishment Request message [29] as indicated by upper layer, or for the reception of discovery message [26], the sl-drx-StartOffset and sl-drx-SlotOffset are derived from the following equations:

sl-drx-StartOffset (ms) = Destination Layer-2 ID modulo sl-DRX-GC-BC-Cycle (ms).

sl-drx-SlotOffset (ms) = (Destination Layer-2 ID modulo the number of slots in one subframe)
/ (the number of slots in one subframe) (ms).

1>	if the SL DRX cycle is used, and [(DFN × 10) + subframe number] modulo (sl-drx-Cycle or sl-DRX-GC-BC-Cycle) = sl-drx-StartOffset:

2>	start sl-drx-onDurationTimer/sl-DRX-GC-BC-OndurationTimer after sl-drx-SlotOffset from the beginning of the subframe.

1>	if an SL DRX is in Active Time:

2>	monitor the SCI (i.e., 1st stage SCI and 2nd stage SCI).

2>	if the SCI indicates a new SL transmission:

3>	if Source Layer-1 ID of the SCI is equal to the 8 LSB of the intended Destination Layer-2 ID and Destination Layer-1 ID of the SCI is equal to the 16 LSB of the intended Source Layer-2 ID and the cast type indicator in the SCI is set to unicast:

4>	start or restart sl-drx-InactivityTimer for the corresponding Source Layer-2 ID and Destination Layer-2 ID pair in the first slot after SCI reception.

3>	if Destination Layer-1 ID of the SCI (i.e., 2nd stage SCI) is equal to the 16 LSB of the intended Destination Layer-2 ID and the cast type indicator in the SCI is set to groupcast:

4>	start or restart sl-DRX-GC-InactivityTimer for the corresponding Destination Layer-2 ID in the first slot after SCI reception.

2>	if the SCI indicates an SL transmission:

3>	if a next retransmission opportunity is indicated in the SCI:

4>	derive the sl-drx-HARQ-RTT-Timer from the retransmission resource timing of the next retransmission resource in the SCI.

3>	else if PSFCH resource is configured for the SL grant associated to the SCI:

4>	set the sl-drx-HARQ-RTT-Timer based on sl-drx-HARQ-RTT-Timer1 configured by upper layer if the cast type associated with the SCI is unicast or sl-DRX-GC-HARQ-RTT-Timer1 configured by upper layer if the cast type associated with the SCI is groupcast when HARQ feedback is enabled, or based on sl-drx-HARQ-RTT-Timer2 configured by upper layer if the cast type associated with the SCI is unicast or sl-DRX-GC-HARQ-RTT-Timer2 configured by upper layer if the cast type associated with the SCI is groupcast when HARQ feedback is disabled, for resource pool configured with PSFCH.

3>	else (i.e., if PSFCH resource is not configured for the SL grant associated to the SCI):

4>	set the sl-drx-HARQ-RTT-Timer as 0 slots.

3>	if PSFCH resource is not configured for the SL grant associated to the SCI:

4>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

3>	if PSFCH resource is configured for the SL grant associated to the SCI:

4>	if HARQ feedback is enabled by the SCI and the cast type associated with the SCI is unicast; or

4>	if HARQ feedback is enabled by the SCI and the cast type associated with the SCI is groupcast and positive-negative acknowledgement is selected;

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization.

4>	if HARQ feedback is enabled by the SCI and the cast type associated with the SCI is groupcast and negative-only acknowledgement is selected;

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is a positive acknowledgement.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is not scheduled in the SCI:

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSFCH resource.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is scheduled in the SCI:

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

NOTE:	Void.

3>	stop the sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer for the corresponding Sidelink process.

1>	if an SL DRX Command MAC CE is received for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast:

2>	stop sl-drx-onDurationTimer for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast;

2>	stop sl-drx-InactivityTimer for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast.

Next Change

5.28.3	Behaviour of UE transmitting SL-SCH Data

The UE transmitting SL-SCH Data should keep aligned with its intended UE receiving the SL-SCH Data regarding the SL DRX Active time as specified in clause 5.28.2.

Furthermore, the UE transmitting SL-SCH Data determines the SL DRX Active time based on SL DRX timers that are running (e.g., sl-drx-onDurationTimer/sl-DRX-GC-BC-OndurationTimer, sl-drx-InactivityTimer/sl-DRX-GC-InactivityTimer, sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer) or will be running in the future (e.g., sl-drx-onDurationTimer/sl-DRX-GC-BC-OndurationTimer, sl-drx-InactivityTimer/sl-DRX-GC-InactivityTimer, sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer) at the UE(s) receiving SL-SCH data. The UE may select resource for the initial transmission of groupcast within the time when sl-DRX-GC-BC-OndurationTimer or sl-DRX-GC-InactivityTimer of the destination is running.

NOTE 1:	A UE may assume that a resource for retransmission is in the Active time if an initial transmission causes the sl-drx-RetransmissionTimer to be started at the receiving UE.

NOTE 2:	A UE may send SL DRX Command MAC CE to receiving UE for unicast and when to send SL DRX Command MAC CE is up to UE implementation.

1>	if the SCI indicates a new transmission where the cast type is set to groupcast is transmitted:

2>	start or restart sl-DRX-GC-InactivityTimer for the corresponding Destination Layer-2 ID in the first slot after SCI transmission.

The MAC entity shall for each pair of the Source Layer-2 ID and the Destination Layer-2 ID corresponding to each PC5-RRC connection which has been established by upper layers:

1>	if the SL DRX Command indication has been triggered by the UE:

2>	if the MAC entity has SL resources allocated for new transmission and the SL-SCH resources can accommodate the SL DRX Command MAC CE and its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate a Sidelink DRX Command MAC CE as defined in clause 6.1.3.52;

3>	cancel the triggered SL DRX Command indication.

2>	else if the MAC entity has been configured with Sidelink resource allocation mode 1:

3>	trigger a Scheduling Request.

End of Change