3GPP TSG-RAN WG2 Meeting #123 	R2-2307272

Toulouse, France, 21st – 25th Aug, 2023

Title:	RANU issues for mobile IAB

Source:	Huawei, HiSilicon

Agenda item:	7.12.3

Document for:	Discussion

- Introduction

This contribution mainly focuses on the remaining issues of RANAC Update for mobile IAB.

- Discussion

Figure 1. RAC update in the Mobile IAB scenario

According to the agreement [1] from RAN3 #119 meeting, we have,

- Capture on stage 2 that the TAC/RANAC broadcast by the mobile IAB-DU can be changed in order to reflect the mIAB-node’s physical location. It needs to be further discussed how the mobile IAB-DU’s TAC/RANAC is changed and what Stage 3 impacts are (if any).

And referring to TS 38.473 [2], we have,

For NG-RAN, the gNB-DU may include the RAN Area Code IE in the GNB-DU CONFIGURATION UPDATE message. The gNB-CU shall store and replace any previously provided RAN Area Code IE by the received RAN Area Code IE.

That is, in legacy F1AP, DU can update the RANAC and notify CU.

Specifically, in addition to the RANAC-based RNA configuration, the RNA can also be configured via a list of cell IDs. The following mainly discusses the impact of the changeable RNA IDs on the on-board and surrounding UEs under the two RNA configurations.

- RNA configured via a cell ID list

The cell ID refers to the NCGI, given the RAN3 agreement that,

- The NCGI of the mobile IAB-DU cell is changed when the F1-terminating donor CU of the mobile IAB-DU is changed.

For partial migration, the NCGI of the mobile IAB-cell can remain unchanged for a long traveling distance, which will anyway cause the surrounding UEs’ RNAU procedures. While for the on-board UEs, even though the RNAU procedure is not triggered, the network can locate the on-board UEs correctly.

For DU migration, if the updated NCGI is within the surrounding UEs’ RNA, the RNAU procedure will not be triggered. For on-board UEs, the NCGI update supports the chance of initiating UEs’ RNAU when necessary, e.g., in case the Xn connectivity with a certain last serving gNB will no longer be available.

- RNA configured via a RANAC list

For partial migration, similar to the NCGI case, the network can correctly locate the on-board UEs without changing the RANAC of the mobile IAB-cell. Even though this may result in the surrounding UEs’ RNAU procedures, the RNAU initiated by the on-board UEs is not required because the serving CU has not changed.

For DU migration, the surrounding UEs’ RNAU procedure may not be triggered for a properly updated RANAC of the mobile IAB-cell, which will also support the chance of initiating on-board UEs’ RNAU if necessary.

Based on the above analysis, for the dynamic RANAC scheme, the impact on either the on-board UEs or the surrounding UEs cannot be avoided. That is, for a given RANAC of a mobile IAB-cell, either the on-board UEs or the surrounding UEs shall perform the RNAU procedure. When deciding whether to update the RANAC, the network should consider the trade-off between the impact on surrounding UEs and on-board UEs (e.g., the number of affected UEs), which is largely implementation dependent.

Observation 1: NW implementation may decide on whether/when to update the RAN-AreaCode, e.g., considering the balance between the impact on surrounding UEs and on-board UEs’ RNAU procedure. And, it is up to RAN3 on how to reconfigure RAN-AreaCode.

One of the motivations for updating the RANAC of a mobile IAB-cell is to avoid the RNAU of surrounding UEs due to the appearance of a mobile IAB cell in an area. However, the RANAC update may trigger the RNAU procedure of on-board UEs. This is not necessary in case of no CU change in mobile IAB partial migration. In this case, the network may instruct the R18 UE to suspend the RNAU procedure, to avoid impact on the on-board UEs.

Proposal 1: In mobile IAB cells, to avoid the impact on on-board UEs (i.e., unnecessary RNAU when the RAN-AreaCode updates), NW can broadcast the disabling/suspending of the RNAU procedure for R18 UEs.

- Conclusion

This paper mainly discusses the remaining issues of interference mitigation for mobile IAB, and the following observations and proposals are provided,

Observation 1: NW implementation may decide on whether/when to update the RAN-AreaCode, e.g., considering the balance between the impact on surrounding UEs and on-board UEs’ RNAU procedure. And, it is up to RAN3 on how to reconfigure RAN-AreaCode.

Proposal 1: In mobile IAB cells, to avoid the impact on on-board UEs (i.e., unnecessary RNAU when the RAN-AreaCode updates), NW can broadcast the disabling/suspending of the RNAU procedure for R18 UEs.

- References

- Chairman notes of 3GPP TSG-RAN WG3 meeting #119-e.

- 3GPP TS 38.473, F1 application protocol (F1AP).