3GPP TSG-RAN WG2 Meeting #123											        R2-2307343

Toulouse, 21-25 August, 2023

Agenda item:			7.7.4.2

Title:						Handover enhancements

Source:					Continental Automotive

Document for:		Discussion, Decision

# Introduction

The work item aims at specifying enhancements for NG-RAN based NTN (Non-Terrestrial Networks) in Rel-18 [1] and has identified NTN-TN and NTN-NTN measurement/mobility and service continuity as important areas for improvements. In particular, the following objective is stated:

- Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4]

  - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4]

  - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3]

  - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4]

  - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between cells. [RAN3]

During RAN2#119bis-e [2] it was agreed:

Agreements:

- RAN2 confirms that at least for the moving cell case the next serving cells can be largely predicted in NTN (at least for UEs not at the cell edge) thanks to the existence of predefined satellite orbits and negligible UE’s mobility in comparison to satellite’s motion (we can further discuss at the next meeting whether this applies to idle mode UEs as well).

- New Proposal 2: RAN2 continues the discussion (e.g. at RAN2#120) on the solution with keeping the same PCI after switching of the satellites. Clarify at least the following:

•	RAN1 impact

•	The need to perform UL beam switching and/or RA

•	Applicability to hard or soft satellite switching

Further, the following agreements related to RACH-less HO were made during RAN2#122 [3]:

Agreements:

- From RAN2 perspective synchronization among source and target cells is not an issue in NTN RACH-less HO.

- Release pre-allocated UL grant after RACH-less HO completion.

- LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI.

- Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact , if any).

In this paper, we share further thoughts on combining RACH-less handover with Conditional Handover (CHO).

# 2. Discussion

As stated in [4], mobility and service continuity in connected mode is expected to be significantly affected by rapid satellite movements, particularly in case of LEO deployments with earth moving cells. For example, a UE may need to perform handover after 6.61 seconds [4], if served by a LEO satellite with a cell diameter of 50 km. Hence, the total number of handovers that need to be performed at a given time can very likely result in significant RACH congestion as well as signalling load in the network.

Recently, RACH-less handover was proposed to address the issues of significant RACH congestion as well as signalling load in the network. Further, it was agreed [3] that both synchronous and asynchronous RACH-less handovers are supported in NTN. As additional enhancement for avoiding peaks of signalling load during handover preparation phase, Conditional Handover (CHO) was proposed to distribute the transmission of CHO configurations to UEs over time. Thus, handover preparation information can be sent to each UE well in advance before CHO execution. The CHO procedure is illustrated in Figure 1 below.

Moreover, it was confirmed [2] that due to the availability of satellite ephemeris data satellite movements and, at least in case of earth moving cells, next serving cells can be predicted. However, it was also noted that prediction of next serving cells for UEs at cell edge is more challenging. This is particularly true when many cell edge UEs are moving at higher speeds. Further, scenarios where the majority of UEs within a cell are highspeed UEs that move through the cell edge area are quite common on roads or train tracks in rural areas. Moreover, UE speed will directly affect the triggering and execution of CHO.

Observation 1: Prediction of next serving cells for highspeed UEs is particularly challenging. Further, scenarios where the majority of UEs within a cell are highspeed UEs that move through the cell edge area are quite common on roads or train tracks in rural areas. Moreover, the UE speed will directly affect the triggering and execution of (conditional) handovers.

Proposal 1: RAN2 should discuss how UE mobility information can be made available so that the network can configure CHO more efficiently. For example, a UE could provide movement details, such as speed and movement direction. Thus, next serving cells of UEs can be predicted more accurately.

Figure 1: CHO procedure [5]

Knowledge of UE mobility can be beneficially used for predicting a sequence of next serving cells. Without information on next serving cells, the network would potentially reserve too many resources at other potential target gNBs, which is inefficient and a waste of resources.

Furthermore, according to [5], the source gNB sends the HANDOVER CANCEL message toward the other signaling connections or other candidate target gNBs to cancel CHO for the UE.

Hence, all resource reservations will be cancelled, although UEs will be arriving after the first CHO to other configured target cell(s). Such an exemplary scenario is illustrated in Figure 2 below, where a UE first performs conditional handover from cell #1 to cell #2 and subsequently to cell #4.

Figure 2: Subsequent handovers of highspeed UE

Observation 2: After CHO, all resource reservations will be cancelled, although UEs will be arriving after the first CHO to other configured target cell(s).

Resource reservation can be enhanced by allocating a timer when UL grant is configured by the potential target gNB. The timer starts when UE receives RRC reconfiguration message or sends RRC reconfiguration complete message. The timer stops when UE uses the UL grant. If the timer expires, UE will release UL grant.

Proposal 2: Resource reservation can be enhanced by allocating a timer when UL grant is configured by the potential target gNB.

Further, if after CHO, congestion is determined in any target cell and/or gNB, target cell and/or gNB indicates to source cell and/or gNB to release configured UL grant resources and to update the CHO configuration accordingly. Thus, any grant-related timer is stopped.

Proposal 3: If after CHO, congestion is determined in any target cell and/or gNB, target cell and/or gNB indicates to source cell and/or gNB to release configured UL grant resources and to update the CHO configuration accordingly.

Therefore, the target cell and/or gNB does not reserve UL grant resources infinitely but can flexibly revoke allocated grants.

# Conclusions

The following observations and proposals have been made in this document:

Observation 1: Prediction of next serving cells for highspeed UEs is particularly challenging. Further, scenarios where the majority of UEs within a cell are highspeed UEs that move through the cell edge area are quite common on roads or train tracks in rural areas. Moreover, the UE speed will directly affect the triggering and execution of (conditional) handovers.

Proposal 1: RAN2 should discuss how UE mobility information can be made available so that the network can configure CHO more efficiently. For example, a UE could provide movement details, such as speed and movement direction. Thus, next serving cells of UEs can be predicted more accurately.

Observation 2: After CHO, all resource reservations will be cancelled, although UEs will be arriving after the first CHO to other configured target cell(s).

Proposal 2: Resource reservation can be enhanced by allocating a timer when UL grant is configured by the potential target gNB.

Proposal 3: If after CHO, congestion is determined in any target cell and/or gNB, target cell and/or gNB indicates to source cell and/or gNB to release configured UL grant resources and to update the CHO configuration accordingly.

# References

- RP-230809 (revision of RP-223534) Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, Thales, RAN#99, March 20-23, 2023.

- R2-2210801, Report from Break-out session on NR-NTN and IoT-NTN, ZTE, RAN2#119bis-e, October 10-19, 2022.

- R2-2306541, Report from Break-out session on NR-NTN and IoT-NTN, ZTE, RAN2#122, May 22-26, 2023.

- 3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), Release 16.

- 3GPP TS 38.300, NR and NG-RAN Overall Description, Stage 2, Release 17.