3GPP TSG-RAN WG2 Meeting #123	R2-2307406

Toulouse, France, August 21-25, 2023

Agenda item:	7.20.2

Source: 	Fujitsu

Title: 	Considerations on multi-DCI multi-TRP operation with two TAs

Document for:	Discussion and decision

1. Introduction

This contribution will discuss the following aspects for Multi-DCI Multi-TRP operation with two TAs and provide some proposals, taking RAN1 and RAN2 agreements/conclusions into account:

- Two TAGs per serving cell

- UE-initiated RACH procedure

2. Discussion

2.1 Two TAGs per serving cell

UE behavior when TAT expires

To support two TAGs per serving cell, one RAN2 issue is to specify the UE behaviors for a SpCell and for a SCell when one or both TATs expired for the cell.

The existing UE behaviors when TAT expires are captured as below [1]:

1> when a timeAlignmentTimer expires: 2> if the timeAlignmentTimer is associated with the PTAG: 3> flush all HARQ buffers for all Serving Cells; 3> notify RRC to release PUCCH for all Serving Cells, if configured; 3> notify RRC to release SRS for all Serving Cells, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> consider all running timeAlignmentTimers as expired; 3> maintain NTA (defined in TS 38.211 [8]) of all TAGs. 2> else if the timeAlignmentTimer is associated with an STAG, then for all Serving Cells belonging to this TAG: 3> flush all HARQ buffers; 3> notify RRC to release PUCCH, if configured; 3> notify RRC to release SRS, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> maintain NTA (defined in TS 38.211 [8]) of this TAG.

The UE behaviors upon TAT expiry are related to the modelling of PTAG, i.e. one or two PTAT are supposed. So, we will list all possibilities first.

The use cases for two TAGs per serving cell are shown in Figure 1.

Figure 1: use cases

Considering all listed use cases, there are three kinds of serving cell:

- a serving cell with TRPs only associated to PTAG

  - only if two PTAGs are supposed

- a serving cell with TRPs only associated to STAG

  - if one or two PTAGs are supposed

- a serving cell with TRPs associated to one PTAG and one STAG

  - if one or two PTAGs are supposed

We can determine the UE behaviors upon TAT expiry no matter how PTAG is modelled if it is difficult to make an agreement on PTAG modelling.

According to existing UE behavior, if PTAT expires, all STATs are considered as expired. The same behavior applies in case of two TAGs configured for serving cell. For SpCell, if one PTAG is supposed, the SpCell belongs to serving cell type 3) while the SpCell belongs to serving cell type 1) if there are two PTAGs. No matter how PTAG is modelled, i.e. one or two PTAGs are supported, PTAT(s) should be expired when both TATs for a SpCell are expired.

Proposal 1: For the case both TATs for a SpCell are expired, existing UE behaviors upon PTAT expiry apply no matter how PTAG is modelled.

When both TATs for an SCell are expired, existing UE behaviors upon sTAT expiry apply to the SCell. For SCell, if two PTAGs are supposed, it can belong to any of abovementioned types while the SCell belongs to type 2) or 3) if there is one PTAG. This is not related to PTAG modelling if PTAT is running because only serving cell 2) meets both conditions (both TATs for SCell are expired and PTAT is running) and serving cell 2) is only associated to STAGs.

Proposal 1bis: For the case both TATs for an SCell are expired, if PTAT is running, existing UE behaviors upon sTAT expiry apply for both TRPs for the SCell no matter how PTAG is modelled.

Regarding UE behaviour when TAT expired, in RAN1 LS [2], RAN1 confirms that when the TA timer associated to one TRP expires for a TAG associated with a TCI state, UL or DL operation associated to the another TRP is not impacted. So, at least UL transmission associated to related TRP except the RA is stopped when one of TATs expired. This is not related to PTAG modelling if PTAT is running.

Proposal 2: For the case one TAT is expired and the other TAT is running for a SpCell or for a SCell, if PTAT is running, the following UE behaviors apply for the TRP corresponding to the expired TAT no matter how PTAG is modelled:

- not perform any uplink transmission except the Random Access Preamble and MSGA transmission;

- maintain NTA of this TAG.

PTAG modelling

RAN1 agreed to support configuring two TAGs belonging to a serving cell. RAN2 agreed to configure one TAT per TAG to support two TAs for a serving cell, i.e., in this case 2 TAGs are configured for the serving cell.

The definition for Timing Advance Group is captured in the MAC specification [1]:

Timing Advance Group: A group of Serving Cells that is configured by RRC and that, for the cells with a UL configured, using the same timing reference cell and the same Timing Advance value. A Timing Advance Group containing the SpCell of a MAC entity is referred to as Primary Timing Advance Group (PTAG), whereas the term Secondary Timing Advance Group (STAG) refers to other TAGs.

According to the current definition of TAG, there could be two primary TAGs per MAC entity if an SpCell is configured with two TAGs.

Observation: According to the current definition of TAG, the UE may manage at most two Primary TAGs per MAC entity, if two TAGs are configured to the SpCell.

Although two PTAGs per MAC entity are aligned with the definition of TAG, some issues can be observed. For example, PTAG has TAG ID 0 in the existing TAC MAC CE. If there are two PTAGs, the UE cannot determine which PTAG should apply the TA value according to the TAG ID.

If one of the two TAGs associated with SpCell is considered as PTAG, some modifications to the definition of TAG are required and the mechanism to determine PTAG needs to be discussed. In this case, there are two TAGs associated to SpCell, including one PTAG and one STAG.

Proposal 3: RAN2 discusses whether to support at most one or two Primary TAGs per MAC entity.

2.2 UE-initiated RACH procedure

TAG association

RAN2 agreed that per TRP UE-initiated RACH procedure is not supported [3]. However, currently, even for multi-DCI based multi-TRP operation with two TA enhancement, RA procedure can be initiated by the UE when UL is not synchronized and there is UL data arrival. In this case, according to RAN2 agreement, RACH procedure is initiated according to current mechanism first and then the network can trigger per TRP RACH procedure by PDCCH order if needed. With this, it is required to discuss which TAG is applied or which TA timer is restarted when TA value is received in the UE initiated RACH procedure, as shown in Figure 2.

Figure 2: Potential issue in UE initiated RACH procedure

Considering the options for PDCCH ordered RACH discussed in RAN1, we think that SSBs partition, i.e., separate set of SSBs for each TRP is a simple and unified solution.

Proposal 4: Separate set of SSBs for each TRP is configured so that the UE can determine whether TAG1 or TAG2 is applied for UE-initiated RACH.

RA procedure initialization

According to existing MAC specification [1], there is only one Random Access procedure ongoing at any point in time in a MAC entity. If a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure. This can apply for the multi-DCI multi-TRP operation with two TAs. However, RAN2 needs to discuss how to determine whether a RA procedure is a new RA or not.

Currently, on one hand, if there was an ongoing Random Access procedure that is triggered by a PDCCH order while the UE receives another PDCCH order indicating the same Random Access Preamble, PRACH mask index and uplink carrier, the Random Access procedure is considered as the same Random Access procedure as the ongoing one [1]. It means that RA preamble, PRACH mask index and uplink carrier in PDCCH order are considered to determine whether a new RA is triggered or not. On the other hand, for inter-cell multi-DCI based multi-TRP operation with TA enhancement, RAN1 agreed to support indication of which PRACH configuration to be used for RA procedure in PDCCH order and it needs further discussion on whether additionalPCI or a generic identifier is indicated [4]. So, in addition to existing information, cell information needs to be considered.

Proposal 5: For inter-cell multi-DCI based multi-TRP operation with TA enhancement, RA preamble, PRACH mask index, uplink carrier and cell index in PDCCH order are considered to determine whether a new RA is triggered or not.

3. Conclusion

In this contribution, we have some discussions on the Multi-DCI Multi-TRP operation with two TAs and the following observations and proposals are made:

Two TAGs per serving cell

Proposal 1: For the case both TATs for a SpCell are expired, existing UE behaviors upon PTAT expiry apply no matter how PTAG is modelled.

Proposal 1bis: For the case both TATs for an SCell are expired, if PTAT is running, existing UE behaviors upon sTAT expiry apply for both TRPs for the SCell no matter how PTAG is modelled.

Proposal 2: For the case one TAT is expired and the other TAT is running for a SpCell or for a SCell, if PTAT is running, the following UE behaviors apply for the TRP corresponding to the expired TAT no matter how PTAG is modelled:

- not perform any uplink transmission except the Random Access Preamble and MSGA transmission;

- maintain NTA of this TAG.

Observation: According to the current definition of TAG, the UE may manage at most two Primary TAGs per MAC entity, if two TAGs are configured to the SpCell.

Proposal 3: RAN2 to discuss whether to support at most one or two Primary TAGs per MAC entity.

UE-initiated RACH procedure

Proposal 4: Separate set of SSBs for each TRP is configured so that the UE can determine whether TAG1 or TAG2 is applied for UE-initiated RACH.

Proposal 5: For inter-cell multi-DCI based multi-TRP operation with TA enhancement, RA preamble, PRACH mask index, uplink carrier and cell index in PDCCH order are considered to determine whether a new RA is triggered or not.

References

- TS 38.321: ‘NR; Medium Access Control (MAC) protocol specification’, V17.3.0

- R2-2307018, ‘Reply on LS 2TA for multi-DCI multi-TRP’, RAN2#123

- R2-2306552, ‘Report from NR MIMO evolution session’, RAN2#122

- RAN1 chair notes, RAN1#112bis