3GPP TSG-RAN WG2 Meeting #123                                                       R2-2307414

Toulouse, France, August 21st-25th, 2023

Source:	CATT

Title:	Discussion on GNSS operation in connected mode

Agenda Item:	7.6.2.2

Document for:	Discussion and Decision

Introduction

For the GNSS operation in connected mode, some agreements have been achieved during the last RAN1 and RAN2 meeting, and some open issues were left after discussion in RAN2. In this contribution, we give some discussion on the GNSS operation for IoT NTN UE in long connection mode, based on RAN1 and RAN2 agreements, and open issues.

Discussion

Negotiation for long connection mode

In our contribution [1] in RAN2#121bis, we have analyzed that, the network need to know whether UE supports the improved GNSS operation, to avoid invalid configuration related with improved GNSS operation, for example, to trigger connected UE perform GNSS operation. And the UE need also to know whether the network supports the improved GNSS operation, to avoid unnecessary report of GNSS position fix time duration for measurement.

Observation: Indications are needed to indicate whether the network/UE supporting the improved GNSS operations.

According to the following agreement:

Agreements:

- New SIBxx is introduced to broadcast the neighbor cell/satellite information.

A new SIB for neighbor cell/satellite information will be introduced for Rel-18 IoT NTN cell. Then the UE can determine whether a cell supporting improved GNSS operations or not by the broadcasting of the new SIB for neighbor cell/satellite information, with the assumption that all the Rel-18 IoT NTN cell will support improved GNSS operations.

Proposal 1: The UE determines the network supporting improved GNSS operations by the broadcasting SIB for neighbor cell/satellite information.

During the offline discussion of [AT121][101][IoT NTN enh] GNSS operation (CATT) [2], companies thought that, UE capability may be needed, but can be discussed later.

Proposal 2: Introduce separate UE capability for improved GNSS operations.

Based on the indication above, network supporting the improved GNSS operations can configure an UE supporting the improved GNSS operations to re-acquire GNSS position fix, aperiodically or autonomously (when configured by the network), if the network wants the UE continue to keep RRC_CONNECTED after the current GNSS position becoming out-of-date, according to the following RAN1 agreements:

Agreement For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap - FFS details of gap configuration The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement - FFS based on configured timing

If the network does not configure the UE to re-acquire GNSS position fix, regardless the network does not support the improved GNSS operations, or the network does want the UE to keep RRC_CONNECTEDD after the GNSS position becoming out-of-date, the UE should leave RRC_CONNECTED state after the GNSS position becoming out-of-date, as what the Rel-17 IoT NTN UE does.

Proposal 3: By default, Rel-18 IoT NTN UE (supporting the improved GNSS operations) will leave RRC_CONNECTED state when current GNSS position becoming out-of-date, unless it has been indicated by network to perform GNSS measurement.

We can discuss the case of UE requiring performing GNSS measurement actively, if RAN1 has any agreements on this case.

The UE behavior when the UE can’t re-acquire GNSS position fix during the GNSS measurement gap duration

According to the following RAN1 agreements, the UE can perform GNSS measurement during a gap for eNB aperiodically triggering GNSS measurement, or a GNSS measurement timer for autonomous GNSS re-acquisition:

Agreement For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap - FFS details of gap configuration The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement - FFS based on configured timing

And

Agreement For NB-IoT and eMTC, at least for the case where the network configuration does not include a periodicity (if supported), for autonomous GNSS re-acquisition, the UE may re-acquire GNSS autonomously during GNSS measurement timer, the start time of the autonomous GNSS measurement timer is based on the original GNSS validity duration. - FFS: additional delay and details of delay (if any), e.g. delay can be zero or can be equal to/larger than the duration X where UL transmission can be allowed after original GNSS validity duration expires without GNSS re-acquisition. - Note1: Autonomous GNSS re-acquisition mechanism is enabled or disabled by network. - Note2: The length of GNSS measurement timer can be configured by network and the length of GNSS measurement timer is equal to the latest reported GNSS position fix time duration for measurement when the length of GNSS measurement timer is not configured - Note3: The autonomous GNSS re-acquisition can be periodic in certain conditions without further spec impact

So the network can configure a suitable gap or GNSS measurement timer, for example, to let UE try one or several attempts of GNSS measurement if necessary. In this case, the UE should go to IDLE if the UE fails to re-acquire the GNSS position fix during the GNSS measurement gap or timer.

Proposal 4: The UE should go to IDLE state if the UE can’t re-acquire GNSS position fix during the GNSS measurement gap duration/timer.

- It can be left to network implementation to let UE try one or several attempts of GNSS measurement

GNSS measurement during inactive state of C-DRX

RAN2 has agreed to discuss the case of UE autonomously reacquire GNSS during inactive state of C-DRX in RAN2#121bis:

Agreements online:

- RAN2 can discuss UE autonomously reacquire GNSS during inactive state of C-DRX based on RAN1’s input in the next RAN2 meeting

According to our understanding, firstly, if the UE tries to reacquire GNSS during the inactive state of C-DRX, there should not be impacts on current C-DRX procedure. Then, it can be left to UE implementation to determine whether to autonomously reacquire GNSS during inactive state of C-DRX, no specific enhancement is needed.

Proposal 5: It should be left to UE implementation for UE autonomously reacquiring GNSS during inactive state of C-DRX. No specification impact is needed.

Whether to support specific SR to GNSS validity duration MAC CE

This issue has been discussed in last meeting:

R2-2306641 [offline-101] GNSS operation enhancements Apple discussion Rel-18 IoT_NTN_enh-Core Proposal 5: (5/16 Yes, 3/16 Maybe, 8/16 No) FFS whether to introduce specific SR to GNSS validity duration MAC CE.

The view was much dispersed. But we have confirmed that the GNSS validity duration UE reports is the remaining validity duration, in last meeting:

Agreements:

- Confirm the working assumption that GNSS validity duration UE reports is the remaining validity duration.

So it is not so critical to immediately trigger the validity duration MAC CE transmission. And generally, we think the network may know the time when the UE completing the GNSS measurement, according to the measurement gap, or the timer for UE autonomous GNSS measurement case, so maybe it is the network implementation to ensure the UE to have UL resources for GNSS validity duration MAC CE (assuming the priority of GNSS validity duration is high) report. No specific SR is needed.

Proposal 6: RAN2 will not introduce specific SR to GNSS validity duration MAC CE.

Closed loop time correction

For the closed loop time correction, RAN1 has the following agreement:

Agreement From RAN1 perspective, at least for the case when frequency error and timing error are within frequency and timing error requirements with legacy closed loop time correction, UL transmission can be allowed in a duration X after original GNSS validity duration expires without GNSS re-acquisition. RAN1 will decide further details of the above.

We think the duration X should be configured by network, not a pre-defined value. Because the X value can be different for different or same UE, in different situation, at different time. The question is, whether the X value is always configured, what the UE behavior if the network does not configure this X value.

We think one option is, the UE can continue UL transmission after original GNSS validity duration expires without GNSS re-acquisition, until the eNB gives another indication, e.g, the eNB triggers the UE perform GNSS measurement, or the eNB release RRC connection.

Proposal 7: For closed loop time correction case, if no X value is configured by network, UE can continue UL transmission after original GNSS validity duration expires without GNSS re-acquisition, until the network gives another indication.

Security issue of MAC CE

For GNSS validity duration reporting and eNB triggering UE to perform GNSS measurement, RAN2 has given the following working assumption:

Working Assumptions:

1. 	An UL MAC CE for GNSS validity duration reporting is used for NB-IoT user plane solution and eMTC UE as well, in addition to previously agreed NB-IoT control plane solution

2.	A new DL MAC CE is introduced to trigger connected UE to perform GNSS measurement.

We think there is no critical security issue here. So we suggest to confirming the working assumption.

Proposal 8: Confirm the following working assumption:

Working Assumptions:

1. 	An UL MAC CE for GNSS validity duration reporting is used for NB-IoT user plane solution and eMTC UE as well, in addition to previously agreed NB-IoT control plane solution

2.	A new DL MAC CE is introduced to trigger connected UE to perform GNSS measurement.

Conclusion

In this contribution, we give some discussion on the GNSS operation for IoT NTN UE in long connection mode, based on RAN1 and RAN2 agreements, and open issues, and the following observations and proposals are given:

Observation: Indications are needed to indicate whether the network/UE supporting the improved GNSS operations.

Proposal 1: The UE determines the network supporting improved GNSS operations by the broadcasting SIB for neighbor cell/satellite information.

Proposal 2: Introduce separate UE capability for improved GNSS operations.

Proposal 3: By default, Rel-18 IoT NTN UE (supporting the improved GNSS operations) will leave RRC_CONNECTED state when current GNSS position becoming out-of-date, unless it has been indicated by network to perform GNSS measurement.

Proposal 4: The UE should go to IDLE state if the UE can’t re-acquire GNSS position fix during the GNSS measurement gap duration/timer.

- It can be left to network implementation to let UE try one or several attempts of GNSS measurement

Proposal 5: It should be left to UE implementation for UE autonomously reacquiring GNSS during inactive state of C-DRX. No specification impact is needed.

Proposal 6: RAN2 will not introduce specific SR to GNSS validity duration MAC CE.

Proposal 7: For closed loop time correction case, if no X value is configured by network, UE can continue UL transmission after original GNSS validity duration expires without GNSS re-acquisition, until the network gives another indication.

Proposal 8: Confirm the following working assumption:

Working Assumptions:

1. 	An UL MAC CE for GNSS validity duration reporting is used for NB-IoT user plane solution and eMTC UE as well, in addition to previously agreed NB-IoT control plane solution

2.	A new DL MAC CE is introduced to trigger connected UE to perform GNSS measurement.

Reference

- R2-2302558 Discussion on GNSS operation in connected mode, RAN2# 121bis meeting

- R2-2301951 Report of AT121101IoT NTN enh GNSS operation (CATT), RAN2# 121 meeting