3GPP TSG RAN WG2 #123												   R2-2307441

Toulouse, France, August 21-25, 2023

Agenda Item:	7.8.2

Source: 	NEC

Title:	Discussion on Measurement Reports Enhancements

Document for:	Discussion

- Introduction

In RAN2#121bis-e the below agreements were made [1]:

- Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR. FFS on the number of height ranges. - As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy behaviour applies. - For UE behavior on L1 and L3 measurement, it is left to UE implementation whether to keep/discard the old samples while UE moves to a new height region with a different SSB-ToMeasure value. - New event types will be introduced on the combination of event Ax and event Hx, at least for event A4 + event H1/H2. FFS for other event Ax + event H1/H2. FFS on details, e.g. whether to include one height threshold (H1 or H2 threshold) or a height range (both H1 and H2 threshold) in the new event, how to configure height-dependent numberOfTriggeringCells, etc. This will be applied to all height dependent MR parameters. - Whether UE height is included when UAV specific MR is triggered is configurable by the network. - We will use LTE UEheight.

This contribution provides our considerations on measurement reports enhancements.

- Discussion

2.1 Joint condition for measurement report triggering

In previous meeting, companies agreed to introduce a new event types on the combination of event Ax and event Hx. Here we offering two examples. One is introducing a new event type “eventU1” in EventTriggerConfig.

EventTriggerConfig::=                       SEQUENCE {

eventId                                     CHOICE {

eventA1                                     SEQUENCE {

a1-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

< omitted >

[[

eventU1                                     SEQUENCE {

firstEvent                                  CHOICE {

eventA4                                     SEQUENCE {

a4-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useAllowedCellList                          BOOLEAN

},

eventAx                                     SEQUENCE {

-- if other Ax event is used, add it here

},

}

secondEvent                                 CHOICE {

eventH1                                     SEQUENCE {

-- refering to eventH1

},

eventH2                                     SEQUENCE {

-- refering to eventH2

},

}

},

]]

...,

< omitted >

},

Another example is to introducing a new report type “EventCombinedTriggered” in ReportConfigNR. This structure seems cleaner but referring to EventTriggerConfig may cause some overhead issue as there are some mandatory fields in EventTriggerConfig.

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

< omitted >

[[

eventCombinedTriggered                      EventCombinationConfig

]]

}

}

EventCombinationConfig ::=                     SEQUENCE {

firstEvent					EventTriggerConfig,

secondEvent					EventTriggerConfig,

...,

}

...,

< omitted >

}

To achieve joint condition for measurement report triggering, consider from following two options:

- Introduce a new event type “eventU1”, which associate to two measurement events.

- Introduce a new report type “eventCombinedTriggered”, which associate to two measurement events.

As shown above, measurement reporting will be triggered when the first and the second event are fulfilled simultaneously. We know that event Ax and event Hx are evaluated by different measurement samples and both of them need timeToTrigger to avoid ping-pong. Therefore, regarding to “Being fulfilled simultaneously”, there may be several options of the UE behavior:

- Upon entering conditions of one event is fulfilled for TTT, check another event, until another event is also fulfilled for TTT, trigger measurement reporting.

It means that the earlier fulfilled event shall be still satisfied at the time when the other even is fulfilled. Details are up to UE implementation.

- Upon entering conditions of one event is fulfilled for TTT, start a timer, during the running of the timer, if another event is also fulfilled for TTT, stop the timer and trigger measurement reporting. Upon expiry of the timer, reporting can also be triggered by the fulfilled event.

Considering that sometimes the fulfilling of an independent Ax or Hx event will also be useful for network, this can save some additional eventTriggered-type report configurations.

RAN2 to clarify the UE behavior of triggering a joint event:

- Upon entering conditions of one event is fulfilled for TTT, check another event, until another event is also fulfilled for TTT, trigger measurement reporting

- Upon entering conditions of one event is fulfilled for TTT, start a timer, during the running of the timer, if another event is also fulfilled for TTT, stop the timer and trigger measurement reporting. Upon expiry of the timer, reporting can also be triggered by the fulfilled event.

2.2 Height-dependent adjustment for MO parameters

After receiving multiple measurement object configurations from the network, UE may need to switch between different configurations due to vertical moving, so network need to keep align with UAV UE on what the actual configuration the UE currently uses.

From our perspective, network can track the height of the UE roughly by height-dependent measurement reporting. Upon receiving the measurement report triggered by UE height, the network will know the altitude of the UAV UE so that it may also aware of the configuration the UAV UE applies. In that case, the measurement configuration switch need to occur based on the height-dependent measurement event or the threshold of the height-dependent measurement event need to be align with the height boundary of applying the measurement configuration switch.

Another way to keep the alignment is to explicitly report to the network upon switching between measurement configurations. E.g. Similar to what we have done in RLM/BFD measurement relaxation, which UE need to provide an indication to the NW upon change of its relaxation state for RLM/BFD measurements.

RAN2 to discuss how to ensure that the measurement configuration used by the UAV UE is aligned between the UAV UE and the network (i.e., switch configurations based on height-dependent measurement event or inform the network upon switch).

2.3 Height triggered reporting

For LTE Rel-15 aerial UEs, same height event H1 or H2 may be triggered again after leaving the corresponding event. Therefore, if UAV UE is flying around the configured reporting height threshold, measurement report will be triggered frequently.

Height-triggered measurement reporting may also cause signaling overhead.

To reduce the amount of height triggered reporting, new height event reflecting height variation could be considered. Since the height of the UAV UE can be included in each measurement report, the variation of the height will be more valuable for network to track the altitude of the UAV. Based on this approach, reporting due to enter or leave a same height event may also be limited.

RAN2 to consider new height event triggered by height variation.

- Conclusion

Observation 1:	Height-triggered measurement reporting may also cause signaling overhead.

Proposal 1:	To achieve joint condition for measurement report triggering, consider from following two options:

a.	Introduce a new event type “eventU1”, which associate to two measurement events.

b.	Introduce a new report type “eventCombinedTriggered”, which associate to two measurement events.

Proposal 2:	RAN2 to clarify the UE behavior of triggering a joint event:

a.	Upon entering conditions of one event is fulfilled for TTT, check another event, until another event is also fulfilled for TTT, trigger measurement reporting

b.	Upon entering conditions of one event is fulfilled for TTT, start a timer, during the running of the timer, if another event is also fulfilled for TTT, stop the timer and trigger measurement reporting. Upon expiry of the timer, reporting can also be triggered by the fulfilled event.

Proposal 3:	RAN2 to discuss how to ensure that the measurement configuration used by the UAV UE is aligned between the UAV UE and the network (i.e., switch configurations based on height-dependent measurement event or inform the network upon switch).

Proposal 4:	RAN2 to consider new height event triggered by height variation.

- References

[1] R2-23xxxxx, Report of 3GPP TSG RAN WG2 meeting #122, ETSI MCC.