3GPP TSG-RAN WG2 Meeting #123	 R2-2307461

Toulouse, France, August 21-25, 2023

Agenda Item:	7.22.2

Source: 	NEC

Title:  	Discussion on the considerations for LPWUS in RRC_IDLE INACTIVE

Document for:	Discussion and Decision

- 1 Introduction

In RAN2#122 meeting [1], the following agreements are made:

RAN2 expect that different coverage LR/MR may have RAN2 impact, e.g. UE need to stop using LP WUS when moving out of LR coverage, other aspects FFS. What to cover (if anything) in TS 38.304 is FFS.

For UE in RRC_IDLE/RRC_INACTIVE state, it is FFS to what extent the network is or need to be aware of which receiver the UE uses MR/LR or both (for paging reception etc). A potential drawback of not knowing could be increased LP WUS load, a potential drawback of awareness is increased signalling.

RAN2 assumes that UE uses LP WUS when pre-configured condition(s) are fulfilled. 
(Other control methods not precluded)

RAN2 assumes that using subgrouping for LP-WUS could be beneficial to reduce false alarms rate (depend on L1 capacity to carry payload).

- 2 Discussion

- 2.1 Transition from RRC_IDLE/INACTIVE to ultra-deep-sleep mode

Before the discussion, it is noted that the transition from ultra-deep-sleep mode (i.e., LPWUS mode) to normal RRC_IDLE/INACTIVE state should also be considered. But the main reason to perform the transition from ultra-deep-sleep mode to RRC_IDLE/INACTIVE is that UE receives LPWUS indication. Although there is some other criteria could trigger this transition, we only consider the basic functionality design here, i.e., transition from RRC_IDLE/INACTIVE to ultra-deep-sleep mode.

Regarding the transition from RRC_IDLE/INACTIVE to ultra-deep-sleep mode, there could be two options:

1. UE enters ultra-deep-sleep mode based on preconfigured condition

It is noted that RAN2 has already agreed that UE uses LP WUS when pre-configured condition(s) are fulfilled, and more detail is that what kind of conditions we will use.

To our understanding, due to the low complexity of low receiver (LR), the reliability of LPWUS reception should be considered in ultra-deep-sleep mode. If one UE is in low mobility / stationary and/or not-at-cell-edge, the channel situation will be stable enough to receive LPWUS. Therefore, the legacy RRM relaxation criteria could be considered as the ultra-deep-sleep mode triggering condition.

But one thing needs to consider is network awareness. If UE determines to enter ultra-deep-sleep mode by itself, network side probably does not know whether the UE is in normal RRC_IDLE/INACTIVE or ultra-deep-sleep mode. If network always transmit LPWUS for such kind of UE, the resource efficiency could come down, if the UE notifies this fulfilled condition to network before entering ultra-deep-sleep mode, signal overhead is introduced. This is a trade-off that we need to consider.

Proposal 1: For preconfigured condition, low mobility / stationary and/or not-at-cell-edge criteria can be considered as the trigger condition.

Proposal 2: Network awareness should be considered if preconfigured condition is used.

2. Network indication to trigger UE entering ultra-deep-sleep mode

For this option, network may (based on implementation) indicate the UE to enter ultra-deep-sleep mode for LPWUS monitoring in an ultra-deep-sleep mode. But there is still concern on such a network indication as the UE stays in RRC_IDLE/INACTIVE, no dedicated RRC signalling can reach to the UE. But if we use common signalling such as system information, a group of UEs will be notified to enter ultra-deep-sleep mode, obviously different UE has different situation, thus it is desired to notify individual one to enter ultra-deep-sleep mode instead of a group of UEs.

To our understanding, if network wants to notify an individual UE in RRC_IDLE/INACTIVE in legacy, only paging message can achieve this functionality. UE ID (e.g., 5G-S-TMSI / I-RNTI) can be used to notify which UE should establish/resume RRC connection. Here we can consider using paging message to notify a RRC_IDLE/INACTIVE UE of entering ultra-deep-sleep mode.

Besides paging notification, a new common channel still can be considered (let’s say low power control channel), network can put multiple UE IDs in this channel as an indication of entering ultra-deep-sleep mode. And a LPWUS capable UE needs to monitor a low power control channel during RRC_IDLE/INACTIVE, if its own UE ID matches the one in low power control channel, UE will enter ultra-deep-sleep mode.

By means of the solutions described above, network can individually notify a RRC_IDLE/INACTIVE UE of entering ultra-deep-sleep mode and network is also aware of UE’s state.

Proposal 3: RAN2 is suggested to consider the network indication to trigger RRC_IDLE/INACTIVE UE entering ultra-deep-sleep mode.

- 2.2 LPWUS subgrouping

In legacy RRC_IDLE/INACTIVE, multiple UEs may use the same paging resource/occasion to monitor its own paging message. However even though one UE receives the paging indication, sometimes the paging message including specific UE ID may not indicate this UE but for other UEs, which causes unnecessary paging reception (unnecessary power consumption), i.e., false alarm issue. That is the reason why we design the subgrouping in PEI (Paging early indication) mechanism.

Similarly, LPWUS monitoring may have the same problem as paging monitoring. Thus subgrouping for LPWUS can be considered. More detail is that how much group capacity of LPWUS we can have and what kind of subgrouping formula we will adopt. It is up to RAN1 to tell us how many groups that RAN2 can have no matter it is designed based on time/frequency resource or bit string, and from RAN2 perspective, once we get the information of group number, the grouping formula from PEI can be referred, e.g., UE calculates its own group ID based on UE ID, group number, etc, or network directly assign a group ID to one UE, which can be further studied based on RAN1 conclusion.

Proposal 4: Subgrouping for LPWUS should be considered and detail of how to design subgrouping can be based on RAN1 conclusion and legacy PEI subgrouping mechanism.

- 2.3 LPWUS mobility

Supporting LPWUS mobility is important as UE does not have to always fall back to normal RRC_IDLE/INACTIVE when cell reselection happens, which reduces signal overhead as well as UE experience. In legacy mobility for RRC_IDLE/INACTIVE, cell reselection is performed by UE to let the UE camp on a suitable cell, and once downlink data/signal is available, network use paging mechanism to find this UE in one or multiple cells in TA or RNA. That is to say, mobility is supported by cell re-selection and paging in other cells. What’s more, RAN3 impact is also introduced (similar to CN/RAN paging message in Ng/Xn interface) which can be FFS later.

It is noted that reference signal (e.g., SSB or new low power synchronization signal (LP-SS)) as well as cell identity is needed for cell re-selection, and TAU/RNAU is required to support paging in other cells. Thus if the LR is only used to monitor LPWUS, the mobility of LPWUS will not be supported.

Proposal 5: To support LPWUS mobility, RAN2 is suggested to consider how to provide reference signal, cell identity, TAU and RNAU.

Furthermore, if we introduce synchronization/reference signal in ultra-deep-sleep mode. There could be two types of reference signal available for measurement, i.e., this new LP-SS and SSB. Thus we need to further consider the co-existence issue, for example, sometimes MR is used to measure SSB and sometimes LR is used to measure new reference signal. And some problems need to be solved like whether the receiving sensitivity between two types of signal varies a lot, whether both signals can be evaluated together for the same cell reselection criterion, etc.

Proposal 6: If new reference signal is introduced in ultra-deep-sleep mode, RAN2 is suggested to consider the issue on co-existence between new reference signal and SSB.

Regarding TAU/RNAU, in legacy, UE needs to match the TAC of the camped cell with the TAI list within registration area provided by NAS signalling for TAU. And UE needs to match the camped cell identity with the RNA info provided by RRCRelease. It is assumed that UE has already stored TAI list and RNA info, but the problem is that how a UE in ultra-deep-sleep mode can get the camped cell’s identity, TAC, PLMN. For example, it should be studied whether the UE needs to check the SIB1 or not during RRC_IDLE/INACITVE mobility in ultra-seep-sleep mode.

Proposal 7: If TAU/RNAU is introduced in ultra-deep-sleep mode, RAN2 is suggested to consider how UE can get the camped cell’s identity, TAC, PLMN.

- 2.4 LPWUS content

The payload size of LPWUS should be determined by RAN1, and RAN2 needs to consider how to use it. In principal, the payload size of LPWUS should not be large as we try to design a simple signal.

Some potential contents of LPWUS are:

1. Wake up indication

2. Group indication

3. Cell related information

4. Some early indication/information, e.g., short message

To our understanding, there is no controversial for 1. Wake up indication, but the monitoring behaviour can be further discussed like whether it is explicit or implicit. For 2. Group indication, a reasonable capacity for group indication is needed to avoid false alarm issue, as this can further reduce UE power consumption. For 3. Cell related information, which can provide the Cell identity to support cell reselection, is also needed if mobility is supported, but need FFS whether to put this info into LP-SS. For other information like PLMN, TAC which is used to support TAU/RNAU, it is not sure for now due to the low capacity of LPWUS. For 4. Some early indication/information (e.g., short message), we think at least for short message this is an over-optimized mechanism, the UE actually can monitor paging after LPWUS reception, and we are concerning that more and more contents added into LPWUS could increase the complexity of LPWUS design. Also it is noted that even for PEI mechanism, there is no short message in PEI.

Proposal 8: RAN2 is suggested to consider whether the content of LPWUS (besides wake-up indication) should include group indication and cell related information.

Proposal 9: Not introduce specific content within short message into to LPWUS.

- 2.5 UE behaviours in RRC_IDLE/INACTIVE after LPWUS reception

Actually there are three solutions for the UE behaviour after LPWUS reception:

1. Monitor PO;

2. Monitor PEI+PO

3. Initiate random access

Fig.1 UE behaviour after LPWUS reception

The difference between solution 1 and 2 is that whether UE still needs to monitor PEI firstly. Intuitively, there is no need to use both LPWUS+PEI simultaneously as they has the similar function (i.e., indicate whether there is paging/data/signal available), monitoring both can increase unnecessary power consumption. However due to false alarm issue, it is also not clear that whether only to use LPWUS (i.e., exclusive of PEI) is good enough. For example, if group capacity in LPWUS is insufficient (e.g., only 2 groups), unnecessary LPWUS reception increase the UE power consumption instead, thus in this case, PEI (e.g., 8 groups) can be used to “double filter” the false alarm issue. But if the group capacity of LPWUS is same as PEI’s, there is no need to use both then.

Proposal 10: RAN2 is suggested to consider whether/how to use both LPWUS and PEI simultaneously.

Another solution is that UE could directly initiate random access after LPWUS reception, so that UE could omit the (PEI+) paging reception procedure. But again, false alarm issue could make UE unnecessarily enter RRC_CONNECTED which is not a good solution. To our understanding, if we want to achieve this functionality, random access procedure needs to be enhanced.

Proposal 11: If UE could directly initiate random access after LPWUS reception, the enhancement of random access procedure should be considered.

- 3 Conclusions

Proposal 1: For preconfigured condition, low mobility / stationary and/or not-at-cell-edge criteria can be considered as the trigger condition.

Proposal 2: Network awareness should be considered if preconfigured condition is used.

Proposal 3: RAN2 is suggested to consider the network indication to trigger RRC_IDLE/INACTIVE UE entering ultra-deep-sleep mode.

Proposal 4: Subgrouping for LPWUS should be considered and detail of how to design subgrouping can be based on RAN1 conclusion and legacy PEI subgrouping mechanism.

Proposal 5: To support LPWUS mobility, RAN2 is suggested to consider how to provide reference signal, cell identity, TAU and RNAU.

Proposal 6: If new reference signal is introduced in ultra-deep-sleep mode, RAN2 is suggested to consider the issue on co-existence between new reference signal and SSB.

Proposal 7: If TAU/RNAU is introduced in ultra-deep-sleep mode, RAN2 is suggested to consider how UE can get the camped cell’s identity, TAC, PLMN.

Proposal 8: RAN2 is suggested to consider whether the content of LPWUS (besides wake-up indication) should include group indication and cell related information.

Proposal 9: Not introduce specific content within short message into to LPWUS.

Proposal 10: RAN2 is suggested to consider whether/how to use both LPWUS and PEI simultaneously.

Proposal 11: If UE could directly initiate random access after LPWUS reception, the enhancement of random access procedure should be considered.

- 4 References

[1] R2_122_ChairNotes EOM