3GPP TSG-RAN WG2 Meeting #123	 R2-2307462

Toulouse, France, August 21-25, 2023

Agenda Item:	7.22.3

Source: 	NEC

Title:  	Discussion on the considerations for LPWUS in RRC_CONNECTED

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#122 meeting, there is no progress on LPWUS applied in RRC_CONNECTED.

Since this is a SID phase, it is still worth to analyse LPWUS applied in RRC_CONNECTED no matter it is a high priority or not.

- 2 Discussion

- 2.1 Background

First of all, the difference between LPWUS applied in RRC_IDLE/INACTIVE and in RRC_CONNECTED should be clarified. To our understanding, when LPWUS is applied in RRC_IDLE/INACTIVE, i.e., the UE will use Low Receiver (LR) to monitor LPWUS while the Main Radio (MR) is in an ultra-deep-sleep mode or even turned off. But when LPWUS is applied in RRC_CONNECTED, besides LR is activated, the MR is not allowed to enter ultra-deep-sleep mode or turn off, instead the MR may temporarily stay in a non-active mode or light/micro sleep mode, which means only part of MR functionality will be deactivated, e.g., PDCCH monitoring.

Since this is for RRC_CONECTED, in this contribution we will discuss the relationship between LPWUS and C-DRX, LPWUS mobility (e.g., handover), and false alarm issue.

- 2.2 The relationship between LPWUS and C-DRX

In RRC_CONNECTED, the main power consumption is from PDCCH monitoring within a C-DRX mode. When the C-DRX is in Active Time, the UE shall monitor PDCCH based on the configuration (e.g., search space, CORESET, RNTIs, etc.). When the UE uses “LPWUS mode” during RRC_CONENCTED, the PDCCH monitoring of MR should be deactivated, and once UE detects LPWUS indicating there is data/signalling coming, the UE activates the MR to operate C-DRX functionality for PDCCH monitoring.

More detail is that for LPWUS monitoring, there is two patterns:

1. Duty cycle monitoring;

2. Always-on monitoring.

Different monitoring behaviour has different impact on C-DRX operation.

For duty cycle monitoring pattern, one solution is that legacy DCP could be replaced by LPWUS, as DCP also belongs to a kind of PDCCH monitoring, using LPWUS could bring more power saving gain. But there is a limitation that the periodicity of LPWUS should be always configured as same as C-DRX / DCP cycle, also one issue should be considered is that the offset of LPWUS should be larger before the start of drx-onDurationTimer as the MR may need time to activate, this is not friendly especially when C-DRX cycle is configured small. Another solution is that the periodicity of LPWUS is not related to C-DRX cycle (e.g., the periodicity of LPWUS is more dense or sparse), dense case as shown in fig.1, the configuration could be more flexible but there is still a drawback that the UE may not get an Active Time immediately after LPWUS reception.

Fig.1 the periodicity of LPWUS is not related to C-DRX cycle

Proposal 1: For duty cycle monitoring pattern, RAN2 is suggested to consider the following solutions:

- DCP is replaced by LPWUS

- The periodicity of LPWUS is not related to C-DRX cycle

For always-on monitoring pattern, the UE continuously detects LPWUS within RRC_CONNECTED for the possible data/signalling coming but the MR is deactivated or stays in micro/light sleep. The intention of Always-on monitoring should reduce the data/signal reception latency on the cost of a little bit more power consumption, but due to C-DRX mode, there is still a drawback that the UE may not get an Active Time immediately after the LPWUS reception when the LPWUS reception happens in non-active time of DRX, as shown in fig.2.

Fig.1 the always-on monitoring pattern of LPWUS

Proposal 2: For always-on monitoring pattern, RAN2 is suggested to consider how UE enters Active Time of DRX in a better way when LPWUS is detected.

- 2.3 Mobility

RRC_CONNECTED mobility is supported by RRM measurement management including handover. Performing measurements is required. When the UE uses LR to monitor LPWUS during RRC_CONNECTED, although the MR is deactivated or stay in a micro/light sleep mode, which means part of MR operations will be deactivated, e.g., PDCCH monitoring. But to support mobility, the MR still needs to perform measurements in this kind of LPWUS mode. Furthermore, regarding measurement report after performing measurements, the MR should be able to initiate uplink transmission (e.g., SR) regardless of C-DRX state which is same as legacy behaviour. One more question is that should UE fall back to normal RRC_CONNECTED (i.e., PDCCH monitoring as normal) once measurement report is transmitted, to our understanding, no need. If handover happens, the buffered RRC signalling will trigger LPWUS transmission, so that UE can still receive handover command after LPWUS detection.

Proposal 3: To support LPWUS mobility in RRC_CONNECTED, the Main Radio still needs to perform RRM measurement management as normal.

- 2.4 False alarm issue

It is up to LPWUS design, when multiple UEs are related to the same LPWUS monitoring resource, false alarm issue would happen. In legacy DCP mechanism, although DCP is a group common DCI, a specific block number can respond to each specific UE. Thus there is no false alarm issue in DCP mechanism. If LPWUS is not a UE-specific signal, when the UE in RRC_CONNECTED detects LPWUS which not indicate this UE to operate C-DRX, this UE will unnecessarily start Active Time of C-DRX for PDCCH monitoring which cause unnecessary power consumption. This should be considered.

Proposal 4: If LPWUS applied in RRC_CONNECTED is not a UE-specific signal, RAN2 is suggested to consider how to reduce the probability of UE unnecessarily starting Active Time of C-DRX for PDCCH monitoring.

- 3 Conclusions

Proposal 1: For duty cycle monitoring pattern, RAN2 is suggested to consider the following solutions:

- DCP is replaced by LPWUS

- The periodicity of LPWUS is not related to C-DRX cycle

Proposal 2: For always-on monitoring pattern, RAN2 is suggested to consider how UE enters Active Time of DRX in a better way when LPWUS is detected.

Proposal 3: To support LPWUS mobility in RRC_CONNECTED, the Main Radio still needs to perform RRM measurement management as normal.

Proposal 4: If LPWUS applied in RRC_CONNECTED is not a UE-specific signal, RAN2 is suggested to consider how to reduce the probability of UE unnecessarily starting Active Time of C-DRX for PDCCH monitoring.

- 4 References