3GPP TSG-RAN WG2 Meeting #123											   R2-2307673

Toulouse, France, 21-25 Aug, 2023

Source: 	Xiaomi

Title:  	TAT expiry and TAG modeling

Agenda Item:	7.20.2

Document for:	Discussion and Decision

# Introduction

According to the reply LS [3] (for which the RAN1 replies can be found in the Annex) from RAN1, RAN1 “has not agreed to any further restrictions on the association of serving cells and/or TRPs”, and RAN1 confirms that “when the TA timer associated to one TRP expires for a TAG associated with a TCI state, UL or DL operation associated to the another TRP is not impacted”. Following the answers provided by RAN1, we provide our understandings on how to model the TRP-specific TAG and the UE behaviours at the TAT expiry. The corresponding RAN2 agreements are quoted as follows:

RAN2#122 meeting agreements: - Configure one TAT per TAG to support two TAs for a serving cell, i.e., in this case 2 TAGs are configured for the serving cell.

# Discussion

## TAT expiry for two TAGs of a serving cell

Figure 1: TAG modelling for2 TAs of a cell

According to the relies provided by RAN1, it is possible that one cell is configured with two TAGs, and one TAG can also include one or more cells, as illustrated above.

According to the current MAC specification of 38.321, when the TAT of the PTAG expires, the UE stops all uplink transmission of all serving cells of the same MAC entity. The reason is that as PTAG always include SpCell, the uplink transmission of all serving cells should be stopped when SpCell is not able to work.

According to the current MAC specification of 38.321, when the TAT of the STAG expires, the UE stops all uplink transmission of all serving cells belonging to the same STAG. The reason is that if the serving cells of a STAG is not able to work, the UL transmission of other cells should not be impacted.

Observation 1: The current MAC specification specifies the UE behaviours for the TAT expiry for each serving cell separately.

Observation 2: Same as the legacy, if the SpCell is not able to send any UL transmission (except for PRACH) due to the 2 TAT expiry, all UL transmission (except for PRACH) of the same cell group (i.e. either MCG or SCG) should be stopped (i.e. “consider all running timeAlignmentTimers as expired”).

Observation 3: Same as the legacy, if an SCell is not able to send any UL transmission (except for PRACH) due to the 2 TAT expiry, the UL transmission of other cells should not be impacted.

As such, we think that when the SpCell is configured with two TAGs, if the two TATs of the two TAGs of the SpCell expire, the UE should follow the legacy behaviours of the PTAG TAT expiry. When an SCell is configured with two TAGs, if the two TATs of the two TAGs of the SCell expire, the UE should follow the legacy behaviours of the STAG TAT expiry.

Proposal 1: When the 2 TAT(s) of SpCell expire, the UE follows the legacy behaviours of the PTAG TAT expiry.

Proposal 2: When the 2 TAT(s) of SCell expire, the UE follows the legacy behaviours of the STAG TAT expiry.

According to the RAN1 feedbacks “when the TA timer associated to one TRP expires for a TAG associated with a TCI state, UL or DL operation associated to the another TRP is not impacted”, we think that the UE should stop the UL transmission (except for PRACH or MsgA) belonging this TAG of the serving cell, when the serving cell is configured with two TAGs and when one TAT is running and one TAT expires. According to the RAN1 feedbacks, the UE behaviour for the single TAT expiry when two TATs are configured for a serving cell should be common for PTAG and STAG, and common for SpCell and SCell. The RACH procedure should be anyway allowed in order to get a new TA for the TAG with the expired TAT.

Proposal 3: When two TAGs are configured for a serving cell and only one TAT of SpCell or SCell expires, the UE stops all uplink transmission of the corresponding TAG (except for PRACH and MsgA), following RAN1 agreement.

## Maximum uplink transmission timing difference between TAGs

38.321: When the MAC entity stops uplink transmissions for an SCell due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell as expired.

As quoted above from the 38.321, it is possible that when two TAGs are configured for a serving cell, the uplink transmission timing difference between one TAG of this serving cell and another TAG is more than the maximum uplink transmission timing difference between TAGs. Then the UE is not able to perform UL transmission for these two TAGs due to the RF chain limitation. From our understanding, the legacy requirements for the maximum uplink transmission timing difference between TAGs should still be applicable, and the UE should stop the uplink transmission towards one TAG by considering the corresponding TAT as expired.

Proposal 4: When the maximum uplink transmission timing difference between TAGs is exceeded, the TAT associated with one TAG is considered as expired, as legacy.

## No running TAT for two TAGs of a serving cell

38.321: The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble and MSGA transmission when the timeAlignmentTimer associated with the TAG to which this Serving Cell belongs is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not on-going. Furthermore, when the timeAlignmentTimer associated with the PTAG is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when the cg-SDT-TimeAlignmentTimer is not running during the ongoing CG-SDT procedure as triggered in clause 5.27 and the inactivePosSRS-TimeAlignmentTimer is not running.

As quoted above, when the TAT of a serving cell is not running, the UE shall not perform any uplink transmission except for PRACH and MsgA. Then when two TAGs are configured, if two TATs are not running, we think that the UE should also stop the uplink transmission towards the serving cell, except for PRACH and MsgA. For SpCell, if two TATs of SpCell are not running, we think that the UE should stop the uplink transmission towards all serving cells except for PRACH and MsgA.

Proposal 5: When the 2 TAT(s) of SCell are not running, the MAC entity stops all uplink transmission on the SCell (except for PRACH and MsgA), as legacy.

Proposal 6: When the 2 TAT(s) of SpCell are not running, the MAC entity stops all uplink transmission on all serving cells (except for PRACH and MsgA), as legacy.

## Modelling of PTAG or STAG

38.321: Timing Advance Group: A group of Serving Cells that is configured by RRC and that, for the cells with a UL configured, using the same timing reference cell and the same Timing Advance value. A Timing Advance Group containing the SpCell of a MAC entity is referred to as Primary Timing Advance Group (PTAG), whereas the term Secondary Timing Advance Group (STAG) refers to other TAGs.

As quoted above, the PTAG needs to include at least SpCell and zero or more SCell(s). The STAG includes one or more SCell(s). From our understanding, there is no issue to clarify the UE behaviours for TAT expiry, no matter whether the two TAGs configured for a serving cell are modelled as PTAG or STAG. Thus we think that the concept of PTAG or STAG can be used when two TAGs are configured for SpCell or SCell.

Proposal 7: PTAG includes at least SpCell and zero or more SCells, i.e. no specification change for the PTAG definition.

Proposal 8: STAG includes one or more SCells, i.e. no specification change for the STAG definition.

According to Proposal 7 and 8, if the concept of PTAG is reused, this means that the two TAGs configured for SpCell are two PTAGs, and the two TAGs configured for an SCell can be either PTAG or STAG.

Proposal 9: The 2 TAGs of SpCell are configured as 2 PTAG(s).

Proposal 10: The 2 TAGs of SCell can be configured as either PTAG or STAG.

According to the discussion provided above, we have a text proposal to capture the proposals in the MAC specification.

Observation 4: The text proposal for 38.321 capturing all the proposals can be found in Annex B.

# Conclusion

According to the above analysis, we have the following observations and proposals.

Observation 1: The current MAC specification specifies the UE behaviours for the TAT expiry for each serving cell separately.

Observation 2: Same as the legacy, if the SpCell is not able to send any UL transmission (except for PRACH) due to the 2 TAT expiry, all UL transmission (except for PRACH) of the same cell group (i.e. either MCG or SCG) should be stopped (i.e. “consider all running timeAlignmentTimers as expired”).

Observation 3: Same as the legacy, if an SCell is not able to send any UL transmission (except for PRACH) due to the 2 TAT expiry, the UL transmission of other cells should not be impacted.

Observation 4: The text proposal for 38.321 capturing all the proposals can be found in Annex B.

Proposal 1: When the 2 TAT(s) of SpCell expire, the UE follows the legacy behaviours of the PTAG TAT expiry.

Proposal 2: When the 2 TAT(s) of SCell expire, the UE follows the legacy behaviours of the STAG TAT expiry.

Proposal 3: When two TAGs are configured for a serving cell and only one TAT of SpCell or SCell expires, the UE stops all uplink transmission of the corresponding TAG (except for PRACH and MsgA), following RAN1 agreement.

Proposal 4: When the maximum uplink transmission timing difference between TAGs is exceeded, the TAT associated with one TAG is considered as expired, as legacy.

Proposal 5: When the 2 TAT(s) of SCell are not running, the MAC entity stops all uplink transmission on the SCell (except for PRACH and MsgA), as legacy.

Proposal 6: When the 2 TAT(s) of SpCell are not running, the MAC entity stops all uplink transmission on all serving cells (except for PRACH and MsgA), as legacy.

Proposal 7: PTAG includes at least SpCell and zero or more SCells, i.e. no specification change for the PTAG definition.

Proposal 8: STAG includes one or more SCells, i.e. no specification change for the STAG definition.

Proposal 9: The 2 TAGs of SpCell are configured as 2 PTAG(s).

Proposal 10: The 2 TAGs of SCell can be configured as either PTAG or STAG.

# Reference

[1] R1-2302226, LSin from RAN1, “LS to RAN2/4 on Agreements for Rel-18 MIMO”

[2] R1-2213004, LSin from RAN1, “LS on Multi-DCI Multi-TRP with two TAs”

[3] R1-2306249, LSin from RAN1, “Reply on LS 2TA for multi-DCI multi-TRP”

# Annex A

The followings are extracted from the RAN1 reply LS in [3]:

Question 1 on TAG grouping RAN2 discussed how the cells/TRPs configured for the UE, are to be grouped if UE is configured with two TA groups per serving cell. Currently, NR does not impose any requirements in configuring the association of serving cells and TAGs. Q1a: For the 2TA operation, are there any restrictions on the association of serving cells and/or TRPs to the TAGs? Answer: Apart from the agreements RAN1 has sent in LS R1-2302226 to RAN2 before, RAN1 has not agreed to any further restrictions on the association of serving cells and/or TRPs to the TAGs at this point. If RAN1 agrees to such restrictions, RAN1 will inform RAN2. Q1b: NR currently supports up to 4 TAGs per cell group. Are the 4 TAGs enough or does RAN1 see a need to increase the number of TAGs per cell group? Answer: RAN1 has not reached consensus to increase the current number of TAGs per cell group. Question 2 on operation Q2: When the time-alignment timer associated with one of the TRPs of a serving cell expires, are certain UL or DL operation only impacted towards that TRP while they are not impacted towards the another TRP? If so, which UL or DL operation? Answer: RAN1 confirms that when the TA timer associated to one TRP expires for a TAG associated with a TCI state, UL or DL operation associated to the another TRP is not impacted. This further depends on PTAG/STAG definition, which is up to RAN2 to decide. Which UL or DL operation is impacted have not been discussed in RAN1.

# Annex B

According to the above proposals, we have to following text proposal:

Text proposal for 2 TAs of a cell in 3GPP TS 38.321: 1> when a timeAlignmentTimer expires: 2> if the timeAlignmentTimer is associated with the PTAG, and the SpCell includes only one TAG; 2> else if the timeAlignmentTimer is associated with the PTAG, and the SpCell includes two TAG(s) for which another timeAlignmentTimer also expires or is not running: 3> flush all HARQ buffers for all Serving Cells; 3> notify RRC to release PUCCH for all Serving Cells, if configured; 3> notify RRC to release SRS for all Serving Cells, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> consider all running timeAlignmentTimers as expired; 3> maintain NTA (defined in TS 38.211 [8]) of all TAGs. 2> else if the timeAlignmentTimer is associated with an STAG, then for all SCells belonging to this TAG when the SCell includes only one TAG; 2> else if the timeAlignmentTimer is associated with an STAG, and for all SCells belonging to this TAG when the Serving Cell includes two TAG(s) for which another timeAlignmentTimer(s) also expires or is not running: 3> flush all HARQ buffers; 3> notify RRC to release PUCCH, if configured; 3> notify RRC to release SRS, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> maintain NTA (defined in TS 38.211 [8]) of this TAG. 2> if the timeAlignmentTimer is associated with the PTAG or an STAG, then for the Serving cells belong to this TAG when the Serving cell includes two TAG(s) for which another timeAlignmentTimer is running: 3> stop all UL transmission belonging to this TAG except PRACH and MsgA; 3> maintain NTA (defined in TS 38.211 [8]) of this TAG.

When the MAC entity stops uplink transmissions for an SCell or a TAG of a serving cell when two TAGs are configured for the serving cell due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell or a TAG of a serving cell when two TAGs are configured for the serving cell as expired. The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble and MSGA transmission when any timeAlignmentTimer associated with the TAG to which this Serving Cell belongs is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not on-going. Furthermore, when any timeAlignmentTimer associated with the SpCell is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when the cg-SDT-TimeAlignmentTimer is not running during the ongoing CG-SDT procedure as triggered in clause 5.27 and the inactivePosSRS-TimeAlignmentTimer is not running.