3GPP TSG-RAN WG2 Meeting #123                             	R2-2307759

Toulouse, France, August 21–25, 2023

Agenda item:	7.23.2

Source:	Samsung

Title:	Open Issues on Timing Synchronization

Document for:	Discussion & Decision

Introduction

In RAN2#122 meeting, RAN2 made the following agreements:

Agreements

- Update of event ID is informed to UE by normal SI modification procedure.

- Confirm the AS layer of the UE determines if there a change of event ID and/or gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer. For both IDLE and INACTIVE mode, NAS layer may requests the RRC layer to move to RRC_CONNECTED

- For RRC_CONNECTED mode UEs, the NW has the necessary information to determine whether to send detailed clock quality information to the UE, (i.e. it may choose to always send update or only when needed) and the details can be left to NW implementation

- Event ID is optional.  Under the same gNB, UE considers the change of timing synchronization if event ID field from SIB9 is different.   FFS if gNB always broadcasts event ID

- If the UE is in RRC_INACTIVE, the UE can acquire clock quality information using the SDT procedure, if it supports and/is configured with SDT procedure.

This document discusses remaining open issues on timing synchronization in URLLC WI.

Discussion

Time Synchronization Status Update

Figure 1. Different cells with different Event IDs

An FFS point in RAN2#122 is whether gNB always broadcast Event ID. If Event ID is omitted for no change, there may be an issue at cell reselection to cell with different Event ID. In Figure 1, Cell 1 and Cell 2 provide different synchronization status, so their Event IDs are different. If Event ID is not signalled because each cell’s synchronization status does not change, a UE in RRC_IDLE or RRC_INACTIVE moved from cell with different Event ID has no idea to check whether the Event ID is changed.

Another problematic case is cell reselection to legacy cell which does not broadcast Event ID. Then, the UE may misunderstand that the legacy gNB support the same level of time synchronization status with the previous cell supporting Rel-18 URLLC cell.

To avoid the problematic cases, Event ID should be always signalled by gNB.

Proposal 1: Event ID is always broadcasted by gNB.

Change of Event ID means that at least one UE’s guaranteed clock quality is changed. Thus, Event ID needs to be linked to UE’s clock quality information signalled via DLInformationTransfer. It means that the UE needs to have at least a reference Event ID before it checks Event ID received in SIB9. A simple way is to signal Event ID together with other clock quality information via DLInformationTransfer. By using this Event ID, UE checks if the Event ID is changed in SIB9. If the UE’s stored Event ID is different Event ID received in SIB9, the UE needs to indicate it to the upper layer.

Proposal 2: Event ID is included in DLInformationTransfer. UE stores the received Event ID to check if it changes.

It is assumed that RRC_CONNECTED UE periodically receives time information (i.e. ReferenceTimeInfo) and gNB can directly send clock quality information via DLInformationTransfer. Event ID is needed only if clock quality information cannot be signalled to UE. Also, Event ID may change in case that the clock quality for other UEs changed but the clock quality for that UE is the same. In this case, checking Event ID change is not only unnecessary but also confusing. Thus, it is reasonable that only RRC_INACTIVE or RRC_IDLE UE needs to check Event ID.

Proposal 3: RRC_CONNECTED UE does not check if Event ID is changed.

Delivery of Clock Quality Information

For this WI, RAN2 should focus on Uu signalling of 5G clock quality information from gNB to UE. An issue here is which 5G clock quality information should be signalled from gNB to UE. SA2 discussed and agreed the clock quality detail level as specified in TS 23.501 [1], which indicates what should be provided to the UE for the clock quality information. One level is clock quality metrics and other level is acceptable/not acceptable indication.

- Clock quality detail level. It indicates whether and which clock quality information to provide to the UE and can take one of the following values: clock quality metrics or acceptable/not acceptable indication. - If the clock quality detail level equals "clock quality metrics", the NG-RAN provides clock quality metrics to the UE that reflect its current timing synchronization status. Clock quality metrics refers to the following information: clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state. - If the clock quality detail level equals "acceptable/not acceptable indication", clock quality acceptance criteria for the UE. The NG-RAN provides an acceptable indication to the UE if the NG-RAN's timing synchronization status matches the acceptance criteria received from AMF; otherwise, NG-RAN indicates "not acceptable" to the UE. Acceptance criteria can be defined based on one or more of the following attributes: parent time source, traceability to UTC and to GNSS, synchronization state, clock accuracy, frequency stability. Editor's note: Attributes that can be used for clock quality acceptance criteria depends on RAN capabilities to determine them and pending RAN WGs feedback

Observation 1: The clock quality reporting comprises of clock quality metrics or acceptable/not acceptable indication.

UEs may not be interested in all those components of clock quality information but be interested in whether NR meets the synchronization requirements on 5G clock accuracy. For instance, frequency stability or parent time source may not be used by the UE. Also, a gNB may not know some particular information. Thus, gNB can selectively deliver the clock quality information.

RAN3 agreed to support signalling of clock quality metrics and acceptance/not acceptance indication and those are captured in the endorsed CRs [2, 3]. Also, for clock quality metrics, all components can be optionally signalled according to gNB’s capability. We see this principle can be the same for Uu signalling..

Proposal 4: 5G clock quality information is signalled as either “clock quality metric” or “acceptable/not acceptable indication”

Proposal 5: 5G clock quality metric may include the following optional information elements:

- clock accuracy

- traceability to UTC

- traceability to GNSS

- frequency stability

- parent time source

- synchronization state

A question could be whether the clock accuracy should be newly introduced or not. The total clock accuracy is determined by the following three components:

- Time uncertainty: Accuracy of “time” indicated in ReferenceTimeInfo

- PDC error: Quantization error of propagation delay compensation (PDC)

- Refresh error: Error due to UE mobility and refresh of clock quality information

In ReferenceTimeInfo, “uncertainty” of the time information is equivalent to accuracy of time (a) and has been supported since Rel-16. Also, it is clear that ReferenceTimeInfo is more frequently delivered than clock quality information which does not change so frequently. PDC error (b) is known to UE, so additional signalling may not be needed. Refresh error (c) is not currently known to UE, however it might not be dominant compared with other error component.

Observation 2. uncertainty of ReferenceTimeInfo may replace clock accuracy.

We see signalling of clock accuracy is not essential due to the reasons above. But if RAN2 agrees to introduce the signalling of clock accuracy together with other clock quality metrics, the clock accuracy should be the total accuracy including all three error components a) time uncertainty, b) PDC error and c) refresh error.

Proposal 6: For signalling of clock accuracy, RAN2 to select one of the following options:

- Option 1: no separate signalling on clock accuracy (i.e. uncertainty is reused)

- Option 2: introduce a separate signalling of clock accuracy including time uncertainty, PDC error and refresh error

SA2 discussion focused on unicast delivery due to the size issue of the clock quality information. However, we see the same information can be delivered by SIB9 in broadcast manner. Compared to Rel-16 ReferenceTimeInfo delivered by SIB9, the newly introduced information is considered very small and the maximum size is expected around 20-30 bits even if all components of clock quality metric are transmitted. Moreover, a single cell could have a common clock source with a common quality for most of cases or a few different clock sources with a few different clock qualities. It is clear that size is not an issue.

Observation 3. 5G clock quality delivery via SIB9 does not have any size issue.

A benefit of broadcast delivery of clock quality information is that the UE simply checks whether and which interested clock quality information is changed. It can avoid excessive state transition to RRC_CONNECTED and admission control. Hence, it would be nicer to support the broadcast delivery.

Proposal 7: 5G clock quality information can be delivered by SIB9.

Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1: Event ID is always broadcasted by gNB.

Proposal 2: Event ID is included in DLInformationTransfer. UE stores the received Event ID to check if it changes.

Proposal 3: RRC_CONNECTED UE does not check if Event ID is changed.

Proposal 4: 5G clock quality information is signalled as either “clock quality metric” or “acceptable/not acceptable indication”

Proposal 5: 5G clock quality metric may include the following optional information elements:

- clock accuracy

- traceability to UTC

- traceability to GNSS

- frequency stability

- parent time source

- synchronization state

Proposal 6: For signalling of clock accuracy, RAN2 to select one of the following options:

- Option 1: no separate signalling on clock accuracy (i.e. uncertainty is reused)

- Option 2: introduce a separate signalling of clock accuracy including time uncertainty, PDC error and refresh error

Proposal 7: 5G clock quality information can be delivered by SIB9.

Reference

[1] TS 23.501 v18.1.0, System architecture for the 5G System (5GS); Stage 2 (Release 18)

[2] R3-232551, (BLCR to 38.413) Introduction of 5G Timing Resiliency and URLLC enhancements, Huawei, China Unicom, Nokia, Nokia Shanghai Bell, Samsung, Ericsson, ZTE, CATT

[3] R3-232553 (BLCR to 38.473) Introduction of 5G Timing Resiliency and URLLC enhancements, ZTE, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, Samsung